3GPP TSG-SA5 Meeting #131e 	S5-203271

- e-meeting 25th May-3rd June 2020

- Source:	Ericsson

- Title:	pCR R16 28308-030 Addition of QMC operations and notifications

- Document for:	Approval

Agenda Item:	6.4.1

# 1	Decision/action requested

Approve the proposal.

# 2	References

-

# 3	Rationale

This is the start of specifying the operations and notifications for QMC.

# 4	Detailed proposal

First change.

### 5.2.1		Relationships

### 5.2.2		Inheritance

Next change.

#### 5.3.1.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| serviceType | M | T | T | T | F |
| areaScope | M | T | T | T | F |
| qoECollectionEntityAddress | M | T | T | T | F |
| pLMNTarget | M | T | T | T | F |
| qMCTarget | M | T | T | T | F |
| qoEReference | M | T | T | T | F |
| qMCConfigurationFile | M | T | T | T | F |

Next change.

### 5.5.1		Attribute properties

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| serviceType | It identifies an individual UE for a specified end user service/end user service type. allowedValues: DASH (0), MTSI (1) | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| areaScope | The area scope parameter defines the area in terms of cells or Tracking Area/Routing Area/Location Area where the QMC shall take place. allowedValues: The area scope parameter in UMTS is either: - List of cells, identified by CGI. Maximum 32 CGI can be defined. - List of Routing Area, identified by RAI. Maximum of 8 RAIs can be defined. - List of Location Area, identified by LAI. Maximum of 8 LAIs can be defined. The area scope parameter in LTE is either: - list of cells, identified by E-UTRAN-CGI. Maximum 32 CGI can be defined. - List of Tracking Area, identified by TAC. Maximum of 8 TAC can be defined. | type: String multiplicity: 1..* isOrdered: N/A isUnique: True defaultValue: None isNullable: False |
| qoECollectionEntityAddress | It indicates the IP address to which the QMC records shall be transferred. Ipv4 or Ipv6 address(es) may be used. allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| pLMNTarget | This parameter defines the PLMN for which sessions shall be selected in the network request session in case of area based QMC when several PLMNs are supported in the RAN (this means that shared cells and not shared cells are allowed for the specified PLMN. Furthermore, several PLMNs can be used for not shared RAN cases as well as for shared RAN cases.). Only the sessions may be selected where the PLMN that the UE reports as selected PLMN is the same as the PLMN Target. Note that the PLMN Target might differ from the PLMN specified in the Network Request Session Id, as that specifies the PLMN that is containing the management system requesting the network request session from the NE. | type: PLMNId multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| qMCTarget | The QMC target parameter specifies it the QMC is area based or individual UE based. - Area based QMC (0) - NOTE: Individual UE based QMC is not supported in this release. allowedValues: … | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| qoEReference | The QoE reference parameter specify the network request session. The QoE reference shall be globally unique therefore it is composed as follows: MCC+MNC+QMC ID, where the MCC and MNC are coming with the QMC activation request from the management system to identify one PLMN containing the management system, and QMC ID is a 3 byte Octet String. The QMC ID is generated by the management system or the operator. It is used to identify the QoE measurement collection job in the traffic nodes and in the measurement collection centre. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| qMCConfigurationFile | The QMC configuration file is a container that is specified in 3GPP TS 26.247 and 3GPP TS 26.114 allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |

Next change.

### :

Next change.

## tables.

Next change.

### ollowing is an example.

Next change.

### le.

Next change.

## 6.1	Class diagram representing interfaces

Each interface is defined in one or more UML compliant class diagrams (see also 3GPP TS 32.156 [4]).

Next change.

### 6.3.1		Operation activateAreaQMCJob (M)

Next change.

#### 6.3.1.1	Definition

This operation support IRPManager’s request to create an AreaQMCJob through Itf-N.

Once the AreaQMCJob has been created, the attributes of the QMCJob will not be modified during the lifetime of the QMCJob.

One AreaQMCJob can manage network request sessions in one or more ManagedEntity.

| Referenced TS | Requirement label | Comment |
| --- | --- | --- |
| 3GPP TS 28.307 [3] | REQ-EUSPC-FUN-1 |  |

Next change.

#### 6.3.1.2	Input parameters

| Parameter Name | Qualifier | Information type | Comment |
| --- | --- | --- | --- |
| iOCInstance | M | ManagedEntity.objectInstance | It specifies the DN of ManagedEntity instance where QMC session is to be activated. |
| qoEReference | M | QMCJob.qoeReference | It identifies the QMC Session. |
| qMCTarget | M | QMCJob.qMCTarget | It specifies if the QMC is areabased or individual UE based. In the present release only area based is supported. |
| qoECollectionEntityAddress | M | QMCJob.qoECollectionEntityAddress | It specifies the address to the QoE Collection Entity that is associated to the QMCJob. See 3GPP TS 32.422 [9]. |
| serviceType | M | QMCJob.serviceType | It specifies which service is to record information. |
| areaScope | M | QMCJob.areaScope | It specifies the area (Cells/TA/RA/LA) where the QoE measurements shall be collected. |
| pLMNTarget | CM | QMCJob.pLMNTarget | It specifies which PLMN that the subscriber of the session to be recorded uses as selected PLMN. |
| qMCConfigurationFile | M | QMCJob.qMCConfiguration.File | It specifies the configuration of the QoE measurement to be recorded. See TS 26.247 [4] for DASH and TS 26.114 [9] for MTSI. |

Next change.

#### 6.3.1.4	Pre-condition

validQoEReference

| Assertion Name | Definition |
| --- | --- |
| validQoEReference | The qoEReference input parameter is valid, which means that the QMCIRP is aware of such QMCJob, which has this qoEReference value and is aware of the ManagedEntity holding such network request session. |

Next change.

#### 6.3.1.5	Post-condition

networkRequestSessionisactivated

| Assertion Name | Definition |
| --- | --- |
| networkRequestSessionisactivated | The network request session identified by the qoEReference is activated in the requested ManagedEntity instance and the QMCJob is started. |

:

Next change.

#### 6.3.1.6	Exceptions

Next change.

##### 6.3.1.6.1		notuniqueQoEReference

| Exception Name | Definition |
| --- | --- |
| notuniqueQoEReference | Condition: (validqoEReference) is false. Returned Information: output parameter status is set to "Failure". Exit state: Entry State. |

below:

Next change.

Next change.

### 6.3.2	Operation deactivateQMCJob (M)

Next change.

#### 6.3.2.1	Definition

This operation supports IPRManager’s request to stop a QMCJob through Itf-N. When this operation is received in the requested ManagedEntity instances the network request session shall be deactivated.

| Referenced TS | Requirement label | Comment |
| --- | --- | --- |
| 3GPP TS 28.307 [3] | REQ-EUSPC-FUN-3 |  |

Next change.

#### 6.3.2.a	Input parameters

| Parameter Name | Qualifier | Information type | Comment |
| --- | --- | --- | --- |
| iOCInstance | M | ManagedEntity.objectInstance | It specifies the DN of ManagedEntity instance where QMC session is to be deactivated. |
| qoEReference | M | QMCJob.qoeReference | It identifies the QMC Session. |
| qMCTarget | M | QMCJob.qMCTarget | It specifies if the QMC is area based or individual UE based. In the present release only area based is supported. |

Next change.

#### 6.3.2.b	Output parameters

| Parameter Name | Qualifier | Matching Information | Comment |
| --- | --- | --- | --- |
| status | M | ENUM (Success, Failure, PartialSuccess) | The operation may fail because of a specified or an unspecified reason. |
| unsupportedList | M | List of <ManagedEntity, qoEReference, qMCTarget) | It specifies what attributes are not supported when a QMC session is activated. The list can contain one or all of the elements and relevant only for error cases. |

Next change.

#### 6.3.2.c	Pre-condition

validQoEReference

| Assertion Name | Definition |
| --- | --- |
| validQoEReference | The qoEReference input parameter is valid, which means that the QMCIRP is aware of such QMCJob, which has this qoEReference value and is aware of the ManagedEntity holding such network request session. |

Next change.

#### 6.3.2.d	Post-condition

networkRequestSessionisdeactivated

| Assertion Name | Definition |
| --- | --- |
| networkRequestSessionisdeactivate | The network request session identified by the qoEReference is deactivated in the requested ManagedEntity instance and the QMCJob is stopped. |

Next change.

#### 6.3.2.e	Exceptions

| Exception Name | Definition |
| --- | --- |
| notuniqueQoEReference | Condition: (validQoECReference) is false. Returned Information: output parameter status is set to "Failure". Exit state: Entry State. |

Next change.

### 6.3.x	Operation listQMCJob (M)

Next change.

#### 6.3.x.1	Definition

This operation support IPRManager’s request to list the parameters of a specific QMCJob through Itf-N.

Next change.

#### 6.3.x.2	Input parameters

| Parameter Name | Qualifier | Information type | Comment |
| --- | --- | --- | --- |
| qoEReference | M | QMCJob.qoEReference | It specifies the network request session that is requested for interrogation. |

Next change.

#### 6.3.x.3	Output parameters

| Parameter Name | Qualifier | Matching Information | Comment |
| --- | --- | --- | --- |
| iOCInstance | M | ManagedElement.objectInstance | It specifies the DN of ManagedElement instance where a network request session is requested to be listed. |
| Status | M | ENUM (Success, Failure) | The operation may fail because of a specified or an unspecified reason. |
| qoEReference | M | QMCJob.qoeReference | It identifies the network request session. |
| qMCTarget | M | QMCJob.qMCTarget | It specifies if the QMC is area based or individual UE based. In the present release only area based is supported. |
| qoECollectionEntityAddress | M | QMCJob.qoECollectionEntityAddress | It specifies the address to the QoE Collection Entity that is associated to the QMCJob. See 3GPP TS 32.422 [9]. |
| serviceType | M | QMCJob.serviceType | It specifies which service is to record information. |
| areaScope | M | QMCJob.areaScope | It specifies the area (Cells/TA/RA/LA) in which the QMC is to be performed. |
| pLMNTarget | CM | QMCJob.pLMNTarget | It specifies which PLMN that the subscriber of the session to be recorded uses as selected PLMN. |
| qMCConfigurationFile | M | QMCJob.qMCConfiguration.File | It specifies the configuration of the QoE measurement to be recorded. See TS 26.247 [4] for DASH and TS 26.114 [9] for MTSI. |

Next change.

#### 6.3.x.4	Pre-condition

validQoEReference

| Assertion Name | Definition |
| --- | --- |
| validQoEReference | The qoEReference input parameter is valid, which means that the QMCIRP is aware of such QMCJob, which has this qoEReference value and is aware of the ManagedEntity holding such network request session. |

Next change.

#### 6.3.x.5	Post-condition

qoEReferenceFound

| Assertion Name | Definition |
| --- | --- |
| qoEReferenceFound | The QMCIRP has found the requested QMCJob with the qoEReference and can read the configured parameters. |

Next change.

#### 6.3.x.6	Exceptions

| Exception Name | Definition |
| --- | --- |
| notuniqueQoEReference | Condition: (validQoEReference) is false. Returned Information: output parameter status is set to "Failure". Exit state: Entry State. |

Next change.

### 6.3.y	Operation listActivatedQMCJobs (M)

Next change.

#### 6.3.y.1	Definition

This operation support IRPManager’s request to list all the activated QMCJobs through Itf-N.

Next change.

#### 6.3.y.2	Input parameters

No input parameters for this operation.

Next change.

#### 6.3.y.3	Output parameters

| Parameter Name | Qualifier | Matching Information | Comment |
| --- | --- | --- | --- |
| qoEReferenceList | M | List of < QMCJob.qoeReference.objectinstance > | The qoEReferenceList provides the identification of each activated network request session. If no qoEReference can be found, then this list is empty and status is "Success" |
| status | M | ENUM (Success, Failure) | The operation may fail because of a specified or an unspecified reason. |

Next change.

### 6.3.za		Notification notifyNetworkRequestSessionFailure (O)

Next change.

#### 6.3.z.1	Definition

The QMCIRP notifies all subscribed IRPManagers and the QoE Collection Entity if a network request session in a ManagedEntity has not been started due to any problem.

Next change.

#### 6.3.z.2	Input parameters

| Parameter Name | Qualifiers | Matching Information | Comment |
| --- | --- | --- | --- |
| objectClass | M,Y | QMCIRP.objectClass | Notification header |
| objectInstance | M,Y | QMCIRP.objectInstance | Notification header |
| eventTime | M,Y | -- | Notification header |
| notificationType | M,Y | "notifyNetworkRequestSessionFailure" | Notification header |
| systemDN | M,Y | -- | Notification header |
| notificationID | O,Y | -- | Notification header |
| qoEReference | M,Y | QMCJob.qoEReference |  |
| reason | O,N | -- |  |

Next change.

#### 6.3.z.3	Triggering event

Next change.

##### 6.3.z.3.1	From state

problemInManagedEntity

| Assertion Name | Definition |
| --- | --- |
| problemInManagedEntity | Because of a problem the ManagedEntity cannot start a network request session. |

Next change.

##### 6.3.z.3.2	To state

NetworkRequestSessionFailureReported

| Assertion Name | Definition |
| --- | --- |
| NetworkRequestSessionFailureReported | The "notifyNetworkRequestSessionFailure" notification is emitted to the subscribed IRPManager(s). |

Next change.

#### w:

Next change.

Next change.

Next change.

Next change.

Next change.

End of changes.