3GPP TSG-SA5 Meeting #131e 	S5-203290

- e-meeting 25th May-3rd June 2020

- Source: Ericsson, Deutsche Telekom

- Title: Add attributes to assurance control loop

- Document for: Approval

Agenda Item: 6.4.7

# 1	Decision/action requested

In this box give a very clear / short /concise statement of what is wanted.

# 2	References

[1]	3GPP TS 28.625: "State Management Data Definition Integration Reference Point (IRP); Information Service (IS)".

[2]	ITU-T Recommendation X.731: "Information technology - Open Systems Interconnection - Systems Management: State management function".

# 3	Rationale

This contribution proposes to add attributes to the AssuranceControlLoop object class.

The AssuranceControlLoopGoal is the proxy class for the ServiceProfile and SliceProfile, the goal is the input to the control loop, the status (report) of the control loop is the output. The status (report) is the result of the actions and of the analytics in the control loop and can be either the actual measurements of the goal attributes or a prediction of the goal attributes.

The goal status is described by the AssuranceGoalStatus attribute which is a datatype as it consist of multiple atrributes as the goal itself may consist of multiple attributes.

The observation time can be specified in seconds, minutes etc. and is a datatpe called ObservationTimePerTimeUnit.

Other attributes that describe the properties of an AssuranceControlLoop are

- operational and administrative state [1], [2] to state management of the control loop instance.

- lifecycle management phase to support the operator to design, verify and test control loops without interfering with or using existing control loops

# 4	Detailed proposal

First change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	ETSI GS ZSM 002 V1.1.1 (2019-08): “Zero-touch network and Service Management (ZSM); Reference Architecture”

[3]	3GPP TS 28.550: "Management and orchestration; Performance assurance”

[4]	3GPP TS 28.545: "Management and orchestration; Fault Supervision (FS) "

[5]	3GPP TS 28.622: "Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)"

[6]	3GPP TS 28.541: "Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3"

[7]	3GPP TS 28.532: "Management and orchestration; Management services"

[8]	3GPP TS 32.302: "Telecommunication management; Configuration Management (CM); Notification Integration Reference Point (IRP); Information Service (IS)

[a]	3GPP TS 28.531: "Management and orchestration; Provisioning".

Change added after review

### 4.1.2	Model

#### 4.1.2.1 	Imported and associated information entities

##### 4.1.2.1.1 	Imported information entities and local labels

| Label reference | Local label |
| --- | --- |
| TS 28.622 [5], IOC, Top | Top |
| TS 28.622 [5], IOC, SubNetwork | SubNetwork |
| TS 28.622 [6], ProxyClass, ManagedEntity | ManagedEntity |
| TS 28.541 [6], dataType, ServiceProfile | ServiceProfile |
| TS 28.541 [6], dataType, SliceProfile | SliceProfile |

#### 4.1.2.2	Class diagram

#### 4.1.2.2.1	Relationships

Figure 4.1.2.2.1.1: Assurance management NRM fragment

#### 4.1.2.2.2	Inheritance

Figure 4.1.2.2.2.1: Assurance management inheritance relationships

Second change

#### 4.1.2.3	Class definitions

##### 4.1.2.3.1	AssuranceControlLoop

###### 4.1.2.3.1.1	Definition

This IOC represents the capabilities of a control loop, these include:

- to automatically adjust a ManagedEntity (for example a network slice) to meet the objective described in AssuranceControlLoopGoal

- to report the effectiveness of an AssuranceControlLoop

- state management of an AssuranceControlLoop

- to keep track of the lifecycle of an AssuranceControlLoop

###### 4.1.2.3.1.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| operationalState | M | T | F | F | T |
| administrativeState | M | T | T | F | T |
| controlLoopLifeCyclePhase | M | T | T | F | T |
| observationTimePeriod | M | T | T | F | T |
| assuranceGoalStatus | M | T | F | F | T |

4.1.2.3.1.3	Constraints

No constraints have been defined for this release

###### 4.1.2.3.1.4	Notifications

The common notifications defined in clause 4.1.2.5 are valid for this IOC, without exceptions or additions.

##### 4.1.2.3.2	AssuranceControlLoopGoal <<ProxyClass>>

###### 4.1.2.3.2.1	Definition

This IOC represents the <<dataType>> ServiceProfile and <<dataType>> SliceProfile, defined in network slice NRM in [6].

###### 4.1.2.3.2.2	Attributes

The attributes are defined in network slice NRM in [6].

###### 4.1.2.3.2.3	Attribute constraints

The attribute constraints are defined in network slice NRM in [6].

###### 4.1.2.3.2.3	Notifications

The notifications of IOCs using the <<dataType>> ServiceProfile or <<dataType>> SliceProfile are defined in network slice NRM in [6].

##### 4.1.2.3.3	AssuranceGoalStatus

###### 4.1.2.3.3.1	Definition

This class represents the status of the controlLoopGoal at the end of an observationPeriod. The status can be reported as actual status and predicted status. Data that is monitored by an assuranceControlLoop, and  includes measurements [x] and KPI’s [y] and predictions that are applicable to the assuranceControlLoopGoals.

An assuranceGoalStatus holds the value of the observation amd where applicable the value of a prediction. Depending on the AssuranceGoal the type of the AssuranceGoalStatusObserved and AssuranceGoalStatusPredicted can be different for different AssuranceGoalStatus MOIs

###### 4.1.2.3.3.2	Attributes

ribute this release

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| AssuranceGoalStatusObserved | M | T | T | F | T |
| AssuranceGoalStatusPredicted | O | T | T | F | T |

.

###### 4.1.2.3.3.3	Attribute constraints

No constraints have been defined for this release

###### 4.1.2.3.3.4	Notifications

The common notifications defined in subclause 4.1.2.5 are valid for this IOC, without exceptions or additions.

##### 4.1.2.3.4	ObservationTimePeriod <<dataType>>

###### 4.1.2.3.4.1	Definition

This datatype represents the time that a goal is observed which can be specified in seconds, minutes, hours or days depending on the goal that is being observed.

###### 4.1.2.3.4.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| observationTime | M | T | T | F | T |
| timeUnit | M | T | T | F | T |

###### 4.1.2.3.3.3	Attribute constraints

No constraints have been defined for this release

###### 4.1.2.3.3.4	Notifications

The common notifications defined in subclause 4.1.2.5 are valid for this IOC, without exceptions or additions.

#### 4.1.2.4	Attribute definitions

##### 4.1.2.4.1	Attribute properties

The following table defines the properties of attributes that are specified in the present document.

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| controlLoopLifeCyclePhase | It indicates the lifecycle phase of the ControlLoop. AllowedValues: Preparation, Commisioning, Operation and Decommisioning. | type: Enum multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| observationTimePeriod | It indicates the time duration over which a controlLoopGoal is observed. During the observation period various observation data is collected to assess assess if the controlLoopGoal has been met. The observation time is expressed in timeUnits. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| timeUnit | It indicates the unit of time used to express the observationTime AllowedValues: second, minute, hour, day | type: Enum multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| observationTime | It indicates the observation time expressed in number of timeUnit. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| assuranceGoalStatusObserved | It indicates the actual value of the controlLoopGoal at the end of an observation period | type: AssuranceGoalStatusObserved multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| assuranceGoalStatusPredicted | It indicates the predicted value of the controlLoopgoal at the end of an observation period see NOTE x, or of a future observation period, see NOTE y. | type: AssuranceGoalStatusPredicted multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |

NOTE x: the predictive capability is provided by using a different population for assessment than the population for which measurements are available.

NOTE y: the predictive capability is provided by using a method for predicting the most likely status in the future.

4.1.2.4.2	Constraints

No constraints have been identified for this release

#### 4.1.2.5	Notifications

This subclause presents a list of notifications, defined in [7], that provisioning management service consumer can receive. The notification parameter objectClass/objectInstance, defined in [a], would capture the DN of an instance of an IOC defined in the present document.