3GPP TSG-SA5 Meeting #131e 	S5-203296

- e-meeting 25th May-3rd June 2020

- Source:	Nokia, Nokia Shanghai Bell

- Title:	Update flows based on internal CTF

- Document for:	Approval

Agenda Item:	7.4.3

# 1	Decision/action requested

This is a pCR to TS 28.202 to Update flows based on internal CTF

# 2	References

[1]	3GPP TS 32.845 Study on Charging Aspects of Network Slicing

[2]	3GPP TS 28.202 "Charging management; Network slice management charging in the 5G System (5GS); Stage 2".

# 3	Rationale

This pCR proposes to update flows based on internal CTF.

# 4	Detailed proposal

The following changes are proposed to be incorporated into TS 28.202 [2]

First change

### 5.2.2	Message flows

#### 5.2.2.1	General

The flows in the present document specify the interaction between the different entitiesinvolved for Network Slice Management charging functionality, in different scenarios, based on TS 28.531 [252] clause 7 procedures and flows and TS 28.532 [253] provisioning MnS description.

Involved entities are:

-	The Service Producer (CTF) and the CHF in architecture with internal CTF.

-	The Service Producer, the eCTF and the CHF in architecture with eCTF.

The table 5.2.2.1.1 describes the mapping between the messages used in the flows, and the Operations specified in TS 28.531 [252].

- Table 5.2.2.1.1: Messages mapping to Operations

| Message | Operations |
| --- | --- |
| NSI Creation Request/Response | createMOI |
| NSI Modification Request/Response | modifyMOIAttributes |
| NSI Termination Request/Response | deleteMOI |

5.2.2.2	NSI Creation - PEC

The scenario where a NSI Creation request is invoked towards a Service Producer is described for the two architecture options in PEC mode.

Figure 5.2.2.2.1 describes the message flows with CTF internal to Service Producer:

Figure 5.2.2.2.1: NSI creation - Internal CTF - PEC

1.	The Service Producer (CTF) receives an NSI Creation Request from an MnS consumer.

2.	The Service producer (CTF) performs NSI Creation process by creating a new NSI or using an existing NSI.

3.	The Service Producer (CTF) sends the NSI Creation Response with appropriate result.

3ch-a. The Service Producer (CTF) sends Charging Data Request [Event] to CHF associated to the NSI Creation transaction.

3ch-b. The CHF creates a CDR for this NSI Creation transaction.

3ch-c. The CHF acknowledges by sending Charging Data Response [Event] to the Service Producer (CTF).

5.2.2.3	NSI Modification - PEC

The scenario where a NSI Modification request is invoked towards a Service Producer is described for the two architecture options in PEC mode.

Figure 5.2.2.3.1 describes the message flows with CTF internal to Service Producer:

Figure 5.2.2.3.1: NSI Modification - internal CTF - PEC

1.	The Service Producer (CTF) receives an NSI Modification Request from an MnS consumer.

2.	The Service Producer (CTF) performs NSI Modification process.

3.	The Service Producer sends the NSI Modification Response with appropriate result.

3ch-a. The Service Producer (CTF) sends Charging Data Request [Event] to CHF associated to the NSI Modification transaction.

3ch-b. The CHF creates a CDR for this NSI Modification transaction.

3ch-c. The CHF acknowledges by sending Charging Data Response [Event] to the Service producer (CTF).

5.2.2.4	NSI Termination - PEC

The scenario where a NSI Termination request is invoked towards a Service Producer is described for the two architecture options in PEC mode.

Figure 5.2.2.4.1 describes the message flows with CTF internal to Service Producer:

Figure 5.2.2.4.1: NSI Termination - internal CTF - PEC

1.	The Service Producer (CTF) receives an NSI Termination Request from an MnS consumer.

2.	The Service Producer (CTF) performs the NSI Termination process.

32.	The Service Producer (CTF) sends the NSI Termination Response.

4ch-a. The Service Producer (CTF) sends Charging Data Request [Event] to CHF associated to the NSI Termination transaction.

4ch-b. The CHF creates a CDR for this NSI Termination transaction.

4ch-c. The CHF acknowledges by sending Charging Data Response [Event] to the Service Producer (CTF).

End of change