3GPP TSG-SA5 Meeting #131e 	S5-203300

- e-meeting 25th May-3rd June 2020

- Source:	Nokia, Nokia Shanghai Bell

- Title:	Introduce CDR description

- Document for:	Approval

Agenda Item:	7.4.3

# 1	Decision/action requested

This is a pCR to TS 28.202 to Introduce CDR description

# 2	References

[1]	3GPP TS 32.845 Study on Charging Aspects of Network Slicing

[2]	3GPP TS 28.202 "Charging management; Network slice management charging in the 5G System (5GS); Stage 2".

# 3	Rationale

This pCR proposes to Introduce CDR description.

# 4	Detailed proposal

The following changes are proposed to be incorporated into TS 28.202 [2]

First change

### 6.1.x	Ga message contents

See clause 5.2.4.

### 6.1.y	CDR description on the Bnsm interface

#### 6.1.y.1	General

This clause describes the CDR content and format generated for Network Slice Management charging.

The following tables provide a brief description of each CDR parameter. The category in the tables is used according to the charging data configuration defined in clause 5.4 of TS 32.240 [1]. Full definitions of the CDR parameters, sorted by the name in alphabetical order, are provided in TS 32.298 [57].

#### 6.1.x.2	Network Slice Management charging CHF CDR data

If enabled, CHF CDRs for Network Slice Management charging shall be produced for each NSI operations.

The fields of Network Slice Management charging CHF CDR are specified in table 6.1.x.2.1.

Table 6.1.x.2.1: Network Slice Management charging CHF record data

| Field | Category | Description |
| --- | --- | --- |
| Record Type | M | CHF record. |
| Recording Network Function ID | OM | This field holds the name of the recording entity, i.e. the CHF id. |
| Subscriber Identifier | OM | This field holds the identifier of the MnS consumer. |
| NF Consumer Information | M | This field holds the information of the entity that used the charging service (i.e. Service Producer (CTF), eCTF). |
| NF Functionality | M | This field contains the function of the entity: Service Producer (CTF) or eCTF |
| NF Name | OC | This field holds the name of the entity. |
| NF Address | OC | This field holds the IP Address of the entity |
| NF PLMN ID | Oc | This field holds the PLMN identifier (MCC MNC) of the entity. |
| Record Opening Time | M | Described in TS 32.298 [57] |
| Duration | M | Described in TS 32.298 [57] |
| Record Sequence Number | C | Described in TS 32.298 [57] |
| Cause for Record Closing | M | Described in TS 32.298 [57] |
| Diagnostics | OM | Described in TS 32.298 [57] |
| Local Record Sequence Number | OM | Described in TS 32.298 [57] |
| Record Extensions | OC | Described in TS 32.298 [57] |
| Network Slice Management Charging information | OM | This field holds the Network Slice Management Charging information defined in clause 6.2.1.2. |

Editor's Note: the Subscriber Identifier is ffs.

End of change