3GPP TSG-SA5 Meeting #131-e	S5-203304

- Online, , 25th May 2020 - 3rd Jun 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.531 | CR | 0052 | rev | - | Current version: | 16.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-16 CR 28.531 Correction of provisioning services and procedures |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell, Orange |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NETSLICE-PRO_NS, TEI16 |  |  |  |  |  | Date: |  |  | 2020-05-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Improper use of non-management domain and other SDO terminology resulted in inconsistent documentation of 3GPP management procedures and generation of unnecessary content. Significant part of the TS is non-implementable. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove unnecessary and redundant content. Re-align the use of management terminology to ensure consistency with other SA5 specifications. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incosistent, misleading specification that is impossible to implement. As a result, lack of support for standardized management and orchestration of Network Slicing in 3GPP Rel-16. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1, 6.2, 6.4, 6.5 (void), 7(void), 8 (void) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | S5-203304 |  |  |  |  |  |  |  |  |

1st Modified Section

## 6.1	Management services for network slice provisioning

The management services for network slice provisioning are listed in table 6.1-1.

Table 6.1-1: Management services for network slice provisioning

| MnS Name | MnS Component Type A (operations and notifications) | MnS Component Type B (information model) | Note |
| --- | --- | --- | --- |
| Provisioning for network slice | Operations defined in clause 5 of TS 28.532 [8]: - createMOI operation - deleteMOI operation - getMOIAttributes operation - modifyMOIAttributes operation | Network slice information model defined in clause 6.3 of TS 28.541 [6] | This management service enables its consumer to request allocating, deallocating, or modifying a NetworkSlice MOI. The typical scenario is "Network Slices as NOP internals" model where this MnS is consumed by operators. |
| Provisioning data report for network slice | Operations defined in clause 5 of TS 28.532 [8]: - subscribe operation - unSubscribe operation Notifications defined in clause 5 of TS 28.532 [8]: - notifyMOICreation notification - notifyMOIDeletion notification - notifyMOIAttributeValueChanges notification | Network slice information model defined in clause 6.3 of TS 28.541 [6] | This management service enables its consumer to obtain notifications about NetworkSlice MOI data. The typical scenario is "Network Slices as NOP internals" model where this MnS is consumed by operators |
| Provisioning exposure for network slice | Operations defined in clause 5 of TS 28.532 [8]: - createMOI operation - deleteMOI operation - getMOIAttributes operation - modifyMOIAttributes operation | Network slice information model defined in clause 6.3 of TS 28.541 [6] | This management service enables its consumer to request allocating, deallocating or modifying an NetworkSlice MOI. The typical scenario is NSaaS model where this MnS is consumed by vertical industry. |
| Provisioning data report exposure for network slice | Operations defined in clause 5 of TS 28.532 [8]: - subscribe operation - unSubscribe operation Notifications defined in clause 5 of TS 28.532 [8]: - notifyMOICreation notification - notifyMOIDeletion notification - notifyMOIAttributeValueChanges notification | Network slice information model defined in clause 6.3 of TS 28.541 [6] | This management service enables its consumer to obtain notifications about NetworkSlice MOI data. The typical scenario is NSaaS model where this MnS is consumed by vertical industry. |

2nd Modified Section

## 6.2	Management services for network slice subnet provisioning

The management services for network slice subnet provisioning are listed in table 6.2-1.

Table 6.2-1: Management services for network slice subnet provisioning

| MnS Name | MnS Component of type A (Operations and notifications) | MnS Component of type B (information model) |
| --- | --- | --- |
| Provisioning for network slice subnet | Operations defined in clause 5 of TS 28.532 [8]: - createMOI operation - modifyMOIAttributes operation - getMOIAttributes operation - deleteMOI operation | NetworkSliceSubnet IOC information model defined in clause 6.3 of TS 28.541 [6] |
| Provisioning data report for network slice subnet | Operations defined in clause 5 of TS 28.532 [8]: - subscribe operation - unSubscribe operation Notifications defined in clause 5 of TS 28.532 [8]: - notifyMOICreation notification - notifyMOIDeletion notification - notifyMOIAttributeValueChanges notification | NetworkSliceSubnet IOC information model defined in clause 6.3 of TS 28.541 [6] |

3rd Modified Section

## 6.4	Management services for network and sub-network provisioning

The management services for network and sub-networks provisioning are listed in table 6.4-1.

Table 6.4-1: Management services for network and sub-network provisioning

| MnS name | MnS Component of type A (Operations and notifications) | MnS Component of type B (information model) |
| --- | --- | --- |
| Provisioning for network and sub-networks | Operations defined in clause 5 of TS 28.532 [8]: - createMOI operation - modifyMOIAttributes operation - getMOIAttributes operation - deleteMOI operation | NetworkSliceSubnet IOC, as defined in TS 28.541 [6] |
| Provisioning data report for sub-networks | Operations defined in clause 5 of TS 28.532 [8]: - subscribe operation - unSubscribe operation Notifications defined in clause 5 of TS 28.532 [8]: - notifyMOICreation notification - notifyMOIDeletion notification - notifyMOIAttributeValueChanges notification | NetworkSliceSubnet IOC(s) as defined in TS 28.541 [6] |

4th Modified Section

## 6.5.	Void

.1.3	Output parameters

.2.2	Input parameters

.2.3	Output parameters

arameters

rameters

parameters

.2	Input parameters

arameters

5th Modified Section

# 7	Void

6th Modified Section

# 8	Void

## ons to SS equivalents

2-1: Mapping of IS operation input parameters to SS equivalents (HTTP POST)

T)

uivalents (HTTP POST)

quivalents (HTTP DELETE)

quivalents (HTTP DELETE)

f IS operation input parameters to SS equivalents (HTTP POST)

OST)

End of Modifications