3GPP TSG-SA5 Meeting #132e 	S5-204138

# e-meeting 17th-28th August 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.532 | CR | 0136 | rev | - | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Change RESTFUL definition for performance data file report MnS to generic file data report MnS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_SLICE_e, TM_SBMA |  |  |  |  |  | Date: |  |  | 2020-08-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The performance data file report MnS described in TS 28.532 is specifc for performance data file, however, the data file report MnS for other management data (e.g. trace data) is missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change RESTFUL definition for performance data file report MnS to generic file data report MnS |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The data file report MnS for other management data (e.g. trace data) is missing |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 12.3.1.1, 12.X(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

12.3.1.1	Void

2nd Change

12.X	File data reporting service

12.X.1	RESTful HTTP-based solution set

12.X.1.1	Mapping of operations

12.X.1.1.1	Introduction

The IS operations are mapped to SS equivalents according to table 12.X.1.1.1-1.

Table 12.3.1.1.1-1: Mapping of IS operations to SS equivalents

| IS operation | HTTP Method | Resource URI | Qualifier |
| --- | --- | --- | --- |
| listAvailableFiles | GET | /Files | M |
| subscribe | POST | /subscriptions | M |
| unsubscribe | DELETE | /subscriptions | M |
|  | DELETE | /subscriptions/{subscriptionId} | M |

12.X.1.1.2	Operation "listAvailableFiles"

The IS operation parameters are mapped to SS equivalents according to table 12.X.1.1.2-1 and table 12.X.1.1.2-2.

Table 12.X.1.1.2-1: Mapping of IS operation input parameters to SS equivalents (HTTP GET)

| IS operation parameter name | SS parameter location | SS parameter name | SS parameter type | Qualifier |
| --- | --- | --- | --- | --- |
| managementDataType | query | managementDataType | managementDataType-Type | M |
| beginTime | query | beginTime | dateTime-Type | M |
| endTime | query | endTime | dateTime-Type | M |

Table 12.X.1.1.2-2: Mapping of IS operation output parameters to SS equivalents (HTTP GET)

| IS operation parameter name | SS parameter location | SS parameter name | SS parameter type | Qualifier |
| --- | --- | --- | --- | --- |
| fileInfoList | response body | n/a | fileInfoRetrieval-ResponseType | M |
| status | response status codes response body | n/a error | n/a error-ResponseType | M |

The message flow is as follows:

1. The Service Consumer sends a HTTP GET request to the Service Provider.

- The URI identifies the "…/Files" collection resource.

- The query part may contain filter parameter. Absence of the query part means all available management data files shall be returned.

- The request message body shall be empty.

2. The Service Provider sends a HTTP GET response to the Service Consumer.

- On success "200 OK" shall be returned. The response message body shall carry the information of available files. The response format is defined by "fileInfoRetrieval-ResponseType ".

- On failure, an appropriate error code shall be returned. The response message body may carry an error object.

12.X.1.1.3	Operation "subscribe"

See clause 12.2.1.1.8, with the discrepance that the subscribe operation in this clause is for file data reporting related notifications (i.e., notifyFileReady and notifyFilePreparationError).

12.X.1.1.4		Operation "unsubscribe"

See clause 12.2.1.1.9, with the discrepance that the unsubscribe operation in this clause is for file data reporting related notifications (i.e., notifyFileReady and notifyFilePreparationError).

12.X.1.2	Mapping of notifications

12.X.1.2.1		Introduction

The IS notifications are mapped to SS equivalents according to table 12.X.1.2.1-1.

Table 12.X.1.2.1-1: Mapping of IS notifications to SS equivalents

| IS notifications | HTTP Method | Resource URI | SQ |
| --- | --- | --- | --- |
| notifyFileReady | POST | /notificationSink | M |
| notifyFilePreparationError | POST | /notificationSink | M |

12.X.1.2.1		Notification "notifyFileReady"

The IS notification parameters are mapped to SS equivalents according to table 12.X.1.2.1-1.

Table 12.X.1.2.1-1: Mapping of IS notification input parameters to SS equivalents (HTTP POST)

| IS operation parameter name | SS parameter location | SS parameter name | SS parameter type | SQ |
| --- | --- | --- | --- | --- |
| objectClass | request body | href | uri-Type | M |
| objectInstance |  |  |  |  |
| notificationId | request body | notificationId | notificationId-Type | M |
| eventTime | request body | eventTime | dateTime-Type | M |
| notificationType | request body | notificationType | notificationType-Type | M |
| fileInfoList | request body | fileInfoList | array(fileInfo-Type) | M |
| additionalText | request body | additionalText | additionalText-Type | O |

12.X.1.3.1		Notification "notifyFilePreparationError"

The IS notification parameters are mapped to SS equivalents according to table 12.X.1.3-1.

Table 12.X.1.3.1-1: Mapping of IS notification input parameters to SS equivalents (HTTP POST)

| IS operation parameter name | SS parameter location | SS parameter name | SS parameter type | SQ |
| --- | --- | --- | --- | --- |
| objectClass | request body | href | uri-Type | M |
| objectInstance |  |  |  |  |
| notificationId | request body | notificationId | notificationId-Type | M |
| eventTime | request body | eventTime | dateTime-Type | M |
| notificationType | request body | notificationType | notificationType-Type | M |
| fileInfoList | request body | fileInfoList | array(fileInfo-Type) | M |
| reason | request body | reason | reason-Type | O |
| additionalText | request body | additionalText | additionalText-Type | O |

12.X.1.3	Resources

12.X.1.3.1	Resource structure

Figure 12.X.1.3.1-1 shows the resource structure of the file data reporting service.

Figure 12.X.1.3.1-1: Resource URI structure of the file data reporting service

Table 12.X.1.3.1-2 provides an overview of the resources and applicable HTTP methods.

Table 12.X.1.3.1-2: Resources and methods overview

| Resource name | Resource URI | HTTP method | Description |
| --- | --- | --- | --- |
| Files | /Files | GET | Retrieve the information of the available files |
| subscriptions | /subscriptions | POST | Create a subscription |
| subscriptions | /subscriptions | DELETE | Delete all subscriptions made with a consumerReferenceId |
| subscription | /subscriptions/{subscriptionId} | DELETE | Delete a single subscription |
| notificationSink | /notificationSink | POST | Send notifications |

12.X.1.3.2	Resource definitions

12.X.1.3.2.1	Resource “/Files”

12.X.1.3.2.1.1	Description

This resource represents the information about a collection of available files.

12.X.1.3.2.1.2	URI

Resource URI = {MnSRoot}/FileDataReportingMnS/{MnSVersion}/Files

The resource URI variables a defined in the following table.

Table 12.X.1.3.2.1.1-1: URI variables

| Name | Definition |
| --- | --- |
| MnSRoot | See clause 4.4 of TS 32.158 [15] |

12.X.1.3.2.1.3	HTTP methods

12.X.1.3.2.1.3.1	HTTP GET

This method shall support the URI query parameters specified in the following table.

Table 12.X.1.3.2.1.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | Description | SQ |
| --- | --- | --- | --- |
| managementDataType | managementDataType | To filter the available files based on the management data type. | M |
| beginTime | dateTime-Type | To filter the available files who became ready no later than this time stamp. | M |
| endTime | dateTime-Type | To filter the available files who became ready no earlier than this time stamp. | M |

This method shall support the request data structures, the response data structures and response codes specified in the following tables.

Table 12.X.1.3.2.1.3.1-2: Data structures supported by the GET request body on this resource

| Data type | Description | SQ |
| --- | --- | --- |

Table 12.X.1.3.2.1.3.1-3: Data structures supported by the GET response body on this resource

| Data type | Response codes | Description | SQ |
| --- | --- | --- | --- |
| fileInfoRetrieval-ResponseType | 200 OK | The resource representation of the infomraiton about the available files retrieved. | M |
| error-ResponseType | 4xx/5xx | Returned in case of an error | M |

12.X.1.3.2.2	Resource "/subscriptions"

12.X.1.3.2.2.1	Description

This resource is a container resource for individual subscriptions.

12.X.1.3.2.2.2	URI

The resource URI is:

Resource URI: {MnSRoot}/FileDataReportingMnS/{MnSVersion}/subscriptions

The resource URI variables a defined in the following table.

Table 12.X.1.3.3.2.2.2-1: URI variables

| Name | Definition |
| --- | --- |
| MnSRoot | See clause 4.4 of TS 32.158 [15] |

12.X.1.3.2.2.3	HTTP methods

12.X.1.3.2.2.3.1	POST

This method shall support the URI query parameters specified in table 12.X.1.3.2.2.3.1-1.

Table 12.X.1.3.2.2.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | Description | Qualifier |
| --- | --- | --- | --- |
| n/a | n/a | n/a | n/a |

This method shall support the request data structures specified in table 12.X.1.3.2.2.3.1-2 and the response data structures and response codes specified in table 12.X.1.3.2.2.3.1-3.

Table 12.X.1.3.2.2.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | Description | SQ |
| --- | --- | --- |
| subscription-RequestType | Details of the subscription to be created | M |

Table 12.X.1.3.2.2.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | Response codes | Description | SQ |
| --- | --- | --- | --- |
| subscription-ResponseType | 201 Created | In case of success the representation of the created subscription is returned. | M |
| error-ResponseType | 4xx/5xx | In case of failure the error object is returned. | M |

12.X.1.3.2.2.3.2	DELETE

This method shall support the URI query parameters specified in table 12.X.1.3.2.2.3.2-1.

Table 12.X.1.3.2.2.3.2-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | Description | Qualifier |
| --- | --- | --- | --- |
| consumerReferenceId | consumerReferenceId-QueryType | Identifies the consumer whose subscriptions shall be deleted | M |

This method shall support the request data structures specified in table 12.X.1.3.2.2.3.2-2 and the response data structures and response codes specified in table 12.X.1.3.2.2.3.2-3.

Table 12.X.1.3.2.2.3.2-2: Data structures supported by the DELETE Request Body on this resource

| Data type | Description | SQ |
| --- | --- | --- |
| n/a | n/a | n/a |

Table 12.X.1.3.2.2.3.2-3: Data structures supported by the DELETE Response Body on this resource

| Data type | Response codes | Description | SQ |
| --- | --- | --- | --- |
| n/a | 204 No Content | In case of success no message body is returned | n/a |
| error-ResponseType | 4xx/5xx | In case of failure the error object is returned. | M |

12.X.1.3.2.3	Resource "/subscriptions/{subscriptionId}"

12.X.1.3.2.3.1	Description

This resource represents a subscription.

12.X.1.3.2.3.2	URI

The resource URI is:

Resource URI: {MnSRoot}/FileDataReportingMnS/{MnSVersion}/subscriptions/{subscriptionId}

Table 12.X.1.3.2.3.2-1: URI variables

| Name | Definition |
| --- | --- |
| MnSRoot | See clause 4.4 of TS 32.158 [15] |
| subscriptionId | Subscription identifier |

12.X.1.3.2.3.3	HTTP methods

12.X.1.3.2.3.3.1	DELETE

This method shall support the URI query parameters specified in table 12.X.1.3.2.3.3-1.

Table 12.X.1.3.2.3.3-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | Description | Qualifier |
| --- | --- | --- | --- |
| n/a | n/a | n/a | n/a |

This method shall support the request data structures specified in table 12.X.1.3.2.3.3-2 and the response data structures and response codes specified in table 12.X.1.3.2.3.3-3.

Table 12.X.1.3.2.3.3-2: Data structures supported by the DELETE Request Body on this resource

| Data type | Description | SQ |
| --- | --- | --- |
| n/a | n/a | n/a |

Table 12.X.1.3.2.3.3-3: Data structures supported by the DELETE Response Body on this resource

| Data type | Response codes | Description | SQ |
| --- | --- | --- | --- |
| n/a | 204 No Content | In case of success no message body is returned | M |
| error-ResponseType | 4xx/5xx | In case of failure the error object is returned. | M |

12.X.1.3.2.4	Resource "/notificationSink"

12.X.1.3.2.4.1	Description

This resource represents a resource to which notifications are sent to.

12.X.1.3.2.4.2	URI

The resource URI is provided by the notification subscriber when creating the subscription.

12.X.1.3.2.4.3	HTTP methods

12.X.1.3.2.4.3.1	POST

This method shall support the URI query parameters specified in table 12.X.1.3.2.4.3.1-1.

Table 12.X.1.3.2.4.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | Description | Qualifier |
| --- | --- | --- | --- |
| n/a | n/a | n/a | n/a |

This method shall support the request data structures specified in table 12.X.1.3.2.4.3.1-2 and the response data structures and response codes specified in table 12.X.1.3.2.4.3.1-3.

Table 12.X.1.3.2.4.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | Description | SQ |
| --- | --- | --- |
| notifyFileReady-NotifType | Type in case a notifyFileReady notification is sent | M |
| notifyFilePreparationError-NotifType | Type in case a notifyFilePreparationError notification is sent | M |

Table 12.X.1.3.2.4.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | Response codes | Description | SQ |
| --- | --- | --- | --- |
| n/a | 204 No Content | In case of success no message body is returned | M |
| error-ResponseType | 4xx/5xx | In case of failure the error object is returned. | M |

12.X.1.4	Data type definitions

12.X.1.4.1	General

Table 12.X.1.4.1-1: Data types defined in this specification

| Data type | Reference | Description |
| --- | --- | --- |
| General types |  |  |
| dataTime-Type | 12.X.1.4.6.2 | Data type of date and time. |
| uri-Type | 12.X.1.4.6.2 | The data type of a URI. |
| Types used in paths |  |  |
| Types used in query parts |  |  |
| managementDataType-Type | 12.X.1.4.6.3 | Used in listing the information of available files describing the management data type of the files. |
| consumerReferenceId-QueryType | 12.X.1.4.6.2 | Used in the query part of HTTP DELETE on /Subscriptions to delete all subscriptions made with a specific consumerReferenceId |
| Types used in request bodies |  |  |
| subscription-RequestType | 12.X.1.4.4.1 | Used in the request body of HTTP POST on /subscriptions to create file data reporting notifications subscriptions. |
| Types used in response bodies |  |  |
| fileInfoRetrieval-ResponseType | 12.X.1.4.4.2 | Used in the response body of HTTP GET describing the information of the listed files. |
| error-ResponseType | 12.X.1.4.4.3 | Used in the response body describing the error. |
| Types used for resources |  |  |
| subscription-ResourceType | 12.X.1.4.4.4 | Representation of a subscription resource. |
| Types used in notifications |  |  |
| notifyFileReady-NotifType | 12.X.1.4.4.5 | Used in the request body of HTTP POST for the notification type notifyFileReady. |
| notifyFilePreparationError-NotifType | 12.X.1.4.4.6 | Used in the request body of HTTP POST for the notification type notifyFilePreparationError. |
| Types referenced by the definitions above |  |  |
| fileInfo-Type | 12.X.1.4.5.1 | Used for describing the file information. |
| notificationId-Type | 12.X.1.4.6.2 | Notification identifier as defined in ITU-T Rec. X. 733 [4] |
| notificationType-Type | 12.X.1.4.6.2 | Notification type (notifyFileReady, etc.) |
| additionalText-Type | 12.X.1.4.6.2 | Allows a free form text description to be reported as defined in ITU-T Rec. X. 733 [4] |
| reason-Type | 12.X.1.4.6.2 | Used to describe the reason causing the file preparation error. |

Table 12.X.1.4.1-2: Data types imported

| Data type | Reference | Description |
| --- | --- | --- |

12.X.1.4.2	Structured general data types

None.

12.X.1.4.3	Structured path data types

None.

12.X.1.4.4	Query, message body and resource data types

12.X.1.4.4.1	Type subscription-RequestType

Table 12.X.1.4.4.1-1: Definition of type subscription-RequestType

| Attribute name | Data type | Description | SQ |
| --- | --- | --- | --- |
| data | subscription-ResourceType | Used in the request body of HTTP POST on /subscriptions describing the representation of the subscription to be created | M |

12.X.1.4.4.2	Type fileInfoRetrieval-ResponseType

Table 12.X.1.4.4.2-1: Definition of type fileInfoRetrieval-ResponseType

| Attribute name | Data type | Description | SQ |
| --- | --- | --- | --- |
| data | array(fileInfoType) | The information of the available files | M |

12X.1.4.4.3	Type error-ResponseType

Table 12.X.1.4.4.3-1: Definition of type error-ResponseType

| Attribute name | Data type | Description | SQ |
| --- | --- | --- | --- |
| error | object | Key indicating the response body containing an error | M |
| > errorInfo | string | Attribute allowing to convey error information in string format | M |

12.X.1.4.4.4	Type subscription-ResourceType

Table 12.X.1.4.4.4-1: Definition of type subscription-ResourceType

| Attribute name | Data type | Description | SQ |
| --- | --- | --- | --- |
| consumerReference | uri-Type | The URI of the endpoint to send the notification to (/notificationSink). | M |
| timeTick | long-Type | Time window within which the subscriber intends to subscribe again to confirm its subscription, see clause 11.2.2.2.5.1 | O |
| filter | filter-Type | Filter settings for this subscription, to define the subset of all notifications this subscription relates to. A notification is sent to the subscriber if the filter matches, or if there is no filter. | O |

12.X.1.4.4.5	Type notifyFileReady-NotifType

Table 12.X.1.4.4.5-1: Definition of type notifyFileReady-NotifType

| Attribute name | Data type | Description | SQ |
| --- | --- | --- | --- |
| header |  |  |  |
| > href | uri-Type | URI of the resource indicating the file data reporting service | M |
| > notificationId | notificationId-Type | Notification identifier as defined in ITU-T Rec. X. 733 [4] | M |
| > notificationType | notificationType-Type | Notification type (notifyFileReady, etc.) | M |
| > eventTime | dateTime-Type | Event occurrence time (e.g., the file ready time) | M |
| body |  |  |  |
| > fileInfoList | array(fileInfo-Type) | The information of the available files | M |
| > additionalText | additionalText-Type | Allows a free form text description to be reported as defined in ITU-T Rec. X. 733 [4] | O |

12.X.1.4.4.6	Type notifyFilePreparationError-NotifType

Table 12.X.1.4.4.6-1: Definition of type notifyFilePreparationError-NotifType

| Attribute name | Data type | Description | SQ |
| --- | --- | --- | --- |
| header |  |  |  |
| > href | uri-Type | URI of the resource indicating the file data reporting service | M |
| > notificationId | notificationId-Type | Notification identifier as defined in ITU-T Rec. X. 733 [4] | M |
| > notificationType | notificationType-Type | Notification type (notifyFileReady, etc.) | M |
| > eventTime | dateTime-Type | Event occurrence time (e.g., the file ready time) | M |
| body |  |  |  |
| > fileInfoList | array(fileInfo-Type) | The information of the available files | M |
| > reason | reason-Type | The reason that caused the error of the file preparation. |  |
| > additionalText | additionalText-Type | Allows a free form text description to be reported as defined in ITU-T Rec. X. 733 [4] | O |

12.X.1.4.5	Referenced structured data types

12.X.1.4.5.1	Type fileInfo-Type

Table 12.X.1.4.5-1: Definition of fileInfo-Type

| Attribute name | Data type | Description | SQ |
| --- | --- | --- | --- |
| fileLocation | uri-Type | Usd to indicate the location of the file. | M |
| fileSize | long-Type | The size of the file with positive Integer value (the unit is byte). | M |
| fileReadyTime | dataTime-Type | Indicates the date and time when the file was last closed and made available in the management service producer and the file content will not be changed. | M |
| fileExpirationTime | dataTime-Type | Indicates the date and time beyond which the file may be deleted. | M |
| fileCompression | string | Identifies the name of the compression algorithm used for the file. | M |
| fileFormat | string | Identifies the encoding technique used by the file. Its value should indicate the version of the file format specification plus to indicate if "ASN1" or "XML-schema" is used. | M |

12.X.1.4.6	Simple data types and enumerations

12.X.1.4.6.1	General

This clause defines simple data types and enumerations that are used by the data structures defined in the previous clauses.

12.X.1.4.6.2	Simple data types

Table 12.X.1.4.6.2-1: Simple data types

| Type name | Type definition | Description |
| --- | --- | --- |
| dataTime-Type | string | The data type for date and time in “date-time” format. |
| uri-Type | string | The type of a URI. |
| consumerReferenceId-QueryType | uri-Type | Used in the query part of HTTP DELETE on /subscriptions to delate all subscriptions made with a specific consumerReferenceId. |
| filter-Type | string | Filter of a subscription resource. |
| notificationId-Type | long | Notification identifier as defined in ITU-T Rec. X. 733 [4] |
| additionalText-Type | string | Allows a free form text description to be reported as defined in ITU-T Rec. X. 733 [4] |
| reason-Type | string | THe type for describing the reason that caused the file preparation error. |

12.X.1.4.6.3	Enumeration managementDataType-Type

Table 12.X.1.4.6.3-1: Enumeration managementDataType-Type

| Enumeration value | Description |
| --- | --- |
| PERFORMANCE | It indicates that the management data file type is “Performance data” |
| TRACE | It indicates that the management data file type is “Trace data” |
| ANALYTICS | It indicates that the management data file type is “Analytics data” |
| PROPRIETARY | It indicates that the management data file type is “Proprietary data” |

12.X.1.4.6.4	Enumeration notificationType-Type

Table 12.X.1.4.6.4-1: Enumeration notificationType-Type

| Enumeration value | Description |
| --- | --- |
| notifyFileReady | Notification type is notifyFileReady |
| notifyFilePreparationError | Notification type is notifyFilePreparationError |

End of Change