3GPP TSG-SA5 Meeting #133e 	S5-205122

# e-meeting 12th - 21st October 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.274 | CR | 0077 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on Triggers field for converged charging |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1-SMSCH, TEI16 |  |  |  |  |  | Date: |  |  | 2020-10-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Triggers for SMS are represented by SM message type, and there is no need for an additional specific Triggers field in converged charging for SMS. Some clean-up is needed on wrong clauses number. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Describe the Triggers field as “not applicable” Correct clauses number Add PEC IEC and ECUR in abbreviations |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing Triggers could cause Inter-Operability issues if expected. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.3, 5.3.2.11.4, 5.3.2.12.3.2, 6.2a.1.2.1, 6.2a.3.2, 6.5.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1], TS 32.240 [2] and the following apply:

5GS	5G System

ECUR	Event Charging with Unit Reservation

IEC	Immediate Event Charging

PEC	Post Event Charging

NCGI	NR Cell Global Identity

Next change

##### 5.3.2.11.4	SMS submission to SMS-SC for Device Triggering - Replace procedure

Figure 5.3.2.11.4.1 describes the scenario where the MTC-IWF submits a request to SMS-SC for a replace procedure of Device Triggering, in IEC operation model:

Figure 5.3.2.11.4.1: Online charging - SMS submission to SMS-SC for replace Device Triggering

1)	The SMS-SC receives an incoming "Device Trigger Request" indicating "Replace" from an MTC-IWF over T4.

2)  The SMS-SC determines the trigger message identified by the External Identifier or MSISDN, SCS Identifier, and old trigger reference number in the received Device Trigger Replace message, is pending at SMS-SC: the new trigger message replaces the previous one.

3)	The SMS-SC triggers a Debit Units Request message to the OCS.

4)	The OCS performs the appropriate credit processing based on the received replace request.

5)	The OCS responds with a Debit Units Response message to the SMS-SC.

6)	The SMS-SC returns "Device Trigger Answer" with appropriate result of the successful or unsuccessful replace procedure.

7)	In case of successful replace, the new SM to be delivered uses the same procedure as per clause 5.3.2.11.2.

Next change

###### 5.3.2.12.3.2	MSISDN-less SMS MO via T4 - failure at the MTC-IWF

Figure 5.3.2.12.3.2.1 describes the scenario where MSISDN-less UE originates SMS-MO destined to a recipient SCS/AS using MSISDN-less SMS MO via T4 procedure, and failure at the MTC-IWF:

Figure 5.3.2.12.3.2.1; Online charging MSISDN-less SMS MO via T4 - failure at the MTC-IWF

1)	The SMS-SC receives a "SMS Submit" incoming message originated by a MSISDN-less UE to deliver small data to SCS/AS.

2)	The SMS-SC triggers a Reserve Units Request [Initial] message to the OCS.

3)	The OCS performs the appropriate credit processing based on the received request.

4)	The OCS responds with a Reserve Units Response [Initial] message to the SMS-SC.

5)	If authorized, the SMS-SC sends the "MO payload delivery Request" message to the MTC-IWF address (as pre-configured in the SMS-SC for this SCS/AS), with the SMS payload and the destination SME address (long/short code of the SCS/AS).

6)	Failure in handling the submitted SMS-MO in the MTC-IWF, including failure submission to the SCS/AS step 7 and 8 in Figure 5.3.2.11.2.1.

7)	The MTC-IWF returns the failure delivery indication to SMS-SC.

8)	The SMS-SC triggers a Debit Units Request [Terminate] message to the OCS, reporting the used unit for the service to zero.

9)	The OCS performs the appropriate credit processing based on the received request.

10)	The OCS responds with a Debit Units Response message to the SMS-SC.

11)	The SMS-SC indicates failure back to UE.

Next change

##### 6.2a.1.2.1	Charging Data Request message

Table 6.2a.1.2.1.1 illustrates the basic structure of a Charging Data Request message as used for SMS converged charging.

# Table 6.2a.1.2.1.1: Charging Data Request message contents

| Information Element | Category | Description |
| --- | --- | --- |
| Session Identifier | OC | Described in TS 32.290 [57] |
| Subscriber Identifier | OM | Described in TS 32.290 [57] |
| NF Consumer Identification | M | Described in TS 32.290 [57] |
| NF Functionality | M | Described in TS 32.290 [57] |
| NF Name | OC | Described in TS 32.290 [57] |
| NF Address | OC | Described in TS 32.290 [57] |
| NF PLMN ID | OC | Described in TS 32.290 [57] |
| Invocation Timestamp | M | Described in TS 32.290 [57] |
| Invocation Sequence Number | M | Described in TS 32.290 [57] |
| Retransmission Indicator | - | This field is not applicable. |
| One-time Event | OC | Described in TS 32.290 [57] |
| One-time Event Type | OC | Described in TS 32.290 [57] |
| Service Specification Information | OC | Described in TS 32.290 [57] |
| Notify URI | OC | This field is not applicable. |
| Triggers | - | This field is not applicable. |
| Multiple Unit Usage | OC | This field is present when the number of units is beyond one (i.e. more than one SMS) |
| Rating Group | M | Described in TS 32.290 [57] |
| Requested Unit | OC | Described in TS 32.290 [57] |
| Used Unit Container | OC | This field holds SMS charging information when more than one SMS. It may have multiple occurrences. |
| SMS Charging Information | OM | This field holds the SMS specific information described in clause 6.5.2 |

Next change

#### 6.2a.3.2	SMS charging CHF CDR data

If enabled, CHF CDRs for SMS charging shall be produced for SMS chargeable events.

The fields of SMS charging CHF CDR are specified in table 6.2a.3.2.1.

Table 6.2a.3.2.1: SMS charging CHF record data

| Field | Category | Description |
| --- | --- | --- |
| Record Type | M | Described in TS 32.298 [3] |
| Recording Network Function ID | OM | Described in TS 32.298 [3] |
| Subscriber Identifier | M | This field holds the 5G Subscription Permanent Identifier (SUPI) of the served party. |
| NF Consumer Information | M | This field holds the information of the SMSF that used the charging service. |
| NF Functionality | M | This field contains the function of the node (i.e. SMSF) |
| NF Name | OC | This field holds the name of the SMSF used. |
| NF Address | OC | This fields holds the IP Address of the SMSF used. |
| NF PLMN ID | Oc | This field holds the PLMN identifier (MCC MNC) of the SMSF. |
| Triggers | - | This field is not applicable. |
| List of Multiple Unit Usage | OM | This field is present when the number of units is beyond one (i.e. more than one SMS) |
| Record Opening Time | OC | Described in TS 32.298 [3] |
| Duration | M | Described in TS 32.298 [3] |
| Record Sequence Number | C | Described in TS 32.298 [3] |
| Cause for Record Closing | M | Described in TS 32.298 [3] |
| Local Record Sequence Number | OM | Described in TS 32.298 [3] |
| Record Extensions | OC | Described in TS 32.298 [3] |
| SMS Charging Information | OM | This field holds the SMS specific information defined in clause 6.5.2 |

Next change

### 6.5.3	Detailed message format for converged charging

The following clause specifies per Operation Type the charging data that are sent by SMS Node for 5G SMS converged charging.

The Operation Types are listed in the following order: I (Initial)/T (Termination)/E (Event). Therefore, when all Operation Types are possible it is marked as ITE. If only some Operation Types are allowed for a node, only the appropriate letters are used (i.e. IT or E) as indicated in the table heading. The omission of an Operation Type for a particular field is marked with "-" (i.e. I-E). Also, when an entire field is not allowed in a node the entire cell is marked as "-".

Table 6.5.3.1 defines the basic structure of the supported fields in the Charging Data Request message for SMS converged charging.

# Table 6.5.3.1: Supported fields in Charging Data Request message

| Information Element |  | Node Type | SMSF |  |
| --- | --- | --- | --- | --- |
|  |  | Supported Operation Types | ITE |  |
| Session Identifier |  |  | ITE |  |
| Subscriber Identifier |  |  | ITE |  |
| NF Consumer Identification |  |  | ITE |  |
| Invocation Timestamp |  |  | ITE |  |
| Invocation Sequence Number |  |  | ITE |  |
| Retransmission Indicator |  |  | - |  |
| One-time Event |  |  | --E |  |
| One-time Event Type |  |  | --E |  |
| Service Specification Information |  |  | ITE |  |
| Notify URI |  |  | - |  |
| Triggers |  |  | - |  |
| Multiple Unit Usage |  |  | ITE |  |
| Rating Group |  |  | IT- |  |
| Requested Unit |  |  | I-- |  |
| Used Unit Container |  |  | IT- |  |
| SMS Charging Information |  |  |  |  |
| Originator Info |  |  | ITE |  |
| Recipient Info |  |  | ITE |  |
| User Equipment Info |  |  | ITE |  |
| Roamer In Out |  |  | ITE |  |
| User Location Info |  |  | ITE |  |
| UE Time Zone |  |  | ITE |  |
| RAT Type |  |  | ITE |  |
| SMSC Address |  |  | ITE |  |
| SM Data Coding Scheme |  |  | ITE |  |
| SM Message Type |  |  | ITE |  |
| SM Reply Path Requested |  |  | ITE |  |
| SM User Data Header |  |  | ITE |  |
| SM Status |  |  | ITE |  |
| SM Discharge Time |  |  | ITE |  |
| Number of Messages Sent |  |  | ITE |  |
| SM Service Type |  |  | ITE |  |
| SM Sequence Number |  |  | ITE |  |
| SMS result |  |  | ITE |  |
| Submission Time |  |  | ITE |  |
| SMPriority |  |  | ITE |  |
| Message Reference |  |  | ITE |  |
| Message Size |  |  | ITE |  |
| Message Class |  |  | ITE |  |
| Delivery Report Requested |  |  | ITE |  |

Table 6.5.3.2 defines the basic structure of the supported fields in the Charging Data Response message for SMS converged charging.

# Table 6.5.3.2: Supported fields in Charging Data Response message

| Information Element | Node Type | SMSF |
| --- | --- | --- |
|  | Supported Operation Types | ITE |
| Session Identifier |  | ITE |
| Invocation Timestamp |  | ITE |
| Invocation Result |  | ITE |
| Invocation Sequence Number |  | ITE |
| Session Failover |  | I-- |
| Triggers |  | - |
| Multiple Unit information |  | I-E |
| Result Code |  | I-- |
| Rating Group |  | I-- |
| Granted Unit |  | I-- |
| Validity Time |  | I-- |

End of changes