3GPP TSG-SA5 Meeting #133-e	S5-205265

# Online, , 12th Oct 2020 - 21st Oct 2020

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.531 | CR | 0059 | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of allocateNsi operations and procedures |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson LM |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16 |  |  |  |  |  | Date: |  |  | 2020-10-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The allocateNsi operation currently only support service requirements from serviceProfile, not considering network slice requirement, nor the capabilities of already deployed network slices. The allocateNsi operation therefore needs to be corrected, and the procedure clarified. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The allocateNsi input and output operations are updated to support serviceProfile, additionalServiceProfileDataList and additionalNetworkSliceDataList. The deallocateNsi input operation is updated so that serviceProfileId is the input parameter. The “Procedure of Network Slice Instance Allocation” of clause 7.2 are updated to reflect that the input requirements to an allocation must be compared/matched against the actual capabilities (in networkSliceCapabilities) from possible network slices. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It will be unclear how to allocate/deallocate an service to a network slice instance. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.1, 6.5.3, 7.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Proposal discussed in S5-205268, S5-205261 have TS 28.541 updates. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

## 6.5.	Operations of provisioning

### 6.5.1	AllocateNsi operation

#### 6.5.1.1	Description

This operation is invoked by allocateNsi operation service consumer to request the provider to allocate a network slice instance to satisfy network slice related requirements. The provider may create a new NSI or using existing NSI to satisfy the request.

#### 6.5.1.2	Input parameters

| Parameter Name | Support Qualifier | Information Type / Legal Values | Comment |
| --- | --- | --- | --- |
| serviceProfile | M | LIST OF SEQUENCE< attribute name, attribute value> | This parameter specifies the network slice related requirements defined in ServiceProfile in Clause 6.3.3 in TS 28.541 [6]. |
| additionalServiceProfileDataList | O | LIST OF SEQUENCE< attribute name, attribute value> | This parameter specifies additional requirements related to the additionalNetworkSliceDataList in the ServiceProfile. |
| additionalNetworkSliceDataList | O | LIST OF SEQUENCE< attribute name, attribute value> | This parameter specifies additional requirements related to the additionalNetworkSliceDataList in the NetworkSlice. |

#### 6.5.1.3	Output parameters

| Parameter name | Support Qualifier | Matching Information / Legal Values | Comment |
| --- | --- | --- | --- |
| serviceProfile | M | LIST OF SEQUENCE< attribute name, attribute value> | This parameter specifies the network slice related requirements which has been allocated and the actual value assigned to each. |
| additionalServiceProfileDataList | O | LIST OF SEQUENCE< attribute name, attribute value> | This parameter specifies additional requirements related to the ServiceProfile which has been allocated and the actual value assigned to each. |
| additionalNetworkSliceDataList | O | LIST OF SEQUENCE< attribute name, attribute value> | This parameter specifies additional data related to the NetworkSlice which has been allocated and the actual value assigned to each. |
| status | M | ENUM (OperationSucceeded, OperationFailed) | An operation may fail because of a specified or unspecified reason. |
| nSId | M | An attribute uniquely identifies the network slice instance. | It specifies the unifique identifier of the NSI which has been allocated. |

Next change

### 6.5.3	DeallocateNsi operation

#### 6.5.3.1	Description

This operation is invoked by deallocateNsi operation service consumer to request to remove a service (identified by its serviceProfileId identifier) that is no longer  needed for the consumer. The provider may terminate the requested NSI or modify the requested NSI without termination to satisfy the request.

#### 6.5.3.2	Input parameters

| Parameter Name | Support Qualifier | Information Type / Legal Values | Comment |
| --- | --- | --- | --- |
| nSId | M | An attribute uniquely identifies the network slice instance. | It specifies the unifique identifier of the NSI which need to be deallocated. |
| serviceProfileId | M | An attribute uniquely identifies the serviceProfile instance. | It specifies the unifique identifier of the serviceProfile instance which need to be deallocated. |

#### 6.5.3.3	Output parameters

| Parameter name | Support Qualifier | Matching Information / Legal Values | Comment |
| --- | --- | --- | --- |
| status | M | ENUM (OperationSucceeded, OperationFailed) | An operation may fail because of a specified or unspecified reason. |

Next change

## 7.2	Procedure of Network Slice Instance Allocation

The Figure 7.2-1 illustrates the procedure of creating a new NSI or using an existing NSI to satisfy the required network slice related requirements.

Figure 7.2-1: Network Slice Instance Allocation Request procedure

1)	Network Slice Management Service Provider (NSMS_Provider) receives an AllocateNsi request (see AllocateNsi operation defined in clause 6.5.1) from Network Slice Management Service Consumer (NSMS_Consumer) with network slice related requirements defined as the attributes in the ServiceProfile see clause 6.3.3 in TS 28.541 [6] and the additional network slice requirements defined in the additionalNetworkSliceDataList and additionalServiceProfileDataList.

2)	Based on the network slice related requirements and the knowledge over the capabilities (from networkSliceCapabilities) for existing deployed network slices, the NSMS_Provider decides whether to use an existing NSI or create a new NSI. If the network slice related requirements allow the requested NSI to be shared and if an existing suitable NSI can be reused, the NSMS_Provider may decide to use the existing NSI.

3a) If using an existing NSI and the existing NSI needs to be modified to satisfy the network slice related requirements, the NSMS_Provider invokes the procedure to modify the existing NSI as described in clause 7.6.

3b-1) If creating a new NSI, the NSMS_Provider derives the network slice subnet related requirements from the received network slice related requirements. Before NSMS_Provider derives the network slice subnet related requirements, NSMS_Provider may invoke corresponding network slice subnet capability information querying procedure as described in clause 7.8.

3b-2)	The NSMS_Provider invokes the NSSI allocation procedure as described in clause 7.3.

3b-3)	The NSMS_Provider creates the MOI for NSI and configures the MOI with the DN of MOI for the NSSI, other configuration information may be configured for the created MOI.

Note:	The detailed configuration information is described in network slice NRM (see NetworkSlice IOC defined in clause 6.3.1 in TS 28.541 [6]).

4) The NSMS_Provider sends NSI allocation result (see AllocateNsi operation defined in clause 6.5.1) to the NSMS_Consumer. If an existing NSI is modified or a new NSI is created successfully to satisfy the network slice related requirements, the result includes the relevant network slice instance information (see NetworkSlice IOC defined in clause 6.3.1 in TS 28.541 [6]):

-	DN of the MOI for NSI.

Otherwise the result may include the reason of failure, for example, the required latency or user number cannot be satisfied, or the physical resource is not enough.

End of change