- 3GPP TSG SA WG5 Meeting #133-e	S5-205133

- October 12 – 21, 2020, e-Meeting	revision of S5-20abcd

- Source:	Intel

- Title:	pCR 28.814 use case of EAS configuration

- Document for:	Approval

Agenda Item:	6.5.3

# 1	Decision/action requested

The group is asked to discuss and approve the proposals.

# 2	References

[1] 	TR 28.814 “Study on enhancements of edge computing management”

[2] 	S5-204034 "LS on Application Registration in relation to management aspects"

[3] 	3GPP TS 23.558 “Architecture for enabling Edge Applications”"

# 3	Rationale

In the EAS registration procedure, as described in clause 8.4.3.2.1 in TS 28.558 [3], the EAS sends a registration request with the EAS profile shown below, to the EES. SA6 sent a LS [2] to SA5 asking that some of the EAS profile information elements may require inputs from the management plane. This contribution proposes the use case fo EAS configuration to enable consumers to configure the EAS profile.

Table 8.2.4-1: Edge Application Server Profile

| Information element | Status | Description |
| --- | --- | --- |
| EAS ID | M | The identifier of the EAS |
| EAS Endpoint | M | Endpoint information (e.g. URI, FQDN, IP address) used to communicate with the EAS. This information maybe discovered by EEC and exposed to Application Clients so that application clients can establish contact with the EAS. |
| Application Client ID(s) | O | Identifies the Application Client(s) that can be served by the EAS |
| EAS Provider Identifier | O | The identifier of the EAS Provider |
| EAS Type | O | The category or type of EAS (e.g. V2X) |
| EAS description | O | Human-readable description of the EAS |
| EAS Schedule | O | The availability schedule of the EAS (e.g. time windows) |
| EAS Service Area | O | The geographical service area that the EAS serves |
| EAS Service KPIs | O | Service characteristics provided by EAS, detailed in Table 8.2.5-1 |
| Service continuity support | O | Indicates if the EAS supports service continuity or not. |
| EAS Availability Reporting Period | O | The availability reporting period (i.e. heart beat period) that indicates to the EES how often it needs to check the EAS's availability after a successful registration. |
| EAS Required Service APIs | O | A list of the Service APIs that are required by the EAS |
| EAS Status | O | The status of the EAS (e.g. enabled, disabled, etc.) |

# 4	Detailed proposal

1st modified section

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.558 “Architecture for enabling Edge Applications”.

[3]	3GPP TR 23.758 “Study on application architecture for enabling Edge”.

[4]					3GPP TS 23.501 “System architecture for the 5G System (5GS)”.

[5]					3GPP TS 28.531 “Manageemnt and orchestration; Provisioning”.

2nd modified section

# 6	Use cases and potential requirements

## 6.2	Specification level use cases and requirements

### 6.2.x	EAS Profile configuration

#### 6.x.1.1	Goal

In the EAS registration procedure (see clause 8.4.3.2.1 in TS 28.558 [2]), the EAS sends a registration request with the EAS profile to the EES. Some information elements in the EAS profile may need to be be configured by OAM. The goal of this use case is to enable consumers to configure the information related to EAS profile present in EASFunction MOI(s)..

##### 6.x.1.2	Pre-condition

- The EAS VNF instance has been instantiated, and the EAS Function MOI representing the EAS VNF instance have been created.

- The EASFunction MOI contains the EAS profile information

##### 6.x.1.3	Description

1.	A consumer requests the 3GPP management system to configure the EAS profile attributes.

2.	The 3GPP management system configures the attributes in the EASFunction MOI.

3.	The 3GPP management system sends an attribute change notification to the the consumer.

##### 6.x.1.4	Requirements

REQ-EAS-CONFIG-FUN-2: The provisioning service producer should have the capability allowing authorized consumer to configure the EAS profile attributes in the EASFunction MOI(s).

REQ-EAS-CONFIG-FUN-3: The provisioning service producer should have the capability to send a notification to the consumer, indicating that the attributes have been changed.

End of modified section