- 3GPP TSG SA WG5 Meeting #133-e	S5-205036

- October 12 – 21, 2020

- Source:	Samsung

- Title:	pCR 28.814 Use Case of Edge Performance Assurance

- Document for:	Approval

Agenda Item:	6.5.3

# 1	Decision/action requested

The group is asked to discuss and approve the proposals.

# 2	References

[1] 	TR 28.814 “Study on enhancements of edge computing management”

[2] 	3GPP TS 23.558: "Architecture for enabling Edge Applications;"

# 3	Rationale

# 4	Detailed proposal

1st modified section

### 6.2.x	Edge Performance Assurance

#### 6.2.x.1	Goal

The goal is to measure the performance of the edge components including EES and ECS and enable corrective actions in case of performance degradation.

##### 6.2.x.2	Description

Performance assurance of edge components need to be provided by the management system. EES are considered to be deployed in the operator (e.g ECSP, PLMN operator) premises at the edge of the network whereas ECS is considered to be deployed at a central location serving multiple EES. This use case aims to provide provisions for performance assurance of EES and ECS.

As part of performance evaluation, various edge related performance measurements are collected, based on which the performance assurance is enabled.

##### 6.2.1.2	Requirements

REQ-EDGE-PER-FUN-1: The 3GPP management service producer shall have the capability to provide EES performance assurance based on various edge related performance measurements e.g virtual resource utilization, successful EAS and EEC registration, successful EES service API consumption, successful EAS discovery and successful EEC subscription.

REQ-EDGE-PER-FUN-2: The 3GPP management service producer shall have the capability to provide ECS performance assurance based on various edge related performance measurements e.g virtual resource utilization, successful service provisioning, successful EES registration and successful EEC subscriptions

End of modified section