- 3GPP TSG SA WG5 Meeting #132-e	S5-205135

- August 17 – 28, 2020, e-Meeting	revision of S5-20abcd

- Source:	Intel

- Title:	pCR 28.814 add solutions for ECSP and PLMN deployment

- Document for:	Approval

Agenda Item:	6.5.3

# 1	Decision/action requested

The group is asked to discuss and approve the proposals.

# 2	References

[1] 	TR 28.814 v0.0.0 “Study on enhancements of edge computing management”

# 3	Rationale

This contribution proposes to add the solutions for the ECSP and PLMN deployment to TR 28.814 [1].

# 4	Detailed proposal

1st modified section

# 7	Potential solutions

Editor’s note: this clause is to describe the potential solution to support orchestration and management of edge computing.

## 7.x	The deployment of EDN and mobile networks

### 7.x.1	General

This subclause provides potential solutions for use cases of the deployment of EAS, EES, and ECS to PLMN (see subclause 6.2). It focuses on the deployment of EAS, EES, and ECS to 5GC network functions. Figure 7.x.1-1 shows an example of implementing the edge computing networks (see Figure 4.1-2) that is realized by the composite NS #1containing the nested NS #2 and NS #3. The EDN network functions are connected to 5GC network functions, by virtual links via SAP (Service Access Point).

Figure 7.x.1-1: The realization of edge computing networks in network services

### 7.x.2	Deployment with a single management system

Figure 7.x.2-1 depicts a scenario where ASP consumes the MnS of the ECPL management system that leverages the ETSI NFV MANO to deploy the VNFs for the edge computing networks, as shown in Figure 7.x.1-1.

Figure 7.x.2-1: Deployment of ECSP and PLMN with ECPL management system

1, ASP consumes the MnS to request ECPL management system to instantiate the EAS, by providing the EAS LCM IE (e.g. EAS software image location, service area, …etc) (see the EAS lifecycle management solution in subclause 7.x).

2, 3GPP management system notifies ASP that the EAS instantiation is in progress.

3. 3GPP management system downloads the EAS VNF software image from the EAS software image location.

4. The consumer of 3GPP consumes the provisioning MnS with operation createMOI of the EES VNF IOC with EES LCM IE (e.g. EES software image location, …).

5. 3GPP management system analyses the EES VNF IOC, and downloads the EES VNF software image from the EES software image location,

6. 3GPP management system requests NFVO via the Os-Ma-nfvo interface to create NS #3 with NSD and VNFD to contain EAS and EES VNF instances.

7. NFVO sends a notification to 3GPP management system to indicate that the NS #3 has been created.

8. 3GPP management system creates the MOIs for the EAS and EES VNFs, and sends the MOI creation notification to the consumer of 3GPP management system.

9. 3GPP management system analyses the EAS LCM IE (e.g. service area) to determine where the mobile networks should be deployed based on the EAS services.

10. 3GPP management system investigates if an NS, containing UPF VNF, PCF VNF, …etc., is available to connect to EAS VNF and EES VNF. If such NS is available, then continue; otherwise the following operation is executed:

10.1 3GPP management system requests NFVO via the Os-Ma-nfvo interface to create NS #2 with NSD and VNFD to contain UPF, PCF, and ECS instances.

10.2 NFVO sends a notification to 3GPP management system to indicate that the NS #2 has been created.

10.3 3GPP management system creates the MOIs for the UPF, PCF, and ECS VNFs, and sends the MOI creation notification to the consumer of ECPL.

11. 3GPP management system requests NFVO via the Os-Ma-nfvo interface to create the composite NS #1 with NSD#1 containing nested NSD #2 and NSD #3 to connect VNF instances in the EDN and mobile networks.

12. 3GPP management system notifies ASP that the EAS instance has been instantiated.

NOTE: It is assumed that consumers of ASP and 3GPP management system have subscribed to receive notifications from ECSP management system

End of modified section