3GPP TSG-SA5 Meeting #133-e	S5-205158rev1

- Online, , 12th Oct 2020 - 21st Oct 2020

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.532 | CR | 0152 | rev | - | Current version: | 16.5.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-16 CR 28.532 Correct ThresholdLevelInd (REST SS, OpenAPI definition) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16,REST_SS |  |  |  |  |  | Date: |  |  | 2020-10-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The definition of the threshold used in alarm notifications needs to be clarified, and the OpenAPI definition corrected. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The definition of the threshold used in alarm notifications is clarified, and the OpenAPI definition corrected. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The threshold definition is ambiguous. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 12.2.1.4.1a.1, 12.2.1.4.1a.2, 12.2.1.4.1a.3, A.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First modification

###### 12.2.1.4.1a.1	Type ThresholdHysteresis

Table 12.2.1.4.1a.1-1: Definition of type ThresholdHysteresis

| Attribute name | Data type | Description | S |
| --- | --- | --- | --- |
| high | oneOf(integer, Float) | Higher value of a threshold with hysteris, the integer type is used for counter thresholds and the float type for gauge thresholds. | M |
| low | Float | Lower value of a threshold with hysteresis, absent for counter thresholds. | M |

###### 12.2.1.4.1a.2	Type ThresholdLevelInd

Table 12.2.1.4.1a.2-1: Definition of type ThresholdLevelInd

| Attribute name (choice) | Data type | Description | S |
| --- | --- | --- | --- |
| up | ThresholdHysteresis | Indicates for counter and gauge thresholds that the threshold crossing occurred when going up. | M |
| down | ThresholdHysteresis | Indicates for gauge thresholds that the threshold crossing occurred when going down, not permitted for counter thresholds. | M |

###### 12.2.1.4.1a.3	Type ThresholdInfo

Table 12.2.1.4.1a.3-1: Definition of type ThresholdInfo

| Attribute name | Data type | Description | S |
| --- | --- | --- | --- |
| observedMeasurement | string | The name of the monitored measurement that crossed the threshold and that caused the notification (Rec. ITU-T X. 733 [4]). | M |
| observedValue | oneOf(integer, Float) | The value of the gauge or counter which crossed the threshold. This may be different from the threshold value if, for example, the gauge may only take on discrete values. The integer type is used for counters and the float type for gauges (Rec. ITU-T X. 733 [4]). | M |
| thresholdLevel | ThresholdLevelInd | In the case of a gauge the threshold level specifies a pair of threshold values, the first being the value of the crossed threshold and the second, its corresponding hysteresis; in the case of a counter the threshold level specifies only the threshold value (Rec. ITU-T X. 733 [4]). | O |
| armTime | DateTime | For a gauge threshold, the time at which the threshold was last re-armed, namely the time after the previous threshold crossing at which the hysteresis value of the threshold was exceeded thus again permitting generation of notifications when the threshold is crossed. For a counter threshold, the later of the time at which the threshold offset was last applied, or the time at which the counter was last initialized (for resettable counters) (Rec. ITU-T X. 733 [4]). | O |

Next modification

## A.2.1	OpenAPI document "faultMnS.yaml"

openapi: 3.0.1

info:

title: Fault Supervision MnS

version: 16.5.0

description: >-

OAS 3.0.1 definition of the Fault Supervision MnS

© 2020, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 28.532 V16.5.0; Generic management services

url: http://www.3gpp.org/ftp/Specs/archive/28_series/28.532/

servers:

- url: '{MnSRoot}/FaultSupervisionMnS/{version}'

variables:

MnSRoot:

description: See subclause 4.4.3 of TS 32.158

default: http://example.com/3GPPManagement

version:

description: Version number of the OpenAPI definition

default: XXX

paths:

/alarms:

get:

summary: Retrieve multiple alarms

description: >-

Retrieves the alarms identified by alarmAckState, baseObjectInstance

and filter.

parameters:

- name: alarmAckState

in: query

required: false

schema:

$ref: '#/components/schemas/AlarmAckState'

- name: baseObjectInstance

in: query

required: false

schema:

$ref: 'comDefs.yaml#/components/schemas/Dn'

- name: filter

in: query

required: false

schema:

$ref: 'comDefs.yaml#/components/schemas/Filter'

responses:

'200':

description: >-

Success case ("200 OK").

Returns the alarms identified in the request. The alarmId is the key

of the map.

content:

application/json:

schema:

type: object

additionalProperties:

type: object

allOf:

- type: object

properties:

lastNotificationHeader:

$ref: 'comDefs.yaml#/components/schemas/NotificationHeader'

- $ref: '#/components/schemas/AlarmRecord'

- type: object

properties:

comments:

$ref: '#/components/schemas/Comments'

default:

description: Response in case of error.

content:

application/json:

schema:

$ref: 'comDefs.yaml#/components/schemas/ErrorResponse'

patch:

summary: 'Clear, acknowledge or unacknowledge multiple alarms'

description: >-

Clears, acknowledges or unacknowledges multiple alarms using patch. Depending

on which action is to be performed, different merge patch documents need

to be used.

requestBody:

description: >-

Patch documents for acknowledging and unacknowledging, or clearing multiple

alarms. The keys in the map are the alarmIds to be patched.

content:

application/merge-patch+json:

schema:

oneOf:

- type: object

additionalProperties:

$ref: '#/components/schemas/MergePatchAcknowledgeAlarm'

- type: object

additionalProperties:

$ref: '#/components/schemas/MergePatchClearAlarm'

responses:

'204':

description: >-

Success case ("204 No content").

The response message body is empty.

default:

description: Response in case of error.

content:

application/json:

schema:

type: array

items:

$ref: '#/components/schemas/FailedAlarm'

/alarms/alarmCount:

get:

summary: Get the alarm count per perceived severity

parameters:

- name: alarmAckState

in: query

required: false

schema:

$ref: '#/components/schemas/AlarmAckState'

- name: filter

in: query

required: false

schema:

type: string

responses:

'200':

description: >-

Success case ("200 OK").

The alarm count per perceived severity is returned.

content:

application/json:

schema:

$ref: '#/components/schemas/AlarmCount'

default:

description: Response in case of error. The error case needs rework.

content:

application/json:

schema:

$ref: 'comDefs.yaml#/components/schemas/ErrorResponse'

/alarms/{alarmId}:

patch:

summary: 'Clear, acknowledge or unacknowledge a single alarm'

description: >-

Clears, acknowledges or uncknowldeges a single alarm by patching the alarm

information. A conditional acknowledge request based on the perceived

severity is not supported.

parameters:

- name: alarmId

in: path

description: Identifies the alarm to be patched.

required: true

schema:

type: string

requestBody:

required: true

content:

application/merge-patch+json:

schema:

oneOf:

- $ref: '#/components/schemas/MergePatchAcknowledgeAlarm'

- $ref: '#/components/schemas/MergePatchClearAlarm'

responses:

'204':

description: >-

Success case (204 No content).

The response message body is absent.

default:

description: Response in case of error.

content:

application/json:

schema:

$ref: 'comDefs.yaml#/components/schemas/ErrorResponse'

/alarms/{alarmId}/comments:

post:

summary: Add a comment to a single alarm

description: >-

Adds a comment to an alarm identified by alarmId. The id of the new comment

is allocated by the producer.

parameters:

- name: alarmId

in: path

description: Identifies the alarm to which the comment shall be added.

required: true

schema:

type: string

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/Comment'

responses:

'201':

description: >-

Success case (201 Created).

The representation of the newly created comment resource shall be returned.

content:

application/json:

schema:

$ref: '#/components/schemas/Comment'

headers:

Location:

description: URI of the newly created comment resource.

required: true

schema:

type: string

default:

description: Error case.

content:

application/json:

schema:

$ref: 'comDefs.yaml#/components/schemas/ErrorResponse'

/subscriptions:

post:

summary: Create a subscription

description: >-

To create a subscription the representation of the subscription is

POSTed on the /subscriptions collection resource.

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/Subscription'

responses:

'201':

description: >-

Success case ("201 Created").

The representation of the newly created subscription resource shall

be returned.

content:

application/json:

schema:

$ref: '#/components/schemas/Subscription'

headers:

Location:

description: URI of the newly created subscription resource

required: true

schema:

type: string

default:

description: Error case.

content:

application/json:

schema:

$ref: 'comDefs.yaml#/components/schemas/ErrorResponse'

callbacks:

notifyNewAlarm:

'{request.body#/consumerReference}':

post:

requestBody:

required: true

content:

application/json:

schema:

oneOf:

- $ref: '#/components/schemas/NotifyNewAlarm'

- $ref: '#/components/schemas/NotifyNewSecAlarm'

responses:

'204':

description: >-

Success case ("204 No Content").

The notification is successfully delivered. The response message

body is absent.

default:

description: Error case.

content:

application/json:

schema:

$ref: 'comDefs.yaml#/components/schemas/ErrorResponse'

notifyClearedAlarm:

'{request.body#/consumerReference}':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/NotifyClearedAlarm'

responses:

'204':

description: >-

Success case ("204 No Content").

The notification is successfully delivered. The response message

body is absent.

default:

description: Error case.

content:

application/json:

schema:

$ref: 'comDefs.yaml#/components/schemas/ErrorResponse'

notifyChangedAlarm:

'{request.body#/consumerReference}':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/NotifyChangedAlarm'

responses:

'204':

description: >-

Success case ("204 No Content").

The notification is successfully delivered. The response message

body is absent.

default:

description: Error case.

content:

application/json:

schema:

$ref: 'comDefs.yaml#/components/schemas/ErrorResponse'

notifyChangedAlarmGeneral:

'{request.body#/consumerReference}':

post:

requestBody:

required: true

content:

application/json:

schema:

oneOf:

- $ref: '#/components/schemas/NotifyChangedAlarmGeneral'

- $ref: '#/components/schemas/NotifyChangedSecAlarmGeneral'

responses:

'204':

description: >-

Success case ("204 No Content").

The notification is successfully delivered. The response message

body is absent.

default:

description: Error case.

content:

application/json:

schema:

$ref: 'comDefs.yaml#/components/schemas/ErrorResponse'

notifyCorrelatedNotificationChanged:

'{request.body#/consumerReference}':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/NotifyCorrelatedNotificationChanged'

responses:

'204':

description: >-

Success case ("204 No Content").

The notification is successfully delivered. The response message

body is absent.

default:

description: Error case.

content:

application/json:

schema:

$ref: 'comDefs.yaml#/components/schemas/ErrorResponse'

notifyAckStateChanged:

'{request.body#/consumerReference}':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/NotifyAckStateChanged'

responses:

'204':

description: >-

Success case ("204 No Content").

The notification is successfully delivered. The response message

body is absent.

default:

description: Error case.

content:

application/json:

schema:

$ref: 'comDefs.yaml#/components/schemas/ErrorResponse'

notifyComments:

'{request.body#/consumerReference}':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/NotifyComments'

responses:

'204':

description: >-

Success case ("204 No Content").

The notification is successfully delivered. The response message

body is absent.

default:

description: Error case.

content:

application/json:

schema:

$ref: 'comDefs.yaml#/components/schemas/ErrorResponse'

notifyPotentialFaultyAlarmList:

'{request.body#/consumerReference}':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/NotifyPotentialFaultyAlarmList'

responses:

'204':

description: >-

Success case ("204 No Content").

The notification is successfully delivered. The response message

body is absent.

default:

description: Error case.

content:

application/json:

schema:

$ref: 'comDefs.yaml#/components/schemas/ErrorResponse'

notifyAlarmListRebuilt:

'{request.body#/consumerReference}':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/NotifyAlarmListRebuilt'

responses:

'204':

description: >-

Success case ("204 No Content").

The notification is successfully delivered. The response message

body is absent.

default:

description: Error case.

content:

application/json:

schema:

$ref: 'comDefs.yaml#/components/schemas/ErrorResponse'

/subscriptions/{subscriptionId}:

delete:

summary: Delete a subscription

description: >-

The subscription is deleted by deleting the corresponding subscription

resource. The resource to be deleted is identified with the path

component of the URI.

parameters:

- name: subscriptionId

in: path

description: Identifies the subscription to be deleted.

required: true

schema:

type: string

responses:

'204':

description: >-

Success case ("204 No Content").

The subscription resource has been deleted. The response message body

is absent.

default:

description: Error case.

content:

application/json:

schema:

$ref: 'comDefs.yaml#/components/schemas/ErrorResponse'

components:

schemas:

#---- Definition of AlarmRecord ----------------------------------------------------#

AlarmId:

type: string

AlarmType:

type: string

enum:

- COMMUNICATIONS_ALARM

- QUALITY_OF_SERVICE_ALARM

- PROCESSING_ERROR_ALARM

- EQUIPMENT_ALARM

- ENVIRONMENTAL_ALARM

- INTEGRITY_VIOLATION

- OPERATIONAL_VIOLATION

- PHYSICAL_VIOLATION

- SECURITY_SERVICE_OR_MECHANISM_VIOLATION

- TIME_DOMAIN_VIOLATION

ProbableCause:

description: >-

The value of the probable cause may be a specific standardized string, or any

vendor provided string. Probable cause strings are not standardized in the

present document. They may be added in a future version. Up to then the

mapping of the generic probable cause strings "PROBABLE_CAUSE_001" to

"PROBABLE_CAUSE_005" is vendor specific.

The value of the probable cause may also be an integer. The mapping of integer

values to probable causes is vendor specific.

oneOf:

- anyOf:

- type: string

enum:

- PROBABLE_CAUSE_001

- PROBABLE_CAUSE_002

- PROBABLE_CAUSE_003

- PROBABLE_CAUSE_004

- PROBABLE_CAUSE_005

- type: string

- type: integer

SpecificProblem:

oneOf:

- type: string

- type: integer

PerceivedSeverity:

type: string

enum:

- INDETERMINATE

- CRITICAL

- MAJOR

- MINOR

- WARNING

- CLEARED

TrendIndication:

type: string

enum:

- MORE_SEVERE

- NO_CHANGE

- LESS_SEVERE

ThresholdHysteresis:

type: object

required:

- high

properties:

high:

oneOf:

- type: integer

- $ref: 'comDefs.yaml#/components/schemas/Float'

low:

$ref: 'comDefs.yaml#/components/schemas/Float'

ThresholdLevelInd:

oneOf:

- type: object

properties:

up:

$ref: '#/components/schemas/ThresholdHysteresis'

- type: object

properties:

down:

$ref: '#/components/schemas/ThresholdHysteresis'

ThresholdInfo:

type: object

required:

- observedMeasurement

- observedValue

properties:

observedMeasurement:

type: string

observedValue:

oneOf:

- type: integer

- $ref: 'comDefs.yaml#/components/schemas/Float'

thresholdLevel:

$ref: '#/components/schemas/ThresholdLevelInd'

armTime:

$ref: 'comDefs.yaml#/components/schemas/DateTime'

CorrelatedNotification:

type: object

required:

- source

- notificationId

properties:

sourceObjectInstance:

$ref: 'comDefs.yaml#/components/schemas/Dn'

notificationIds:

type: array

items:

$ref: 'comDefs.yaml#/components/schemas/NotificationId'

CorrelatedNotifications:

type: array

items:

$ref: '#/components/schemas/CorrelatedNotification'

AckState:

type: string

enum:

- ACKNOWLEDGED

- UNACKNOWLEDGED

AlarmRecord:

description: >-

The alarmId is not a property of an alarm record. It is used as key

in the map of alarm records instead.

type: object

properties:

# alarmId:

#  $ref: '#/components/schemas/AlarmId'

objectInstance:

$ref: 'comDefs.yaml#/components/schemas/Dn'

notificationId:

$ref: 'comDefs.yaml#/components/schemas/NotificationId'

alarmRaisedTime:

$ref: 'comDefs.yaml#/components/schemas/DateTime'

alarmChangedTime:

$ref: 'comDefs.yaml#/components/schemas/DateTime'

alarmClearedTime:

$ref: 'comDefs.yaml#/components/schemas/DateTime'

alarmType:

$ref: '#/components/schemas/AlarmType'

probableCause:

$ref: '#/components/schemas/ProbableCause'

specificProblem:

$ref: '#/components/schemas/SpecificProblem'

perceivedSeverity:

$ref: '#/components/schemas/PerceivedSeverity'

backedUpStatus:

type: boolean

backUpObject:

$ref: 'comDefs.yaml#/components/schemas/Dn'

trendIndication:

$ref: '#/components/schemas/TrendIndication'

thresholdinfo:

$ref: '#/components/schemas/ThresholdInfo'

correlatedNotifications:

$ref: '#/components/schemas/CorrelatedNotifications'

stateChangeDefinition:

$ref: 'comDefs.yaml#/components/schemas/AttributeValueChangeSet'

monitoredAttributes:

$ref: 'comDefs.yaml#/components/schemas/AttributeNameValuePairSet'

proposedRepairActions:

type: string

additionalText:

type: string

additionalInformation:

$ref: 'comDefs.yaml#/components/schemas/AttributeNameValuePairSet'

rootCauseIndicator:

type: boolean

ackTime:

$ref: 'comDefs.yaml#/components/schemas/DateTime'

ackUserId:

type: string

ackSystemId:

type: string

ackState:

$ref: '#/components/schemas/AckState'

clearUserId:

type: string

clearSystemId:

type: string

serviceUser:

type: string

serviceProvider:

type: string

securityAlarmDetector:

type: string

#---- Definition of alarm notifications --------------------------------------------#

AlarmNotificationType:

type: string

enum:

- notifyNewAlarm

- notifyChangedAlarm

- notifyChangedAlarmGeneral

- notifyAckStateChanged

- notifyCorrelatedNotificationChanged

- notifyComments

- notifyClearedAlarm

- notifyAlarmListRebuiltAlarm

- notifyPotentialFaultyAlarmList

AlarmListAlignmentRequirement:

type: string

enum:

- ALIGNMENT_REQUIRED

- ALIGNMENT_NOT_REQUIRED

NotifyNewAlarm:

allOf:

- $ref: 'comDefs.yaml#/components/schemas/NotificationHeader'

- type: object

required:

- alarmId

- alarmType

- probableCause

- perceivedSeverity

properties:

alarmId:

$ref: '#/components/schemas/AlarmId'

alarmType:

$ref: '#/components/schemas/AlarmType'

probableCause:

$ref: '#/components/schemas/ProbableCause'

specificProblem:

$ref: '#/components/schemas/SpecificProblem'

perceivedSeverity:

$ref: '#/components/schemas/PerceivedSeverity'

backedUpStatus:

type: boolean

backUpObject:

$ref: 'comDefs.yaml#/components/schemas/Dn'

trendIndication:

$ref: '#/components/schemas/TrendIndication'

thresholdInfo:

$ref: '#/components/schemas/ThresholdInfo'

correlatedNotifications:

$ref: '#/components/schemas/CorrelatedNotifications'

stateChangeDefinition:

$ref: 'comDefs.yaml#/components/schemas/AttributeValueChangeSet'

monitoredAttributes:

$ref: 'comDefs.yaml#/components/schemas/AttributeNameValuePairSet'

proposedRepairActions:

type: string

additionalText:

type: string

additionalInformation:

$ref: 'comDefs.yaml#/components/schemas/AttributeNameValuePairSet'

rootCauseIndicator:

type: boolean

NotifyNewSecAlarm:

allOf:

- $ref: 'comDefs.yaml#/components/schemas/NotificationHeader'

- type: object

required:

- alarmId

- alarmType

- probableCause

- perceivedSeverity

- serviceUser

- serviceProvider

- securityAlarmDetector

properties:

alarmId:

$ref: '#/components/schemas/AlarmId'

alarmType:

$ref: '#/components/schemas/AlarmType'

probableCause:

$ref: '#/components/schemas/ProbableCause'

perceivedSeverity:

$ref: '#/components/schemas/PerceivedSeverity'

correlatedNotifications:

$ref: '#/components/schemas/CorrelatedNotifications'

additionalText:

type: string

additionalInformation:

$ref: 'comDefs.yaml#/components/schemas/AttributeNameValuePairSet'

rootCauseIndicator:

type: boolean

serviceUser:

type: string

serviceProvider:

type: string

securityAlarmDetector:

type: string

NotifyClearedAlarm:

allOf:

- $ref: 'comDefs.yaml#/components/schemas/NotificationHeader'

- type: object

required:

- alarmId

- alarmType

- probableCause

- perceivedSeverity

properties:

alarmId:

$ref: '#/components/schemas/AlarmId'

alarmType:

$ref: '#/components/schemas/AlarmType'

probableCause:

$ref: '#/components/schemas/ProbableCause'

perceivedSeverity:

$ref: '#/components/schemas/PerceivedSeverity'

correlatedNotifications:

$ref: '#/components/schemas/CorrelatedNotifications'

clearUserId:

type: string

clearSystemId:

type: string

NotifyChangedAlarm:

allOf:

- $ref: 'comDefs.yaml#/components/schemas/NotificationHeader'

- type: object

required:

- alarmId

- alarmType

- probableCause

- perceivedSeverity

properties:

alarmId:

$ref: '#/components/schemas/AlarmId'

alarmType:

$ref: '#/components/schemas/AlarmType'

probableCause:

$ref: '#/components/schemas/ProbableCause'

perceivedSeverity:

$ref: '#/components/schemas/PerceivedSeverity'

NotifyChangedAlarmGeneral:

allOf:

- $ref: 'comDefs.yaml#/components/schemas/NotificationHeader'

- type: object

required:

- alarmId

- alarmType

- probableCause

- perceivedSeverity

- changedAlarmAttributes

properties:

alarmId:

$ref: '#/components/schemas/AlarmId'

alarmType:

$ref: '#/components/schemas/AlarmType'

probableCause:

$ref: '#/components/schemas/ProbableCause'

specificProblem:

$ref: '#/components/schemas/SpecificProblem'

perceivedSeverity:

$ref: '#/components/schemas/PerceivedSeverity'

correlatedNotifications:

$ref: '#/components/schemas/CorrelatedNotifications'

backedUpStatus:

type: boolean

backUpObject:

$ref: 'comDefs.yaml#/components/schemas/Dn'

trendIndication:

$ref: '#/components/schemas/TrendIndication'

thresholdInfo:

$ref: '#/components/schemas/ThresholdInfo'

stateChangeDefinition:

$ref: 'comDefs.yaml#/components/schemas/AttributeValueChangeSet'

monitoredAttributes:

$ref: 'comDefs.yaml#/components/schemas/AttributeNameValuePairSet'

proposedRepairActions:

type: string

additionalText:

type: string

additionalInformation:

$ref: 'comDefs.yaml#/components/schemas/AttributeNameValuePairSet'

rootCauseIndicator:

type: boolean

changedAlarmAttributes:

$ref: 'comDefs.yaml#/components/schemas/AttributeNameValuePairSet'

NotifyChangedSecAlarmGeneral:

allOf:

- $ref: 'comDefs.yaml#/components/schemas/NotificationHeader'

- type: object

required:

- alarmId

- alarmType

- probableCause

- perceivedSeverity

- serviceUser

- serviceProvider

- securityAlarmDetector

- changedAlarmAttributes

properties:

alarmId:

$ref: '#/components/schemas/AlarmId'

alarmType:

$ref: '#/components/schemas/AlarmType'

probableCause:

$ref: '#/components/schemas/ProbableCause'

perceivedSeverity:

$ref: '#/components/schemas/PerceivedSeverity'

correlatedNotifications:

$ref: '#/components/schemas/CorrelatedNotifications'

additionalText:

type: string

additionalInformation:

$ref: 'comDefs.yaml#/components/schemas/AttributeNameValuePairSet'

rootCauseIndicator:

type: boolean

serviceUser:

type: string

serviceProvider:

type: string

securityAlarmDetector:

type: string

changedAlarmAttributes:

$ref: 'comDefs.yaml#/components/schemas/AttributeNameValuePairSet'

NotifyCorrelatedNotificationChanged:

allOf:

- $ref: 'comDefs.yaml#/components/schemas/NotificationHeader'

- type: object

required:

- alarmId

- correlatedNotifications

properties:

alarmId:

$ref: '#/components/schemas/AlarmId'

correlatedNotifications:

$ref: '#/components/schemas/CorrelatedNotifications'

rootCauseIndicator:

type: boolean

NotifyAckStateChanged:

allOf:

- $ref: 'comDefs.yaml#/components/schemas/NotificationHeader'

- type: object

required:

- alarmId

- alarmType

- probableCause

- perceivedSeverity

- ackState

- ackUserId

properties:

alarmId:

$ref: '#/components/schemas/AlarmId'

alarmType:

$ref: '#/components/schemas/AlarmType'

probableCause:

$ref: '#/components/schemas/ProbableCause'

perceivedSeverity:

$ref: '#/components/schemas/PerceivedSeverity'

ackState:

$ref: '#/components/schemas/AckState'

ackUserId:

type: string

ackSystemId:

type: string

NotifyComments:

allOf:

- $ref: 'comDefs.yaml#/components/schemas/NotificationHeader'

- type: object

required:

- alarmId

- alarmType

- probableCause

- perceivedSeverity

- comments

properties:

alarmId:

$ref: '#/components/schemas/AlarmId'

alarmType:

$ref: '#/components/schemas/AlarmType'

probableCause:

$ref: '#/components/schemas/ProbableCause'

perceivedSeverity:

$ref: '#/components/schemas/PerceivedSeverity'

comments:

$ref: '#/components/schemas/Comments'

NotifyPotentialFaultyAlarmList:

allOf:

- $ref: 'comDefs.yaml#/components/schemas/NotificationHeader'

- type: object

required:

- reason

properties:

reason:

type: string

NotifyAlarmListRebuilt:

allOf:

- $ref: 'comDefs.yaml#/components/schemas/NotificationHeader'

- type: object

required:

- reason

properties:

reason:

type: string

alarmListAlignmentRequirement:

$ref: '#/components/schemas/AlarmListAlignmentRequirement'

#---- Definition of query parameters -----------------------------------------------#

AlarmAckState:

type: string

enum:

- ALL_ALARMS

- ALL_ACTIVE_ALARMS

- ALL_ACTIVE_AND_ACKNOWLEDGED_ALARMS

- ALL_ACTIVE_AND_UNACKNOWLEDGED_ALARMS

- ALL_CLEARED_AND_UNACKNOWLEDGED_ALARMS

- ALL_UNACKNOWLEDGED_ALARMS

#---- Definition of patch documents ------------------------------------------------#

MergePatchAcknowledgeAlarm:

description: >-

Patch document acknowledging or unacknowledging a single alarm. For

acknowleding an alarm the value of ackState is ACKNOWLEDGED, for unacknowleding

an alarm the value of ackState is UNACKNOWLEDGED.

type: object

required:

- ackUserId

- ackState

properties:

ackUserId:

type: string

ackSystemId:

type: string

ackState:

$ref: '#/components/schemas/AckState'

MergePatchClearAlarm:

description: Patch document for clearing a single alarm

type: object

required:

- clearUserId

- perceivedSeverity

properties:

clearUserId:

type: string

clearSystemId:

type: string

perceivedSeverity:

type: string

enum:

- CLEARED

#---- Definition of method responses -----------------------------------------------#

FailedAlarm:

type: object

required:

- alarmId

- failureReason

properties:

alarmId:

$ref: '#/components/schemas/AlarmId'

failureReason:

type: string

#---- Definition of resources ------------------------------------------------------#

AlarmCount:

type: object

required:

- criticalCount

- majorCount

- minorCount

- warningCount

- indeterminateCount

- clearedCount

properties:

criticalCount:

type: integer

majorCount:

type: integer

minorCount:

type: integer

warningCount:

type: integer

indeterminateCount:

type: integer

clearedCount:

type: integer

Comment:

type: object

properties:

commentTime:

$ref: 'comDefs.yaml#/components/schemas/DateTime'

commentUserId:

type: string

commentSystemId:

type: string

commentText:

type: string

Comments:

description: >-

Collection of comments. The comment identifiers are allocated by the

MnS producer and used as key in the map.

type: object

additionalProperties:

$ref: '#/components/schemas/Comment'

Subscription:

type: object

properties:

consumerReference:

$ref: 'comDefs.yaml#/components/schemas/Uri'

timeTick:

$ref: 'comDefs.yaml#/components/schemas/Long'

filter:

$ref: 'comDefs.yaml#/components/schemas/Filter'

End of modifications