| 3GPP TR 28.816 V0.1.0 (2020-10) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Charging management; Study on charging aspects of 5GS CIoT (Release 17) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2020, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

1	Scope	6

2	References	6

3	Definitions of terms, symbols and abbreviations	6

3.1	Terms	6

3.2	Symbols	7

3.3	Abbreviations	7

4	Concepts and background	7

4.1	General	7

5	Use cases and key issues	7

5.1	Use cases	7

5.1.1	Use case #1	7

5.2	Key issues	8

5.2.2	Key issues	8

5.2.2.1	Description	8

5.2.2.2	Potential requirements	8

6	Solutions	8

Annex A: Title	9

Annex X: Change history	9

Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

1	Scope

The present document investigates charging enhancements on 5G data connectivity domain charging, control plane data transfer domain charging and Monitoring Events charging for 5GS CIoT for the corresponding functionalities based on TS 23.501 [200], TS 32.251 [11], TS 32.255 [30], TS 32.253 [13] and TS 32.278 [38]:

- Control Plane CIoT 5GS Optimisation

- Non-IP Data Delivery (NIDD)

- Support for Monitoring Events

- Support of rate control of user data, including small data rate control

- Category M UEs differentiation

- Exception report

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[11]	3GPP TS 32.251: "Telecommunication management; Charging management; Packet Switched (PS) domain charging"

[200]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[201]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[202]				3GPP TS 23.503: "Policy and charging control framework for the 5G System (5GS); Stage 2".

3	Definitions of terms, symbols and abbreviations

3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Definition format (Normal)

<defined term>: <definition>.

example: text used to clarify abstract rules by applying them literally.

3.2	Symbols

For the purposes of the present document, the following symbols apply:

Symbol format (EW)

<symbol>	<Explanation>

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

Abbreviation format (EW)

<ABBREVIATION>	<Expansion>

4	Concepts and background

4.1	General

The Packet Switched (PS) domain charging supports charging of CIoT functionalities (e.g., the usage of the radio interface for CIoT, CP CIoT Optimisation and Serving PLMN Rate Control) as described in TS 32.251 [11].

CIoT functionality is provided by the visited and home networks when the networks are configured to support CIoT. It applies to both the non-roaming case and the roaming case and some functionality may be dependent upon the existence of appropriate roaming agreements between the operators.

In 5GS CIoT has been evolved and refers to 5GS optimisations and functionality for support of Cellular Internet-of-Things (Cellular IoT, or CIoT) described in TS 23.501 [200] according to service requirements described in TS 22.261 [102]. The specific functionality is described in the affected procedures and features of 5G service aspect, in TS 23.501 [200], TS 23.502 [201] and TS 23.503 [202].

5	Use cases and key issues

5.1	Use cases

5.1.1	Use case #1

This clause is used to provide general use case for charging enhancement for 5GS CIoT. It is recommended to provide additional use case for the particular charging domain(s) (e.g., 5GS data connectivity charging domain, control plane data transfer charging etc.) since those charging aspects are already described in charging specs.

5.2	Key issues

5.2.2	Key issues

This clause is used to provide key issue for charging enhancement(s) for 5GS CIoT. It is recommended to provide key issues regarding the particular charging domain(s) (e.g., 5GS data connectivity charging domain, control plane data transfer charging etc.) since those charging aspects are already described in charging specs.

5.2.2.1	Description

5.2.2.2	Potential requirements

6	Solutions

Annex A:
Title

Annex X:
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2020-09 | SA5#133e | S5-205030 |  |  |  | Initial skeleton. | 0.0.0 |
| 2020-10 | SA5#133e | S5-205465 S5-205466 S5-205229 |  |  |  | pCR 28.816 Add scope pCR 28.816 Add introduction pCR 28.816 Add proposal for skeleton for TR 28.816 | 0.1.0 |