3GPP TSG-SA5 Meeting #134e 	S5-206205

# e-meeting 16th - 25th November 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.535 | CR | draftCR | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Coordination between control loops |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | eCOSLA |  |  |  |  |  | Date: |  |  | 2020-11-2 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | There may be multiple control loops ongoing in the autonomous network. The control loops reside in different domains, including 3GPP Cross Management Domain, 5GC Management Domain, NG-RAN Management Domain, 5GC Domain and NG-RAN Domain etc. Control loops in different domains may need to interact for the overall network automation, e.g. 3GPP Cross Management Domain interact with 5GC Management Domain, NG-RAN Management Domain for assurance goals or assurance goal status . |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add description and requirements for coordination between control loops. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is not clear whether multiple parallel control loops are correlated. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.x(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This is input to the Rel-17 28.535 DraftCR. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st of changes

4.2.x	Coordination between control loops

Different control loops reside in management domains or network function to support the overall autonomous networks. The purposes and results of different control loops may have impacts on one another. Coordination between control loops are needed in the management, 5GC and NG-RAN domains, as shown in the figure 4.2.x-1.

A control loop may coordinate with other control loops in the same domain or in a different domain. Control loops in domain management are responsible for local optimization. Control loops in cross domain management may need to coordinate with control loops in multiple domains for the end to end optimizations.

The coordination between different control loops can be classified into management-based coordination and NF-based loops coordination. Management-based coordinations include the following categories:

- Coordination between Cross Management Domain and the 5GC Management Domain

- Coordination between Cross Management Domain and the NG-RAN Management Domain

- Coordination between 5GC Management Domain and the NG-RAN Management Domain

The NF-based loops coordination include the following categories:

- Coordination between a 5GC NF and a NG-RAN node

- Coordination between a 5GC NF and a 5GC NF

- Coordination between a NG-RAN node and a NG-RAN node

Management-based coordination provides the SLS assurance from the overall management perspective. NF-based coordination contributes to the SLS assurance from the network control perspective. The management-based coordination also provide governace and objective to the NF-based coordination.

NOTE: The NF-based coordination is out of scope of SA5. The description is to provide a complete view of the concept.

Figure 4.2.x-1: Coordination between control loops

2nd of changes

6.2	Requirements

REQ-CSA-CON-01 The 3GPP management system shall have the capability to take actions for a set of communication services serving certain group of UEs based on the target SLS.

REQ-CSA-CON-02 The 3GPP management system shall have the capability to collect service experience information.

REQ-CSA-CON-03 The 3GPP management system shall have the capability to analyse the performance information related to the set of communication services serving certain group of UEs.

REQ-CSA-CON-04 The 3GPP management system shall have the capability to modify the configuration parameters related to the set of communication services serving certain group of UEs.

REQ-CSA-CON-05	The 3GPP management system shall have the capability to collect NSI related data from one or more 5GC NF(s).

NOTE 1:	An example for NSI related data may be QoE data.

REQ-CSA-CON-06	The 3GPP management system shall have the capability to derive which communication service is associated to the QoE data from the collected NSI related QoE data.

REQ-CSA-CON-07	The 3GPP management system shall have the capability to ascertain SLS breach.

REQ-CSA-CON-08	The 3GPP management system shall have the capability to perform the root cause analysis (e.g., identifying the underlying reason) for an SLS breach.

REQ-CSA-CON-09	The 3GPP management system shall have the capability to take corrective actions against the root cause identified.

REQ-CSA-CON-10 The 3GPP management system shall have the capability to translate communicate service requirements to cross domain SLS goal and single domain SLS goal.

REQ-CSA-CON-11 The 3GPP management system shall have the capability to collect single domain SLS analysis as input to cross domain SLS analysis.

REQ-CSA-CON-12	The 3GPP management system shall have the capability to allow its authorized consumer to control the SLS assurance (e.g. specify the SLS to be assured, enable/disable, specify the assurance time and update the SLS assurance requirements).

REQ-CSA-CON-13	The 3GPP management system shall have the capability to allow its authorized consumer to obtain the SLS assurance progress information and fulfil information.

NOTE 2:	The management system refers to the producer of management service for SLS assurance.

REQ-CSA-CON-xx1 The 3GPP management system shall have the capability to allow coordinations between different control loops for SLS assurance.

REQ-CSA-CON-xx2 The 3GPP management system shall have the capability to allow cross domain to configure SLS goals for a single domain.

REQ-CSA-CON-xx3 The 3GPP management system shall have the capability to allow cross domain to configure policies for a single domain for SLS assurance.

REQ-CSA-CON-xx4 The 3GPP management system shall have the capability to collect single domain control loop status to cross domain for SLS analysis.

REQ-CSA-CON-xx5 The 3GPP management system shall have the capability to allow cross domain to intervene one or more steps of a single domain open control loop.

End of changes