3GPP TSG SA WG5 Meeting 134-e		S5-206060

- electronic meeting, online, 16th - 25th November 2020

- Source:	Huawei

- Title:	Suggested conclusion for Alarm incident analysis

- Document for:	Approval

Agenda Item:	6.5.1

# 1	Decision/action requested

The group is asked to approve the proposal.

# 2	References

[1]	3GPP TR 28.809: “Study on enhancement of Management Data Analytics (MDA)”

# 3	Rationale

The conclusion for Alarm incident analysis use case needs to be provided.

Additionally, The approved pCR(S5-204254) was not correctly implemented in the latest version of 28.809 (1.0.0). In clause 6.4.1 (Alarm incident analysis), the virtualized resource related information should be added in the CN domain table (6.4.1.3.3), not in the RAN domain table (6.4.1.3.2).

# 4	Detailed proposal

This contribution proposes to make the following changes in [1].

1st Change

### 6.4.1	Alarm incident analysis

#### 6.4.1.1	Use case

In 5G network, millions of alarms are generated due to the network complexity. Since the topological relations between different network elements and logical relations between different generated alarms, a series of alarms caused by a same root cause should be correlated with each other. In addition, the same root causes may give rise to the network performance deterioration. For example, an alarm in a lower layer of the communication protocol stack or an alarm related to virtualized resource in the virtualization deployment environment may cause multiple alarms in higher layers. Sequence of alarms may be generated in multiple domains along a communication link if one fault in the source domain occurs.

Large amount of alarms brings difficulties in network operation and maintenance. Therefore, the alarms and deteriorated performance measurements of same root cause should be correlated and analysed. Some ML models and algorithms may be used to group or filter the correlated alarms and indicate the root cause. Also, the historical alarms, performance measurements and network topology data can be utilized as the foreknowledge.

To improve the efficiency of network operation and maintenance, the MDAS producer is able to provide the analytics result including the root alarm or root cause by correlate and group the related alarms and performance measurements into an alarm incident.

#### 6.4.1.2	Potential requirements

REQ-ALARM_MDA-01:	The MDAS producer shall have a capability to provide the analytics report describing the alarm incident analysis.

REQ-ALARM_MDA-02:	The analytics report describing the alarm incident should include the following information:

-	Alarm incident Identifier

-	List of Correlated Alarms, performance measurements

-	The start time and end time of the Alarm incident

-	The root cause or root alarm of the Alarm incident

-	Severity level

-	Affected objects

-	Recommended actions

#### 6.4.1.3	Possible Solutions

##### 6.4.1.3.1	Solution description

The MDAS producer correlates and analyses the management data described in the following subclause to identify the alarm incident. The required data can be from RAN domain or CN domain or both. As the table in 6.4.1.3.4 shows, the analytics report is able to be provided by the MDAS producer to describe the root causes and recommendations of identified alarm incident. It can be a domain specific or cross domain analytics report. This procedure may be triggered by the request or periodically.

##### 6.4.1.3.2	Data required for alarm incident analysis for RAN domain

Alarm data and performance measurements from correlated logical and physical resources are able to be utilized to perform the alarm incident analysis. The data listed in following table are the potential management data for RAN domain used to construct the alarm incident.

Table 6.4.1.3.2-1: Potential data required for alarm incident analysis

| Data Category | Required Data |
| --- | --- |
| Alarm Data | The alarm information, e.g., the alarm of NG-U, the alarm of radio frequency unit, the alarm of cell outage, as specified in TS 28.532 [x]. The alarm information related to virtualized resource, see clause 8 of TS 28.545 [22]. |
| Performance measurements | The deteriorated performance or the abnormal performance measurements based on certain performance monitoring threshold. Following performance measurements may be used. UE throughput: The IP throughput of end users, see clause 5.1.1.3 of TS 28.552 [8]; PDCP Data Volume: The transmitted PDCP data volume, see clause 5.1.2.1 and 5.1.3.6 of TS 28.552 [8]; TB related measurements: The TB transmitted in a cell, see clause 5.1.1.7 of TS 28.552 [8]; CQI related measurements: the distribution of Wideband CQI (Channel Quality Indicator) reported by UEs in the cell, see clause 5.1.1.11 of TS 28.552 [8]; MCS related Measurements: the distribution of the MCS scheduled for PDSCH RB by NG-RAN, the distribution of the MCS scheduled for PUSCH RB by NG-RAN, see clause 5.1.1.12 in TS 28.552 [8]; RAN UE throughput: A KPI that shows how NG-RAN impacts the service quality provided to an end-user, see clause 6.3.6 of TS 28.554 [7]; Throughput for network slice instance: Upstream/Downstream throughput for network and Network Slice Instance, see clause 6.3.2 and clause 6.3.3 of TS 28.554 [7]; Throughput at N3 interface: Upstream/Downstream GTP data throughput at N3 interface, see clause 6.3.4 and clause 6.3.5 of TS 28.554 [7]; Inter-gNB handovers: Number of requested/successful handover/failed handover preparations/resource allocations/executions, see clause 5.1.1.6.1 of TS 28.552 [8]. Intra-gNB handovers: Number of requested/successful handover executions, see clause 5.1.1.6.2 of TS 28.552 [8]. RRC connection establishment related measurements: Attempted/ successful RRC connection establishments, see clause 5.1.1.15 of TS 28.552 [8]. RRC connection Re-establishment and RRC connection Resuming related measurement, see clause 5.1.1.17 and 5.1.1.18 of TS 28.552 [8]. Packet loss rate, Packet drop rate and Packet delay including the average delay and distribution of delay in CU-UP, F1-U, gNB-DU, CU-UP both of downlink and uplink. Virtualised resource usage measurement, see clause 6.2 of TS 28.552 [8]. |
| Configuration Data | The execution data including the changes or the configuration of the MOIs. |
| Network topology | The topology of the network deployment. The topology can be reflected by the NG-RAN NRMs as defined in TS 28.541 [20]. |

6.4.1.3.3	Data required for alarm incident analysis for CN domain

Alarm data and performance measurements from correlated logical and physical resources are able to be utilized to perform the alarm incident analysis. The data listed in following table are the potential management data for CN domain used to construct the alarm incident.

Table 6.4.1.3.3-1: Potential data required for alarm incident analysis

| Data Category | Required Data |
| --- | --- |
| Alarm Data | The alarm information, e.g., the alarm of AMF, the alarm of UPF, as specified in TS 28.532 [x]. The alarm information related to virtualized resource, see clause 8 of TS 28.545 [22]. |
| Performance measurements | The deteriorated performance or the abnormal performance measurements based on certain performance monitoring threshold. Following performance measurements may be used. Mobility related measurements for AMF: Number of requested/failed PDU sessions/QoS flows for inter-AMF handovers and number of attempted/successful/failed handover between 5GS and EPC, see clause 5.2.5.1, 5.2.5.3 and 5.2.5.4 of TS 28.552 [8]. Session related measurements for SMF: Number of successful/failed PDU session creation and number of requested/successful/failed PDU session modifications, see clause 5.3.1 of TS 28.552 [8]. QoS flow monitoring for SMF: Number of requested/successful/failed QoS flows to create/modify, see clause 5.3.2 of TS 28.552 [8]. Session related measurements for UPF: Number of requested/failed N4 session establishments, see clause 5.4.3.1 of TS 28.552 [8]. Throughput for network slice instance: Upstream/Downstream throughput for network and Network Slice Instance, see clause 6.3.2 and clause 6.3.3 of TS 28.554 [7]; Throughput at N3 interface: Upstream/Downstream GTP data throughput at N3 interface, see clause 6.3.4 and clause 6.3.5 of TS 28.554 [7]; Virtualised resource usage measurement, see clause 6.2 of TS 28.552 [8]. |
| Configuration Data | The execution data including the changes or the configuration of the MOIs. |
| Network topology | The topology of the network deployment. The topology can be reflected by the 5GC NRMs as defined in TS 28.541 [20]. |

##### 6.4.1.3.4	Analytics report for alarm incident analysis

Following table provides the potential information of the domain specific or cross domain analytics report for alarm incident analysis based on the required data received as described in 6.4.1.3.2 and 6.4.1.3.3.

| Analytics Report of alarm incident | Information | Description |
| --- | --- | --- |
|  | Alarm Incident Identifier | The alarm incident id or name for correlated alarms and performance measurements, e.g., NgU transmission alarm incident, Xn transmission alarm incident. |
|  | List of Correlated AlarmInfo | List of Alarm name or alarm ID, e.g., alarm of NgU setup, alarm of user plane link failure, alarm of user plane failure, alarm of cell outage. |
|  | List of Correlated performance measurements info | Performance measurements and the corresponding value, e.g., NgU handover failure rate, NgU setup failure rate. |
|  | Location | The geographical area or the cells where the alarm incident exists |
|  | Start Time | The start time of alarm incident |
|  | Stop Time | The end time of alarm incident |
|  | Affected objects | The MOIs, e.g., the MOIs of cells or subnetworks or network slices affected by the alarm incident |
|  | Root cause or Root alarm | Root alarm identified or predicted by root cause decision model, e.g. alarm of NgU setup, alarm of virtualized resource failure. |
|  | Severity level | The severity level (e.g., critical, medium, not important) of the alarm incident |
|  | Recommended actions | The recommend actions to clear the alarm incident. The recommended actions could be to replace the hardware unit, reconfigure the protocol, e.g., Xn application protocol |

##### 6.4.1.3.5	Evaluation

It is important to identify the root cause and resolve network malfunctions for the network to remain healthy.

The solution described in clause 6.4.2.3 requires the analytics inputs as described in in clause 6.4.1.3.2 and 6.4.1.3.3, wherein

-	the alarm Data are available in TS 28.532 [x]

- the performance measurements are available in TS 28.552 [8].

-	the network topology can be reflected by the NRMs defined in TS 28.541 [20].

-	the configuration data for NG-RAN and 5GC have been defined in TS 28.541 [20].

With these analytics inputs which are already defined or accessible, the analytics output as described in 6.4.1.3.4 can be derived.

Therefore, this solution is a feasible candidate for Alarm incident analysis for RAN domain, CN domain and cross domain network.

2nd Change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2] 	3GPP TS 28.550: "Management and orchestration; Performance assurance".

[3]	3GPP TS 28.533: "Management and orchestration; Architecture framework".

[4]	3GPP TS 28.530: "Management and orchestration; Concepts, use cases and requirements".

[5]	3GPP TR 28.861: "Study on the Self-Organizing Networks (SON) for 5G networks".

[6] 	3GPP TR 28.805: "Study on management aspects of communication services".

[7]	3GPP TS 28.554: "5G end to end Key Performance Indicators (KPI)".

[8]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[9]	3GPP TS 22.101: "service aspects; service principles".

[10]	3GPP TS 32.500: "Telecommunication management; Self-Organizing Networks (SON); Concepts and requirements".

[11]	3GPP TS 37.816: "Study on RAN-centric data collection and utilization for LTE and NR".

[12]	3GPP TS 37.320: "Radio measurement collection for Minimization of Drive Tests (MDT); Overall description".

[13]	3GPP TS 23.501: "System Architecture for the 5G System (5GS); Stage 2".

[14]	3GPP TS 28.310: "Energy efficiency of 5G".

[15]	3GPP TR 21.866: "Study on Energy Efficiency Aspects of 3GPP Standards".

[16]	3GPP TS 26.247: "Transparent end-to-end Packet-switched Streaming Service (PSS); Progressive Download and Dynamic Adaptive Streaming over HTTP (3GP-DASH)".

[17]	3GPP TS 26.114: "IP Multimedia Subsystem (IMS); Multimedia Telephony; Media handling and interaction".

[18]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[19]	3GPP TS 28.313: "Self-Organizing Networks (SON) for 5G networks".

[20]	3GPP TS 28.541: "Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3".

[21]	3GPP TS 38.304 NR: "User Equipment (UE) procedures in idle mode and in RRC Inactive state".

[22]	3GPP TS 28.545: " Management and orchestration; Fault Supervision (FS) ".

[x]	3GPP TS 28.532: "Management and orchestration; Generic management services".

End of changes