3GPP TSG SA WG5 Meeting 134-e 	TDoc S5-206062rev2

electronic meeting, online, 16th - 25th November 2020

- Source:	Huawei

- Title:	pCR 28.811 Add cross-operator network slicing scenario

- Document for:	Approval

Agenda Item:	6.5.2

# 1	Decision/action requested

The group is asked to approve the proposal.

# 2	References

[1]	3GPP TR 28.811 v0.1.0: “Management and orchestration; Network Slice Management Enhancement”

# 3	Rationale

This contribution adds scenario related to multi-operator network slicing and identifies possible problems.

# 4	Detailed proposal

This contribution proposes to make the following changes in [1].

1st Change

# 5	Scenarios for Network Slicing Management Enhancements

## 5.1	Network Slice covering multiple networks

#### 5.1.1	Description

CSP wishes to provide a communication service in multiple countries or regions. The CSP has selected NOP-A as network slice provider. NOP-A does not have RAN coverage in all of the geographical areas that are required by the CSP.

NOP-A selects NOP-B to provide RAN coverage for one or more geographical areas, and agrees slice subnet requirements with NOP-B.

NOP-A orders the allocation of a network slice subnet by NOP-B. NOP-A may issue the request to NOP-B’s 3GPP management system, or NOP-B may issue the request to its own 3GPP management system.

#### 5.1.2	Identified problems

The Network Slice instance is modelled as a set of MOIs which exist partially in NOP-A’s management system and partially in NOP-B’s management system.

NOP-A’s management system will contain a NetworkSlice MOI, the cross-domain NetworkSliceSubnet MOI, and the CN NetworkSliceSubnet MOI.

NOP-B’s management system will contain a NetworkSlice MOI and RAN NetworkSliceSubnet MOI.

As part of the provisioning phase, the cross-domain NetworkSliceSubnet MOI in NOP-A’s management system should be configured with a reference to the RAN NetworkSliceSubnet MOI in NOP-B’s management system.

In the operation phase, it is unclear whether NOP-A will be able to supervise and modify the network slice subnet and associated resources provided by NOP-B. NOP-A will be limited by security restrictions and by Exposure Governance restrictions.

Editor’s note: Further study is needed on the impact of restrictions on access to NOP-B’s management system, and the minimum operations capabilities needed for NOP-A to effectively operate the network slice.

End of changes