3GPP TSG SA WG5 Meeting 134-e 	S5-206074

- electronic meeting, online, 16th - 25th November 2020		     	          (revision of xx-yyxxxx)

- Source:	Ericsson

- Title:	New WID on GPB for File based Trace format in 5G

- Document for:	Approval

Agenda Item:	6.2

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

# Title: 	GPB for File based Trace Format in 5G

## Acronym: GPB_File

## Unique identifier: 	 {A number to be provided by MCC at the plenary}

Potential target Release: Rel-17

Note that this field above indicates the proposed Release at the time of submission of the WID to TSG approval. It can later be changed without a need to revise the WID. The updated target Release is indicated in the Work Plan.

## 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  |  | X | X |  |
| No | X | X |  |  |  |
| Don't know |  |  |  |  |  |

## 2	Classification of the Work Item and linked work items

### 2.1	Primary classification

This work item is a

| X | Feature |
| --- | --- |
|  | Building Block |
|  | Work Task |
|  | Study Item |

2.2 Parent Work Item

None.

### 2.3	Other related Work Items and dependencies

| Other related Work Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| SP-190782 | New WID Streaming trace reporting | Use solutions from this WI about PM streaming, format, protocol |

## 3	Justification

GPB for streaming trace format has been introduced and added in TS 32.423 in Rel-16. The work needs to be extended with GPB for file based trace format in Rel-17 so that the trace format for streaming trace and file based trace would be handled in a unified way. GPB trace format is not intended to replace the existing XML trace format.

## 4	Objective

To specify:

- Extend GPB Streaming trace format to support file based trace reporting. For the file based trace reporting, The file is encoded in GPB format and shall be transferred as file to MnS Consumer.

- Add new requirement in TS 32.421, Format of the Trace records shall be XML or GPB based on the Schema.

## 5	Expected Output and Time scale

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| TS 32.423 | update of the existing streaming trace format | TSG#90e (Dec. 2020) |  |

## 6	Work item Rapporteur(s)

Ayani, Zhulia, Ericsson, zhulia.Ayani@ericsson.com

## 7	Work item leadership

SA5.

## 8	Aspects that involve other WGs

None

## 9	Supporting Individual Members

Supporting IM name

Ericsson