3GPP TSG-SA5 Meeting #134-e	S5-206077

- November 16 – 25, 2020								        									      		s5-2abcde

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0273 | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add PCI related measurement |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | eSON_5G |  |  |  |  |  | Date: |  |  | 2020-11-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | This measurement is needed to support the PCI configuration SON function to detect PCI collision or confusion issues. A use case if A.58 Monitoring of PCI to detect PCI collision or confusion already exists, but there is no measurement. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the PCIs of neighboring cells measurement. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The centralized PCI configuration SON function cannot be monitored. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.x (new), A.58 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

1st Modified Section

# 5		Performance measurements for 5G Network Functions

## 5.x	Measurements related to UE Measurement Reports

### 5.x.1	PCIs of neighboring cells

a)	This measurement provides the PCI used by neighboring cells. The measurement is derived from MeasResultListNR IE in measResultNeighCells (see clause 6.3.2 in TS 38.331 [20]) where it contains PhysCellId and ssb-Index indicating the PCI value received at SSB beam(s). It contains subcounters per PCI value to indicate the number of MeasResultListNR with such PCI value being received.

b)	CC.

c)	This measurement is obtained by incrementing the subcounter, corresponding the PCI value PhysCellId received in MeasResultListNR IE in measResultNeighCells for the SSB identified by ssb-Index.

For example, MeasResultListNR contains multiple MeasResultNR with physCellId for PCI #5 and ssb-Index #1 and #2. Then, measurements for SSB #1 and SSB #2 with subcounter identified by PCI #5 in subcounters will be incremented.

d)	Each measurement is an integer value.

e)	MR.PCIsOfNeighCell.pciValue

Where pciValue identifies the subcounter associated with the PCI value.

f)	Beam.

g)	Valid for packet switched traffic.

h)	5GS.

i)	One usage of this measurement is to support centralized SON function of PCI re-configuration (see TS 28.313 [30]).

Next Modified Section

# A.58	Monitoring of PCI to detect PCI collision or confusion

Each NR cell is assigned a PCI that enables UE to uniquely identify the cell. PCI values need to be reused, as there are only 1008 PCI values. Typically, operators use network planning tool to assign PCIs to cells when the network is deployed to insure all neighboring cells have different PCIs. However, due to the addition of new cells or changes of neighbour relations from ANR functions, issues can arise, such as PCI collision, PCI confusion.

The measurement of PCI values for neighboring cells can be used by C-SON to detect potential PCI issues.  The example in Fig A.58.-1 shows that Cell-IV consists of 6 beams. PCIs of neighboring cells measurements for SSB #1, SSB #4, and SSB #6 are collected, where

- Measurement for SSB #1:

- Subcounter PCI=7: 2

- Subcounter PCI=9: 2

- Subcounter PCI=13: 3

- Measurement for SSB #4:

- Subcounter PCI=9: 4

- Measurement for SSB #6:

- Subcounter PCI=9: 2

- Subcounter PCI=7: 5

- Subcounter PCI=5: 3

PCI configuration analyses the measurements to find that measurements SSB #1 and SSB #6 contain subcounters of PCI values 7 and 9 that means the potential PCI confusion issues between Cell-XI and Cell-VI (using PCI 7), and between Cell-VIII and Cell-II (using PCI 9).

Figure A.58-1: PCI configuration example

End of Modified Sections