- 3GPP TSG-SA5 Meeting #134e	S5-206085rev3

- 16 to 25 November 2020, E-meeting

- Source:	Intel

- Title:	pCR Add charging scenarios and key issues on inter-provider charging for 5GS capabilities supporting EC

- Document for:	Approval

- Agenda Item:	7.5.1

# 1	Decision/action requested

The group is asked to discuss and agree on the proposal.

# 2	References

[1] 	3GPP TR 28.815 “Study on charging aspects of edge computing”

# 3	Rationale

As described in clause 7.1.1, the MNO may charge its industry consumer (ASP or ECSP) for the total 5GS capabilities provided for edge computing based on the data volume.

The total data volume can be collected and reported by management plane via MnS, therefore the inter-provider charging between the MNO and its consumer could be supported via management plane.

This pCR is to add the potential requirements and key issues on inter-provider charging for 5GS capabilities supporting EC to draft TR 28.815 [1].

# 4	Detailed proposal

1st Modified Section

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

ASP	Application Service Provider

EAS	Edge Application Server

CEF	Charging Enablement Function

ECS	Edge Configuration Server

ECSP	Edge Computing Service Provider

EDN	Edge Data Network

EES	Edge Enabler Server

MNO	Mobile Network Operator

MnS	Management Service

Next Modified Section

## 7.1	5GS charging for edge computing

### 7.1.1	General description and assumptions

5GS provides the capabilities to support edge computing, and the MNO may charge these 5GS capabilities in the following ways:

- 	Charge the individual subscribers for using the 5G capabilities supporting edge computing.

The subscriber charging could be based on data volume or in-session (PDU session) time that the subscriber has consumed/used in 5GS for accessing the edge applications, and potentially with consideration of the following factors:

-	User’s location;

-	Access RAT type;

-	Subscribed QoS information;

-	Authorized QoS information;

-	Charge the ASP and/or ECSP for the total 5GS capabilities provided for edge computing.

The inter-provider charging by MNO could be based on the total data volume transferred in the PLMN for the ASP and/or ECSP.

### 7.1.2	Potential charging requirements

REQ-CH_EC_5GS-01: The SMF should support the following extensions to the charging requirements defined in TS 32.255 [6]:

-	The SMF should support converged charging for each edge application, i.e., charging per rating group per PDU session with information about edge application.

-	The charging information of PDU session, service flow or QoS flow provided by SMF should be able to indicate or be correlated with the edge application that the user accesses to.

-	The SMF should support converged charging and charging information reporting for each edge application optionally based on the following criteria:

-	User’s location;

-	Access RAT type;

-	Subscribed QoS information;

-	Authorized QoS information.

REQ-CH_EC_5GS-02: The charging mechanism for 5GS (including the management system) should support converged inter-provider charging for the 5G capabilities provided to support edge computing to its consumer (e.g., ASP or ECSP) based on the total UL/DL data volume transferred for the edge application.

### 7.1.3	Key issues

The following key issues are identified:

-	Key Issue #1a: The charging information reported by SMF needs to contain the parameter for correlation between 5GS and edge application that the user has accessed to.

-	Key Issue #1b: The charging mechanism for 5GS (including the management system) needs to support converged inter-Provider charging for the 5G capabilities provided to support edge computing to its consumer based on the total UL/DL data volume transferred for the edge application .

7.1.4	Possible solutions

TBD

### 7.1.5	Evaluation

TBD

### 7.1.6	Conclusion

TBD……

End of Modified Sections