- 3GPP TSG-SA5 Meeting #134e	S5-206087rev4

- 16 to 25 November 2020, E-meeting

- Source:	Intel

- Title:	pCR 28.809 Update solutions on subscription and reporting of Management Data Analytics Reports

- Document for:	Approval

- Agenda Item:	6.5.1

# 1	Decision/action requested

The group is asked to discuss and agree on the proposal.

# 2	References

[1] 	3GPP TR 28.809 “Management and orchestration; Study on enhancement of Management Data Analytics (MDA)”

# 3	Rationale

The use case and solutions on subscription and reporting of Management Data Analytics Reports are described in TR 28.809 [1]. The streaming data reporting service and file reporting service defined in TS 28.532 are suitable for Management Data Analytics Reports reporting. And, the streaming data reporting service has already preserved to support streaming of analytics reporting since Rel-16.

However, it is not clearly described in solutions that how the Management Data Analytics Reports reporting is supported by the streaming data reporting service and file reporting service.

This pCR is to update the solutions on subscription and reporting of Management Data Analytics Report in connection with the generic streaming data reporting service and file reporting service defined in TS 28.532.

# 4	Detailed proposal

1st Modified Section

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2] 	3GPP TS 28.550: "Management and orchestration; Performance assurance".

[3]	3GPP TS 28.533: "Management and orchestration; Architecture framework".

[4]	3GPP TS 28.530: "Management and orchestration; Concepts, use cases and requirements".

[5]	3GPP TR 28.861: "Study on the Self-Organizing Networks (SON) for 5G networks".

[6] 	3GPP TR 28.805: "Study on management aspects of communication services".

[7]	3GPP TS 28.554: "5G end to end Key Performance Indicators (KPI)".

[8]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[9]	3GPP TS 22.101: "service aspects; service principles".

[10]	3GPP TS 32.500: "Telecommunication management; Self-Organizing Networks (SON); Concepts and requirements".

[11]	3GPP TS 37.816: "Study on RAN-centric data collection and utilization for LTE and NR".

[12]	3GPP TS 37.320: "Radio measurement collection for Minimization of Drive Tests (MDT); Overall description".

[13]	3GPP TS 23.501: "System Architecture for the 5G System (5GS); Stage 2".

[14]	3GPP TS 28.310: "Energy efficiency of 5G".

[15]	3GPP TR 21.866: "Study on Energy Efficiency Aspects of 3GPP Standards".

[16]	3GPP TS 26.247: "Transparent end-to-end Packet-switched Streaming Service (PSS); Progressive Download and Dynamic Adaptive Streaming over HTTP (3GP-DASH)".

[17]	3GPP TS 26.114: "IP Multimedia Subsystem (IMS); Multimedia Telephony; Media handling and interaction".

[18]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[19]	3GPP TS 28.313: "Self-Organizing Networks (SON) for 5G networks".

[20]	3GPP TS 28.541: "Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3".

[21]	3GPP TS 38.304 NR: "User Equipment (UE) procedures in idle mode and in RRC Inactive state".

[22]	3GPP TS 28.545: "Management and orchestration; Fault Supervision (FS)".

[x]	3GPP TS 28.532: "Management and orchestration; Generic management services".

Next Modified Section

### 6.99.2	Subscription to Management Data Analytics Reports

#### 6.99.2.1	Use case

A MDAS Producer may provide several management data analysis reports. Multiple consumers may wish to receive a selection of these reports.

The consumer submits a request to MDAS producer to subscribe to the MDA reports. This request may include a filter to specify the scope of MDA reports to be subscribed (e.g., type of analytics report such as coverage issue analysis, resource utilization analysis, the managed objects to be analysed, etc.). The MDAS producer activates the data collection if it is not already active. In the subscription, the consumer may indicate the method that the MDA reports are to be reported, i.e., by streaming data reporting for by file reporting.

For all reports, the MDAS producer collects data, analyses the data, and generates the analytics report.

The MDAS producer provides the MDA reports based on the reporting method designated in the subscription by the consumer.

The consumer may send a request to MDAS producer to unsubscribe from the MDA report.

#### 6.99.2.2	Potential requirements

REQ-MDA_SUB-1		The MDAS producer should have a capability to allow an MDAS consumer to subscribe to an analytics report. The subscription request should optionally allow the MDAS consumer to filter the scope of data in the analytics report.

REQ-MDA_SUB-2		The MDAS producer should have a capability to provide the analytics report to subscribed consumers.

REQ-MDA_SUB-3		The MDAS producer should have a capability to allow an MDAS consumer to unsubscribe to an analytics report.

#### 6.99.2.3	Possible solutions

The MDAS consumer sends the MDAReportSubscription request to MDAS producer, with the following information included:

-	identifier of the subscription;

-	reporting method, i.e., file reporting or streaming data reporting;

-	streaming target if the reporting method is designated to streaming data reporting;

-	file information (e.g., file location, ready time, expiration time, size, compression and format) if the reporting method is designated to file reporting;

-	filter for the scope of the MDA report (e.g., type of analytics report, managed objects to be analysed, etc.)

The MDAReportSubscription may be modelled as an IOC and managed via provisioning related operations (such as CreateMOI, ModifyMOI, DeleteMOI).

The MDAS producers provides a response indicating the status of the request.

For the MDA report subscription designating the reporting method of file reporting, the possible solution in connection with file reporting service defined in TS 32.532 [x] is shown in Figure 6.99.2.3-1.

Figure 6.99.2.3-1: MDA report subscription and MDA report file reporting

For the MDA report subscription designating the reporting method of streaming data reporting, the possible solution in connection with streaming data reporting service defined in TS 32.532 [x] is shown in Figure 6.99.2.3-2.

Figure 6.99.2.3-2: MDA report subscription and MDA report streaming

#### 6.99.2.4	Evaluation

The solution described in clause 6.99.2.3 reuses the generic streaming data reporting service and file reporting service defined in TS 28.532 [x] for MDA data reporting, and requires to a mechanism for MDA report subscription that can be defined in the normative work.

Therefore, this solution is a feasible candidate for MDA report subscription and reporting.

End of Modified Sections