- 3GPP TSG-SA5 Meeting #134e	S5-206091rev2

- 16 to 25 November 2020, E-meeting

- Source:	Intel

- Title:	pCR Update on MDA assisted energy saving

- Document for:	Discussion and endorsement

- Agenda Item:	6.5.1

# 1	Decision/action requested

The group is asked to discuss and agree on the proposal.

# 2	References

[1] 	3GPP TR 28.809 “Study on enhancement of Management Data Analytics (MDA)”

[2] 	3GPP TR 28.813 “Study on new aspects of Energy Efficiency (EE) for 5G”

# 3	Rationale

The UC and possible solution have been described for MDA assisted energy saving in TR 28.809 [1].

The network data analysis assisted energy saving has been studied in draft TR 28.813 [2], and the OA&M centric energy saving solution has been described.

This pCR is to align the MDA assisted energy saving in TR 28.809 [1] with the OA&M centric energy saving solution of the network data analysis assisted energy saving described in TR 28.813 [2].

The input on pre-trained prediction models can be supported by the ML model training, therefore it is removed from this UC.

# 4	Detailed proposal

1st Modified Section

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2] 	3GPP TS 28.550: "Management and orchestration; Performance assurance".

[3]	3GPP TS 28.533: "Management and orchestration; Architecture framework".

[4]	3GPP TS 28.530: "Management and orchestration; Concepts, use cases and requirements".

[5]	3GPP TR 28.861: "Study on the Self-Organizing Networks (SON) for 5G networks".

[6] 	3GPP TR 28.805: "Study on management aspects of communication services".

[7]	3GPP TS 28.554: "5G end to end Key Performance Indicators (KPI)".

[8]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[9]	3GPP TS 22.101: "service aspects; service principles".

[10]	3GPP TS 32.500: "Telecommunication management; Self-Organizing Networks (SON); Concepts and requirements".

[11]	3GPP TS 37.816: "Study on RAN-centric data collection and utilization for LTE and NR".

[12]	3GPP TS 37.320: "Radio measurement collection for Minimization of Drive Tests (MDT); Overall description".

[13]	3GPP TS 23.501: "System Architecture for the 5G System (5GS); Stage 2".

[14]	3GPP TS 28.310: "Energy efficiency of 5G".

[15]	3GPP TR 21.866: "Study on Energy Efficiency Aspects of 3GPP Standards".

[16]	3GPP TS 26.247: "Transparent end-to-end Packet-switched Streaming Service (PSS); Progressive Download and Dynamic Adaptive Streaming over HTTP (3GP-DASH)".

[17]	3GPP TS 26.114: "IP Multimedia Subsystem (IMS); Multimedia Telephony; Media handling and interaction".

[18]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[19]	3GPP TS 28.313: "Self-Organizing Networks (SON) for 5G networks".

[20]	3GPP TS 28.541: "Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3".

[21]	3GPP TS 38.304 NR: "User Equipment (UE) procedures in idle mode and in RRC Inactive state".

[22]	3GPP TS 28.545: "Management and orchestration; Fault Supervision (FS)".

[a]	3GPP TS 28.813: "Study on new aspects of Energy Efficiency (EE) for 5G".

[b]	3GPP TS 28.406: "Telecommunication management; Quality of Experience (QoE) measurement collection; Information definition and transport".

### 6.6.1	MDA assisted energy saving

#### 6.6.1.1	Use case

Energy saving is a critical issue for the 5G operators. Energy saving is achieved by activating the energy saving mode of the NR capacity booster cell or 5GC NF (e.g. UPF etc.), and the energy saving activation decision making may be based on the various information such as load information of the related cells/UPFs, the energy saving policies set by operators as specified in TS 28.310 [14] and the energy saving instructions provided by MDAS producer as described in TR 28.813 [a].

As the conclusion from clause 7.2 of the TR 21.866 [15], "The EE Control and Coordination Function: a self-managed automated process to control and coordinate system wide power saving operations including the access networks, core network, backhaul/fronthaul transmission networks, backbone networks and other subsystems", the management system has the overall view of network load information and it could also take the inputs from the control plane analysis (e.g. the analytics provided by NWDAF). The management system may provide the network wide analytics and cooperate with Core and RAN domains and decide on which cell/UPF should move into energy saving mode in a coordinated manner.

There are various performance measurements could be used as inputs by MDA for energy saving analysis, for example, the EE related performance measurements, (e.g. PDCP data volume of cells, PNF temperature, and PNF power consumption etc.) for the gNBs, and the data volume, number of PDU sessions with SSC mode 1 (see TS 23.501 [13]), delay related measurements, and VR usage for UPFs.

The composition of the traffic load could be also considered as inputs for energy saving analysis (e.g., the percentage of high-value traffic in the traffic load). The variation of traffic load may be related to the network data (e.g., historical handover information of the UEs or network congestion status). Collecting and analysing the network data with machine learning tools may provide predictions related to the trends of traffic load. The composition and the trend of the traffic load may be used as references for making decision on energy saving.

MDAS may also obtain NF location or other inventory information such as energy efficiency and the energy cost of the data centers, while analysing historical network information. Based on the collected information, MDAS producer makes analysis and gives suggestions to network management in optimization suggestion for 5G Core NF deployment options in high-value traffic region (e.g. location of VNF in context of energy saving). The information from control plane data analysis from NWDAF, such as UE Communication analytics (see TS 23.288 [18]), may also be used as input for energy saving analysis and instruction.

The decision of core NF and RAN node energy saving should be coordinated by management system to guarantee the overall network and service performance are not affected as much as possible.

To achieve an optimized balance between the energy consumed and the performance provided by the network, MDAS can be used to provide an analytics report by analysing the above information comprehensively to assist the energy saving.

#### 6.6.1.2	Potential requirements

REQ-MDA_ES-CON-1	The MDAS producer should have a capability to provide the analytics report describing the energy saving instruction.

REQ-MDA_ES-CON-2	The analytics report describing the energy saving instruction should contain the following information:

-	The identifier of the energy saving instruction described in the analytics report;

-	Root cause of the energy consumption issue;

-	Recommended NR Cells (ES-Cell) to enter energySaving state;

-  	Recommended candidate cells with precedence for taking over the traffic of each ES-Cell.

-	Recommended UPFs (ES-UPF) to enter energySaving state;

- 	Recommended candidate UPFs with precedence for taking over the traffic of each ES-UPF.

6.6.1.3	Possible solutions

##### 6.6.1.3.1	Solution description

The MDAS producer correlates and analyses the management data described in the following subclause to assist the management energy saving function to make energy efficiency decisions. As the table in clause 6.6.1.3.3 shows, the analytics report is able to be provided by the MDAS producer to describe the analytics result and recommendations of energy saving. This procedure may be triggered by the request or periodically.

Energy saving activation decision making may be based on the various information such as load information. The prediction result of these information can be used by operators to make energy-saving policies. There are many prediction models which may use ML algorithms for predicting these information, such as energy-saving scenarios prediction models and traffic load prediction models.

The prediction models are trained to be able to produce the expected training output from the training input. The data for models training may include RRC connection number, PRB utilization, energy consumption, service experience, etc. After the training process, the pre-trained models for predicting information used to make energy-saving policies can be obtained.

The more accurate the information prediction results are, the better the energy-saving policies based on the information prediction results will be. According to the result of information prediction and the energy-saving benefit, the MDA service can assist the energy saving policy decision making by recommending the optimal information prediction models which can provide more accurate information prediction results for consumers.

For example, the MDA service may take base station energy saving scenarios prediction model and the traffic load prediction model together as input. Based on the results of energy saving scenarios prediction and traffic load prediction, the MDA service may use the ML algorithms to calculate the energy-saving benefit based on base station related information (e.g., service experience changes), the traffic load changes as well as the prediction of the energy saving scenarios. And by maximizing the expected sum of energy-saving benefits, the MDA service may provide the optimal prediction models for making recommendation of base station energy saving policies accordingly.

##### 6.6.1.3.2	Data required for MDA assisted energy saving

The following table shows the potential data required to analyse the energy saving issue.

| Data Category | Required Data |
| --- | --- |
| Performance Measurements | PNF Power Consumption: Power consumed over the measurement period, see clause 5.1.1.19.2 of TS 28.552 [8]; PNF Energy consumption: The energy consumed, see clause 5.1.1.19.3 of TS 28.552 [8]; PNF Energy Temperature: The temperature over the measurement period, see clause 5.1.1.19.4 of TS 28.552 [8]; PNF Voltage: The voltage, see clause 5.1.1.19.5 of TS 28.552 [8]; PNF Current: The current, see clause 5.1.1.19.6 of TS 28.552 [8]; PNF humidity consumption: The percentage of humidity during the measurement period, see clause 5.1.1.19.7 of TS 28.552 [8]; PDCP Data Volume of NR cells: The transmitted PDCP data volume, see clause 5.1.2.1 and 5.1.3.6 of TS 28.552 [8]; Virtual resource usage of NF: The resource usage of virtual network functions, see clause 5.7.1 of TS 28.552 [8]. Data volume of UPF: see clause 5.4 of TS 28.552 [8]. Delay related measurements of UPF: see clause 5.4 of TS 28.552 [8]. Number of PDU sessions with SSC mode 1 on UPF. |
| QoE Data | The measurements that are collected are DASH (see TS 26.247 [16]) and MTSI (see TS 26.114 [17]) measurements (see TS 28.406 [b]). |
| NRM | MOIs of the cells, UPFs and SMFs, see TS 28.541 [20]. |
| Alarm information | The alarm information of the cells, UPFs and SMFs. |
| Network analytics data | The control plane analysis result from the NWDAF defined in TS 23.288 [18], e.g., observed service experience related network data analytics. |

6.6.1.3.3	Analytics report for MDA assist energy saving

The following table shows the potential information carried in the analytics report of MDA assist energy saving.

| Analytics Report of MDA assisted energy saving | Attribute Name | Description |
| --- | --- | --- |
|  | Energy saving analytics identifier | The identifier of the MDA assisted energy saving |
|  | Location | The geographical area, UPFs or the cells where the unreasonable energy consumption exists |
|  | Root cause | The root cause of the part of the energy consumption that may be conserved, e.g., ultra-low traffic load area with energy consumption, excessive energy consumption |
|  | Energy saving recommendations | For ES on NR cells. - recommended NR Cell (ES-Cell) to enter energySaving state. - recommended candidate cells with precedence for taking over the traffic of the ES-Cell. For ES on UPFs. It contains a set of - recommended UPF (ES-UPF) to conduct energy saving; - recommended candidate UPFs with precedence for taking over the traffic of the ES-UPF. |

#### 6.6.1.4	Evaluation

The solution described in clause 6.6.1.3 requires the analytics inputs as described in in clause 6.6.1.3.2, wherein

-	the performance measurements on number of PDU sessions with SSC mode 1 on UPF can be defined in TS 28.552 [8], while other measurements are already available in TS 28.552 [8].

-	QoE data are available in TS 28.406 [b].

-	NRMs are available in TS 28.541 [20].

-	the alarm information is supported by fault supervision MnS (see TS 28.545 [22] and TS 28.541 [20]).

-	the network analytics data can be accessed from NWDAF according to TS 23.288 [18].

With these analytics inputs which either are already defined or can be defined in the normative work, the analytics output as described in 6.6.1.3.3 can be derived.

End of Modified Sections