3GPP TSG SA WG5 Meeting 134-e		S5-206170

- electronic meeting, online, 16th - 25th November 2020

- Source:	Huawei, China Mobile,China Telecom

- Title:	Add use case and requirements for coverage optimization

- Document for:	Approval

Agenda Item:	6.4.1

# 1	Decision/action requested

The group is asked to discuss and approval.

# 2	References

[1]	3GPP draft TS 28.100: “Levels of autonomous network v0.1.0”.

[2]	3GPP TR 28.810: “Study on concept, requirements and solutions for levels of autonomous network v17.0.0”

# 3	Rationale

The Coverage optimization use case example for classification of network autonomy is described in clause 5.2.1 in TS 28.810[2], and the potential requirements for classification of network autonomy for coverage optimization is described in clause 6.2.3 in TS 28.810[2]. This contribution proposes to add use case and requirements for

# 4	Detailed proposal

It proposes to make the following changes to TS 28.100[1].

1st Change

### 5.X.1 Autonomous radio network coverage optimization

#### Editor’s Note: whether the following use case need to be generailized for common optimization scenarios is FFS.

#### 5.X.1.1	Introduction

Radio networks are geographically distributed, and mobile user activity varies significantly in different places and at different times of day. To achieve the optimal coverage, a set of initial coverage configuration parameters (e.g. coverageShape, digitalTilt and digitalAzimuth) may not always meet the requirements. Therefore, the coverage configuration parameters need to be adjusted in a differentiated manner with the change of the radio network environment. It’s almost not possible for manual adjustment due to multiple factors needs to be considered, e.g. interference control, huge data and frequent traffic changes, and also manual adjustment takes time. So it is benefits to introduce the autonomy technologies to achieve the optimal coverage configuration parameters automatically to deliver the best coverage for the complex wireless environments.

Full autonomy of coverage optimization for radio network is a long term goal, it will be beneficial for operator to achieve this goal step by step and have clear view on which typical issues can be addressed by utilizing autonomy mechanism in corresponding steps. The requirements for each level for autonomous radio network coverage optimization are different.

Editor’s Note: The detailed workflow, classification of autonomous network level and corresponding requirements are FFS.

End of changes