3GPP TSG SA WG5 Meeting 134-e		S5-206182

- electronic meeting, online, 16th - 25th November 2020

- Source:	Nokia

- Title:	New use case on slice coverage and throughput optimization

- Document for:	Approval

Agenda Item:	6.5.1

# 1	Decision/action requested

Propose to introduce a new use case and solution related to network slice coverage and throughput assessement in subclause 6.2.5

# 2	References

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2] 	3GPP TS 28.550: "Management and orchestration; Performance assurance".

[3]	3GPP TS 28.533: "Management and orchestration; Architecture framework".

[4]	3GPP TS 28.530: "Management and orchestration; Concepts, use cases and requirements".

[5]	3GPP TR 28.861: "Study on the Self-Organizing Networks (SON) for 5G networks".

[6] 	3GPP TR 28.805: "Study on management aspects of communication services".

[7]	3GPP TS 28.554: "5G end to end Key Performance Indicators (KPI)".

[8]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[9]	3GPP TS 22.101: "service aspects; service principles".

[10]	3GPP TS 32.500: "Telecommunication management; Self-Organizing Networks (SON); Concepts and requirements".

[11]	3GPP TS 37.816: "Study on RAN-centric data collection and utilization for LTE and NR".

[12]	3GPP TS 37.320: "Radio measurement collection for Minimization of Drive Tests (MDT); Overall description".

[13]	3GPP TS 23.501: "System Architecture for the 5G System (5GS); Stage 2".

[14]	3GPP TS 28.310: "Energy efficiency of 5G".

[15]	3GPP TR 21.866: "Study on Energy Efficiency Aspects of 3GPP Standards".

[16]	3GPP TS 26.247: "Transparent end-to-end Packet-switched Streaming Service (PSS); Progressive Download and Dynamic Adaptive Streaming over HTTP (3GP-DASH)".

[17]	3GPP TS 26.114: "IP Multimedia Subsystem (IMS); Multimedia Telephony; Media handling and interaction".

[18]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[19]	3GPP TS 28.313: "Self-Organizing Networks (SON) for 5G networks".

[20]	3GPP TS 28.541: "Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3".

[21]	3GPP TS 38.304 NR: "User Equipment (UE) procedures in idle mode and in RRC Inactive state".

[22]	3GPP TS 28.545: " Management and orchestration; Fault Supervision (FS) ".

[x]	GSMA NG.116, “Generic Network Slice Template”

# 3	Rationale

This contribution introduces a new use case and solution on slice coverage and throughput optimization.

# 4	Detailed proposal

First Change

### 6.2.5	Slice Coverage Optimization

#### 6.2.5.1	Use case

In providing a network slice, a 3rd party (i.e. slice tenant) issues a slice request indicating the desired SLA, which includes among other parameters as indicated in [x], the slice coverage (also referred to as coverage area of the network slice or area of service as per [x]). The main challenge of providing the slice coverage is to map the desired geographical coverage area with the available radio coverage, which depends on the base station planning and deployment. Hence, it is not guaranteed that the desired slice coverage can be mapped perfectly to the “coverage footprint” of a cell or set of cells or that the desired slice coverage is even available since radio coverage may not be available in certain areas.

Currently to resolve this problem more cells may be allocated to a slice to enhance the coverage, if such an option is available, or more capacity resources can be provisioned in the allocated cells. This approach may prove to be inefficient since it wastes network resources.

The distribution of users, the expected radio resource availability and mobility patterns may govern the configuration parameters of each cell (e.g. antenna downtilt, SSB beamforming patterns determining the coverage of a cell, handover parameters, etc.), which can be allocated per slice. Cell configuration parameters may help to adjust the coverage.

To translate the business slice coverage to the actual radio deployment, without overprovisioning while leveraging the benefits of flexible gNB radio features adjustment, MDA can be used. MDA can enable an MDAS consumer to optimize the slice coverage and load distibution on the slice instantiation and runtime considering (i) slice-aware statistics, e.g., slice-UE distributions and mobility patterns, (ii) slice SLA and (iii) access node capabilities.

Depending on the MDAS producer output, RAN parameters can be adjusted to shape the cell edges and load distribution. The main objective is to fulfill a given slice SLA involving as few cells as possible by leveraging the benefits of adjusting cell configurations for satisfying the desired coverage.

#### 6.2.5.2	Potential requirements

REQ-NS_COV_THR_OPT-1		The MDAS producer should have a capability to provide the analytics report describing the slice coverage, slice availability and slice prediction information to authorized consumers, e.g., gNB.

REQ-NS_COV_THR_OPT-2	The analytics report should contain the following information:

-	the identifier that indicates the gNB configuration issue with respect to a slice request;

-	 the geographical location and the affected cell configuration attributes;

- 	the start and stop time in where the report is valid;

-	the originator issue, that may be a coverage problem due to inefficient RAN configuration;

-	the recommended actions that involve options to reconfigure RAN attributes including HO parameters, cell reselection parameters, beam configuration, computing resource and slice support in a cell.

##### 6.2.5.3	Possible solutions

##### 6.2.5.3.1		Solution description

The solution considers slice coverage assessment with respect to RAN deployment. The MDAS producer analyzes performance measurements, slice availability and mobility patterns to identify the RAN attribute configuration options for adjusting selected cells in order to fulfill the SLA of a slice request in terms of the coverage.

##### 6.2.5.3.2		Data required

The following data is required to do the required analysis.

| Data category | Required data |
| --- | --- |
| Service Data | S-NSSAI as defined in clause 5.15.2, TS 23.501. MDAS may derive network topology information |
| Performance Measurements | Radio resource utilization – usage of physical radio resource utilization of the network as per clause 5.1.1.3 of TS 28.552. Virtual resource usage of NF: The resource usage of virtual network functions, see clause 5.7.1 of TS 28.552. |
| Beam Report per Slice | Beam statistics per slice usage in the serving cell: - Beam IDs (SSB IDs or CSI-RS IDs) which have been serving users of a specific network slice - Temporal Information of these Beams serving a specific Slice, (i.e., when, e.g., morning or evening, and how long) - Number of slice users served by a specific Beam ID Beam statistics of a neighboring cell measured by specific slice users: - Relevant neighboring Beam IDs (SSB IDs or CSI-RS IDs) which have been detected by users of a specific network slice |
| Capacity Planning Data | Capacity management as per clause 5.4.15 of TS 28.530 and clause 7.15 of TS 28.531. |
| Slice Unavailability Report | Slice unavailability in serving cell: - Statistics about the rejected or remapped PDU sessions belonging to a specific slice - How long the UEs stayed in a cell not supporting specific slices, e.g., short stay with Slice Unavailability - Temporal Information of the UEs stayed in a cell not supporting specific slices, e.g., when these events occurred - Number of UEs stayed in a cell not supporting specific slices - Cell IDs to which the UEs reconnect after staying in a cell not supporting specific slices, e.g., after short stay with Slice Unavailability |
| MDT Reports | UE measurements related to RSRPs, RSRQs, RLF, RECF of the cells in slice and UE location information |
| NWDAF Mobility Analytics | Group UE mobility analytics as per TS 23.288 All UE mobility analytics within Area of Interest defined via slice coverage as per TS 23.288 |
| Configuration Data | NRM attributes affecting the radio configuration and virtual NF resource allocation and configuration |
| Network Topology | Topology of the network |

##### 6.2.5.3.3		Analytics report

The network congestion analytics report contains the following information.

| Information | Description |
| --- | --- |
| gNB Incident Identifier | Identifier that indicates the gNB configuration case for slice coverage enhancement (or slice unavailability) |
| Type of Analytics | Statistics or Prediction |
| Location | Geographical location affected by the gNB incident |
| Affected Object Attributes | Cell Configurations: Antenna Tilt, HO parameters, cell reselection parameters, beam configuration, compute resources, etc. |
| Start/Stop Time | Starts/stop time of the incident |
| Root Cause | The originator of the issue, e.g. user mobility, load peak, user distribution, beam configuration, etc. |
| Severity Level | The severity level (e.g. critical, medium, not important) |
| Recommended Actions | Recommendation actions to resolve the issue: - Antenna Tilt configuration options - HO parameters configuration options - Cell reselection configuration options - Beam configuration options - Compute resource configuration options - Enable slice support in determined Cell(s) |

#### 6.2.5.4	Evaluation

The solution described in clause 6.2.5.3.1 requires the analytics inputs as described in clause 6.2.5.3.2, wherein:

- Input PM on RAN resource utilization, Virtual resource usage of NF are specified in TS 28.552 [8].

- Input PM that need to be specified include:

  - Beam statistics per slice usage.

  - Slice unavailability related measuements.

- MDT data is specified in TS 37.320 [12].

- NWDAF mobility analytics are specified in TS 23.288 [18].

- Configuration Data and network topology data, by the NRMs defined in TS 28.541 [20].

With these analytics inputs which are already defined or accessible, the analytics output as described in 6.2.5.3.3 can be derived.

Therefore, this solution is a feasible candidate for slice covrage optimization.

End of Change