3GPP TSG SA WG5 Meeting 134-e		S5-206207

- electronic meeting, online, 16th - 25th November 2020

- Source:	Nokia

- Title:	Gap analysis for inter-gNB beam selection Optimization

- Document for:	Approval

Agenda Item:	6.5.1

# 1	Decision/action requested

Add gap analysis for inter-gNB beam selection optimization.

# 2	References

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2] 	3GPP TS 28.550: "Management and orchestration; Performance assurance".

[3]	3GPP TS 28.533: "Management and orchestration; Architecture framework".

[4]	3GPP TS 28.530: "Management and orchestration; Concepts, use cases and requirements".

[5]	3GPP TR 28.861: "Study on the Self-Organizing Networks (SON) for 5G networks".

[6] 	3GPP TR 28.805: "Study on management aspects of communication services".

[7]	3GPP TS 28.554: "5G end to end Key Performance Indicators (KPI)".

[8]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[9]	3GPP TS 22.101: "service aspects; service principles".

[10]	3GPP TS 32.500: "Telecommunication management; Self-Organizing Networks (SON); Concepts and requirements".

[11]	3GPP TS 37.816: "Study on RAN-centric data collection and utilization for LTE and NR".

[12]	3GPP TS 37.320: "Radio measurement collection for Minimization of Drive Tests (MDT); Overall description".

[13]	3GPP TS 23.501: "System Architecture for the 5G System (5GS); Stage 2".

[14]	3GPP TS 28.310: "Energy efficiency of 5G".

[15]	3GPP TR 21.866: "Study on Energy Efficiency Aspects of 3GPP Standards".

[16]	3GPP TS 26.247: "Transparent end-to-end Packet-switched Streaming Service (PSS); Progressive Download and Dynamic Adaptive Streaming over HTTP (3GP-DASH)".

[17]	3GPP TS 26.114: "IP Multimedia Subsystem (IMS); Multimedia Telephony; Media handling and interaction".

[18]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[19]	3GPP TS 28.313: "Self-Organizing Networks (SON) for 5G networks".

[20]	3GPP TS 28.541: "Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3".

[21]	3GPP TS 38.304 NR: "User Equipment (UE) procedures in idle mode and in RRC Inactive state".

[22]	3GPP TS 28.545: " Management and orchestration; Fault Supervision (FS) ".

# 3	Rationale

Add gap analysis in section 6.5.2.4 related to inter-gNB beam selection optimization.

# 4	Detailed proposal

First Change

### 6.5.2	Inter-gNB Beam Selection Optimization

#### 6.5.2.1	Use case

The handover procedure specified in 5G is triggered based on UE measurements, specifically considering the cell level radio quality of the source and target cell(s). In case of beamformed access, one cell can make use of several beams for serving residing users (SSB or CSI-RS) with each user served by a single beam at a time. The cell level quality can be represented as an aggregated metric over one or more beams. So, although handover is performed between two 5G cells, the granularity of handover can be further broken down to beam level.

The target cell provides RACH resources to the UE, which are linked with specific beams. Currently, the target cell can do this, based on beam level measurements performed and reported by the UE. Picking the wrong beam for performing RACH on the target cell could easily result in RLF for the UE and should be avoided.

To address this beam level handover optimization issue, it is desirable to use MDAS to prioritize and/or select a beam in case of handover for a specific target cell, in order to minimize or even avoid RLF.

The MDAS producer provides a beam level HO optimization analytics report considering information about the handover performance of different beam combinations between specific source and target cell pairs. In particular, the MDAS producer considers the beam of the current cell and the selected beam (out of several available beams) of the target cell to keep statistics regarding the handover performance in order to avoid RLF. Beams of the target cell with a successful handover are preferred in the selection.

In other words, the analytics report also provides recommended actions to optimize the beam selection at the target gNB as a function of serving beam on the source cell side. Based on the recommended actions, the MDAS consumer adjusts the priorities for the beam selection at HO, i.e. the beam combinations that are likely to succeed are prioritized, less optimal beam combinations are down prioritized. The analytics report may also be used to aid the target cell to allocate RACH resources in a way that ensures HO success.

#### 6.5.2.2	Potential requirements

REQ-HO_BEAM_OPT_CON-1 		The MDAS producer should have a capability to provide the analytics report describing the handover performance of beam combinations between cell pairs to authorized consumers based on previously recorded HOs.

REQ-HO_BEAM_OPT_CON-2 	The analytics report describing the performance of beam combinations between cell pairs should contain the following information:

-	Number of successful HOs between a given beam pair

-	Number of failed HOs between a given beam pair

-	Indication if a beam pair is to be prioritized or down prioritized

#### 6.5.2.3	Possible solutions

##### 6.5.2.3.1		Solution description

The MDAS producer can analyze the ongoing and/or potential beam handover optimization based on the current and historical beam selection success/failure rate for an inter-gNB handover. The MDAS producer can provide the analytics report as defined in Clause 6.5.2.3.3 triggered by an event or periodically.

##### 6.5.2.3.2		Data required

The following data is required to do the required analysis.

| Data category | Required data |
| --- | --- |
| Performance Measurements | Inter-gNB handover: Number of success/failed handovers in terms of handover preparation/resource preparation; Beam level measurements: CSI-RS, SSB beam related measurements clause 5.1.1.28, TS 28.552 [8]; Success/failure handover rate per beam pair, i.e. serving beam Id in source gNB and selected beam Id in target gNB. |
| MDT Data | UE measurements related to radio conditions and UE location information. |
| Beam selection policy | The current policy configured in the RAN related to beam selection for inter-gNB handover optimization. |

##### 6.5.2.3.3		Analytics report

The gNB resource analytics report contains the following information.

| Analytics Report of gNB resource consumption | Attribute Name | Description |
| --- | --- | --- |
|  | Beam level Inter-gNB Handover performance | Handover success rate per beam ID pair. This can be quantified as high, medium or low success rate. |
|  | Time period | Time period, in the future, for the handover success rate per beam ID pair |
|  | List of gNBs | Objects involved: gNB(s) and cells of gNBs, |

#### 6.5.2.4 	Evaluation

The solution described in clause 6.5.2.3.1 requires the analytics inputs as described in clause 6.5.2.3.2, wherein

- Input PMs on Inter-gNB handover and beam level measurements are specified in TS 28.552 [8].

- Input MDT data is specified in TS 37.320 [12].

- Input PM that need to be specified:

  - success/failure handover rate per beam pair, i.e. serving beam Id in source gNB and selected beam Id in target gNB

  - current configuration related to beam selection for inter-gNB handover optimization

With these analytics inputs which are already defined or accessible, the analytics output as described in 6.2.2.3.4 can be derived.

Therefore, this solution is a feasible candidate for Inter-gNB Beam Selection Optimization provided that Input PM related to success/failure handover rate per beam pair and current configuration related to beam selection are specified.

End of Change