3GPP TSG SA WG5 Meeting 134-e		S5-206263

- electronic meeting, online, 16th - 25th November 2020

- Source:	Nokia

- Title:	Gap Analysis for KPI Anomaly Analysis

- Document for:	Approval

Agenda Item:	6.5.1

# 1	Decision/action requested

Add conclusions section 6.3.6.4

# 2	References

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2] 	3GPP TS 28.550: "Management and orchestration; Performance assurance".

[3]	3GPP TS 28.533: "Management and orchestration; Architecture framework".

[4]	3GPP TS 28.530: "Management and orchestration; Concepts, use cases and requirements".

[5]	3GPP TR 28.861: "Study on the Self-Organizing Networks (SON) for 5G networks".

[6] 	3GPP TR 28.805: "Study on management aspects of communication services".

[7]	3GPP TS 28.554: "5G end to end Key Performance Indicators (KPI)".

[8]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[9]	3GPP TS 22.101: "service aspects; service principles".

[10]	3GPP TS 32.500: "Telecommunication management; Self-Organizing Networks (SON); Concepts and requirements".

[11]	3GPP TS 37.816: "Study on RAN-centric data collection and utilization for LTE and NR".

[12]	3GPP TS 37.320: "Radio measurement collection for Minimization of Drive Tests (MDT); Overall description".

[13]	3GPP TS 23.501: "System Architecture for the 5G System (5GS); Stage 2".

[14]	3GPP TS 28.310: "Energy efficiency of 5G".

[15]	3GPP TR 21.866: "Study on Energy Efficiency Aspects of 3GPP Standards".

[16]	3GPP TS 26.247: "Transparent end-to-end Packet-switched Streaming Service (PSS); Progressive Download and Dynamic Adaptive Streaming over HTTP (3GP-DASH)".

[17]	3GPP TS 26.114: "IP Multimedia Subsystem (IMS); Multimedia Telephony; Media handling and interaction".

[18]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[19]	3GPP TS 28.313: "Self-Organizing Networks (SON) for 5G networks".

[20]	3GPP TS 28.541: "Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3".

[21]	3GPP TS 38.304 NR: "User Equipment (UE) procedures in idle mode and in RRC Inactive state".

[22]	3GPP TS 28.545: " Management and orchestration; Fault Supervision (FS) ".

# 3	Rationale

Add gap analysis in section 6.3.6.4 related to network situation analysis

# 4	Detailed proposal

First Change

### 6.3.6	KPI anomaly analysis

#### 6.3.6.1	Use case

KPI(s) are of great importance for network operators to monitor the key performance of the network. For 5G and beyond, a large amount of KPIs are defined in TS 28.554 [7]. The correlations between different KPIs are complicated and it is hard to monitor the KPI(s) manually. Also, how to assign each KPI threshold is a big challenge for network operators, since each KPI threshold should not be a fixed value considering many factors such as network capacity, service types, end user’s experiences, etc. In addition, the criteria to determine whether a KPI is anomalous also depends on a variety of requirements.

MDAS is expected to have the capability to analyze KPI(s) for both cross domain and single domain. 3GPP Cross Domain MDAS producers may coordinate with domain MDAS producer to identify the anomalous KPI(s) and the corresponding root cause(s).  The KPI anomaly analysis should consider both a single KPI and KPI correlations of different domains. For cross domain KPI anomaly analysis, KPIs or KPI analytical report from each domain should be collected. The MDAS producer should also be able to identify anomalous network situations. A network situation can be characterized as a combination of KPIs with respect to a predefined context that includes time, location, amount of traffic, user characteristics, etc.

MDAS is also expected to have the capability to detect and predict the anomalous KPI(s) and anomalous network situations in different levels e.g. per S-NSSAI, per NSI or per NSSI, etc. The detection and prediction of single KPI anomaly and in relation with multiple correlated KPIs anomaly may be involved. The detection and prediction may also involve a single or multiple KPIs in relation with a network situation. By utilizing  Machine learning technology, historical KPI data and performance data may also be used as the input to perform the ML model training. Besides, along with the KPI or network situation anomaly identification and root cause analysis, MDAS may also recommend more appropriate network configurations to optimize and resolve the KPI anomaly issue and improve the slice QoE.

The KPI anomaly analysis is also expected to have the capability to analyse the correlations between SLS and KPIs or network situations according to service model and identify the most relevant anomalous KPIs and network situations, which cause the SLS degradation, this corresponding analytics report can be considered as an input to support further SLS assurance.

#### 6.3.6.2	Potential requirements

REQ-KPI_ANOMALY_CON-1		The MDAS producer should have a capability to provide the analytics report on KPIs anomaly analysis.

REQ- KPI_ANOMALY_CON-2		The analytics report describing the KPIs anomaly should contain the following information describing the KPI anomaly aspects and potentially future prediction:

-	The predictive anomaly KPI(s) or observed anomaly KPI(s), may split into subcounters in different levels, e.g. per S-NSSAI, per NSI, per NSSI, per 5QI, per UE etc;

-	KPI anomaly indication and root cause analysis;

-	Cross domain and domain KPI anomaly analysis;

-	The recommendations for the configurations of network resource and KPI threshold.

REQ- KPI_ANOMALY_CON-3		The MDAS producer should have a capability to provide the analytics report that identifies a possible anomalous KPIs with respect to network situations for a particular network location and network.

#### 6.3.6.3	Possible solutions

##### 6.3.6.3.1	Solution description

The MDAS producer correlates and analyses the management data described in the following subclause to provide KPI anomaly analysis and identify the root cause. The required data can be from RAN domain or CN domain or both. As the table in 6.3.6.3.4 shows, the analytics report is able to be provided by the MDAS producer to describe the KPI anomaly issue, root causes and recommendations. This procedure may be triggered upon request or periodically.

##### 6.3.6.3.2	Data required for KPI anomaly analysis for RAN domain

Following table shows the potential data required to analyse the RAN domain KPI anomaly.

| Data Category | Required Data |
| --- | --- |
| Performance Measurements | Radio resource utilization: The usage of physical radio resource utilization of the network, see clause 5.1.1.2 of TS 28.552[8]; Performance Measurements for gNB: for example, for RRC connection related KPI anomaly analysis, see clause 5.1, TS 28.552[8], e.g., RRC connection number, RRC connection establishment, RRC connection re-establishment, RRC connection resuming; RAN UE throughput: A KPI that shows how NG-RAN impacts the service quality provided to an end-user, see clause 6.3.6 of TS 28.554 [7]. |
| MDT Data | UE measurements related to RSRP, RSRQ, SINR and UE location information. |
| QoE Data | The details information of QoE data required by this case is FFS. |
| Configuration data | The execution data including the changes or the configuration of the MOIs. |
| Context data | The information on the conditions applicable to the data considered for analytics, e.g. time of day, season or event in relation to location. |

Note: The above parameters may not be the complete list.

##### 6.3.6.3.3	Data required for KPI anomaly analysis for CN domain

Following table shows the potential data required to analyse the CN domain KPI anomaly.

| Data Category | Required Data |
| --- | --- |
| Performance Measurements | Performance Measurements for AMF: for example, for number of registered subscribers related KPI anomaly analysis, see clause 5.2.1, TS 28.552[8]; Performance Measurements for SMF: for example, for PDU session management related KPI anomaly analysis, see clause 5.3.1, TS 28.552[8]; Throughput at N3 interface: KPI related to Upstream/Downstream GTP data throughput at N3 interface, see clause 6.3.4 and clause 6.3.5 of TS 28.554 [7]; |
| QoE Data | The details information of QoE data required by this case is FFS. |

Note: The above parameters may not be the complete list.

##### 6.3.6.3.4	Data required for KPI anomaly analysis for cross domain

For cross domain analysis, the RAN domain and CN domain required data as described in 6.3.6.3.2 and 6.3.6.3.3 may be needed, as well as the potential data described in the following table.

| Data Category | Required Data |
| --- | --- |
| Performance Measurements | Throughput for network slice instance: Upstream/Downstream throughput for network and Network Slice Instance, see clause 6.3.2 and clause 6.3.3 of TS 28.554 [7]; NWDAF analytical data: Slice QoE, see clause 6.4 of TS 23.288 [18]. |
| QoE Data | The details information of QoE data required by this case is FFS. |
| Configuration data | The execution data including the changes or the configuration of the MOIs. |
| Context data | The information on the conditions applicable to the data considered for analytics, e.g. time of day, season or event in relation to location. |

Note: The above parameters may not be the complete list.

##### 6.3.6.3.5	Analytics report for KPI anomaly analysis

Following table shows the potential information of the domain specific or cross domain analytics report for KPI anomaly analysis based on the required data received as described in 6.3.6.3.2, 6.3.6.3.3 and 6.3.6.3.4.

| Analytics Report of network slice KPI anomaly | Attribute Name | Description |
| --- | --- | --- |
|  | KPI anomaly identifier | The identifier of the KPI anomaly; |
|  | Anomalous KPI Name | The name of the KPI(s) which is identified or predicted as anomalous, the KPI name refers to bullet a) in TS28.554 (7); |
|  | Managed Objects of anomalous KPI | The object instances where the KPI is applicable, e.g., SubNetwork Instance, NetworkSlice Instance |
|  | Info of KPI anomaly | Statistics or predictions of the anomalous KPIs, may concern single KPI or multiple correlated KPIs, and may split into subcounters at different levels, e.g. per S-NSSAI, per NSI, per NSSI, per 5QI, per UE etc. |
|  | Root cause | The root cause of the network slice KPI anomaly issues, e.g., unstable handover successful rate, low PRB utilization, low QoS retainability. |

##### 6.3.6.3.6	Analytics report for network situation analysis

Following table shows the potential information of the domain specific or cross domain analytics report based on the required data received as described in 6.3.6.3.2, 6.3.6.3.3, and 6.3.6.3.4.

| Analytics Report on network situations | Attribute Name | Description |
| --- | --- | --- |
|  | Network situation identifier | The identifier of the possible network situations . |
|  | Network situation description | The network performance, QoE and UE location and context data that describe a network situation . |
|  | Network situations anomaly status | The status of a network situation. This may be a simple binary function that indicates a normal or anomalous status. |
|  | Severity level | Describes the degree of anomaly (low, medium, high). |
|  | Location | Geographical location of anomalous network situations |
|  | Managed Objects of network situation | The object instances, e.g., cell, carrier. |
|  | Type of analytics | Statistics or predictions of the anomalous situation. |

#### 6.3.6.4 	Evaluation

##### 6.3.6.4.2	Network situation analysis

The solution described in clause 6.3.6.3.1 requires the analytics inputs as described in clause 6.3.6.3.2, 6.3.6.3.3 and 6.3.6 wherein:

- Input PM related to RAN utilization, gNB measurements, AMF and SMF are specified in TS 28.552 [8].

- Input KPIs related to RAN UE throughput and throughput N3 are specified in TS 28.554 [7].

- Input MDT data are specified in TS 37.320 [12].

- Input related to service experience specified in TS 37.320 [12].

- QoE data is optional.

- Configuration data is available.

- Context data that relates to information on the conditions applicable to the data considered for analytics need to be specified.

With these analytics inputs which are already defined or accessible, the analytics output as described in 6.3.6.3.6 can be derived.

Therefore, the solution is feasible.

End of Change