| 3GPP TR 28.816 V0.2.0 (2020-11) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Charging management; Study on charging aspects of 5GS CIoT (Release 17) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2020, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

1	Scope	6

2	References	6

3	Definitions of terms, symbols and abbreviations	7

3.1	Terms	7

3.2	Symbols	7

3.3	Abbreviations	7

4	Concepts and background	7

4.1	General	7

4.2	 5G data connectivity domain charging	7

4.3	CP data transfer charging	8

4.3.1	General	8

4.3.2	 CP data transfer charging with SCEF	8

4.3.3	 CP data transfer charging with NEF	9

4.4	Monitoring event charging	9

4.4.1	 General	9

4.4.2	 Monitoring event charging with SCEF	9

4.4.3	 Monitoring event charging with NEF	10

5	Use cases and key issues	11

5.1	Use cases	11

5.1.1	Use case #1: Control plane optimization in 5G data connectivity domain charging	11

5.1.1.1 	Description	11

5.1.x	Use case #x	12

5.2 	Potential charging requirements	12

5.x	Key issues	12

5.x.2	Key issues	12

5.x.2.1	Description	12

5.x.2.2	Potential requirements	12

6	Solutions	12

Annex A: Title	13

Annex X: Change history	13

Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In drafting the TS/TR, pay particular attention to the use of modal auxiliary verbs! TRs shall not contain any normative provisions.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

1	Scope

The present document investigates charging enhancements on 5G data connectivity domain charging, control plane data transfer domain charging and Monitoring Events charging for 5GS CIoT for the corresponding functionalities based on TS 23.501 [3], TS 32.251 [4], TS 32.255 [6], TS 32.253 [7] and TS 32.278 [8]:

- Control Plane CIoT 5GS Optimisation

- Non-IP Data Delivery (NIDD)

- Support for Monitoring Events

- Support of rate control of user data, including small data rate control

- Category M UEs differentiation

- Exception report

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications"

[2]	3GPP TS 32.251: "Telecommunication management; Charging management; Packet Switched (PS) domain charging"

[3]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2"

[4]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2"

[5]					3GPP TS 23.503: "Policy and charging control framework for the 5G System (5GS); Stage 2"

[6]	3GPP TS 32.255: "Telecommunication management; Charging management; 5G data connectivity domain charging; Stage 2"

[7]	3GPP TS 32.253: "Telecommunication management; Charging management; Control Plane (CP) data transfer domain charging"

[8]	3GPP TS 32.278: "Telecommunication management; Charging management; Monitoring event charging"

[9]	3GPP TS 32.240: "Telecommunication management; Charging management; Charging architecture and principles"

[10]	3GPP TS 32.254: "Telecommunication management; Charging management; Exposure function northbound Application Program Interfaces (APIs) charging"

3	Definitions of terms, symbols and abbreviations

3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Definition format (Normal)

<defined term>: <definition>.

example: text used to clarify abstract rules by applying them literally.

3.2	Symbols

For the purposes of the present document, the following symbols apply:

Symbol format (EW)

<symbol>	<Explanation>

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

Abbreviation format (EW)

<ABBREVIATION>	<Expansion>

4	Concepts and background

4.1	General

The Packet Switched (PS) domain charging supports charging of CIoT functionalities (e.g., the usage of the radio interface for CIoT, CP CIoT Optimisation and Serving PLMN Rate Control) as described in TS 32.251 [2].

CIoT functionality is provided by the visited and home networks when the networks are configured to support CIoT. It applies to both the non-roaming case and the roaming case and some functionality may be dependent upon the existence of appropriate roaming agreements between the operators.

In 5GS CIoT has been evolved and refers to 5GS optimisations and functionality for support of Cellular Internet-of-Things (Cellular IoT, or CIoT) described in TS 23.501 [3] according to service requirements described in TS 22.261 [xx]. The specific functionality is described in the affected procedures and features of 5G service aspect, in TS 23.501 [3], TS 23.502 [4] and TS 23.503 [5].

4.2		5G data connectivity domain charging

The specification of converged offline and online charging for 5G Data Connectivity domain based on stage 2 description in TS 23.501 [3] is described in TS 32.255 [6].

This charging aspect includes the converged offline and online charging architecture and scenarios specific to the 5G Data Connectivity domain, as well as the mapping of the common 3GPP charging architecture specified in TS 32.240 [9] onto the 5G Data Connectivity domain.

The charging functions specified for the 5G data connectivity charging support PDU session in SMF, service data flows, within PDU session and QoS flows within PDU session. The SMF supports Nchf interface toward CHF, to collect and send charging information as described in TS 32.255 [6].

4.3	CP data transfer charging

4.3.1	General

Functions for NIDD may be used to handle Mobile Originated (MO) and Mobile Terminated (MT) communication for unstructured data (also referred to as Non-IP), as specified in TS 23.501 [3]. The UE may obtain an Unstructured PDU session to the NEF during the PDU Session Establishment procedure. Whether or not use of NIDD API should be invoked is determined during a PDU session by the presence of a "NEF Identity for NIDD". The NEF exposes the NIDD APIs described in TS 23.502 [5] on the N33/Nnef reference point.

4.3.2		CP data transfer charging with SCEF

CP data transfer domain charging is specified in TS 32.253 [7]. Service Capability Exposure which enables the 3GPP network to securely expose its services and capabilities provided by 3GPP network interfaces to external 3rd party service provider applications. The highlighting excerpt that shows 3GPP Nodes involved for Non-IP Data Delivery (NIDD) between the UE and the SCS/AS is described in Figure 4.3.2.1.

Figure 4.3.2.1: 3GPP Architecture Service Capability Exposure for Non-IP Data Delivery

As described in TS 32.253 [7], the following Control Plane data transfer Charging Nodes (CPCN), functioning as the CTF, may generate accounting metrics sets for Control Plane data transfer CDRs:

-	the SCEF, to record Control Plane data transfer;

-	the IWK-SCEF, to record Control Plane data transfer;

-	the MME, to record Control Plane data transfer (optional).

As described in TS 32.254 [10], for offline charging, SCEF functioning as the CTF, may generate accounting metrics sets for Northbound Application Program Interfaces (APIs) CDRs:

-	the SCEF, to record Application Program Interfaces transaction.

As described in TS 32.254 [10], for online charging, northbound API online charging is based on SCEF utilizing Ro interface and application towards the OCS.

4.3.3		CP data transfer charging with NEF

NEF is specified in TS 23.501 [3] to provide Exposure of capabilities and events, NF capabilities and events may be securely exposed by NEF for e.g. 3rd party, Application Functions, Edge Computing. The Non-roaming architecture for Network Exposure Function in reference point representation is depicted in figure 4.3.3.1.

Figure 4.3.3.1: Non-roaming architecture for Network Exposure Function in reference point representation

Converged charging is be performed by the NEF interacting with CHF using Nchf service.

The charging requirements for northbound API for Exposure Function based on NEF Northbound API as described in TS 32.254 [10], is used to specify the offline and online charging description for Northbound Application Program Interfaces (API) and NEF charging CHF CDR.

4.4	Monitoring event charging

4.4.1		General

The Monitoring Events feature is intended for monitoring of specific events in the 3GPP system and reporting such Monitoring Events via the NEF, as described in TS 23.501 [3]. For CIoT, the list of supported monitoring events is specified in table 4.15.3.1-1 of TS 23.502 [4].

4.4.2		Monitoring event charging with SCEF

The charging aspect for Monitoring Events, based on the functional stage 2 description of SCEF is specified in TS 32.278 [8]. The figure 4.4.2.1 shows overall architecture for Service Capability Exposure which enables the 3GPP network to securely expose its services and capabilities provided by 3GPP network interfaces to external 3rd party service provider applications, which includes Monitoring Event functionality described in TS 32.278 [8].

Figure 4.4.2.1: 3GPP Architecture for Service Capability Exposure with SCEF

For offline charging the following Monitoring Event Charging Nodes (MECN), functioning as the CTF, may generate accounting metrics sets for Monitoring Event CDRs:

-	the MME, to record Monitoring Events;

-	the SGSN, to record Monitoring Events;

-	the IWK-SCEF, to record Monitoring Events.

As described in TS 32.254 [10], for online charging SCEF functioning as the CTF may generate accounting metrics sets for Northbound Application Program Interfaces (APIs) CDRs:

-	the SCEF, to record Application Program Interfaces transaction.

As described in TS 32.254 [10], for online charging, northbound API online charging is based on SCEF utilizing Ro interface and application towards the OCS.

4.4.3		Monitoring event charging with NEF

NEF is specified in TS 23.501 [3] to provide Exposure of capabilities and events, NF capabilities and events may be securely exposed by NEF for e.g. 3rd party, Application Functions, Edge Computing. The Non-roaming architecture for Network Exposure Function in reference point representation is depicted in figure 4.4.3.1.

Figure 4.4.3.1: Non-roaming architecture for Network Exposure Function in reference point representation

Converged charging is be performed by the NEF interacting with CHF using Nchf service.

The charging requirements for northbound API for Exposure Function based on NEF Northbound API as described in TS 32.254 [10], is used to specify the offline and online charging description for Northbound Application Program Interfaces (API) and NEF charging CHF CDR.

5	Use cases and key issues

5.1	Use cases

5.1.1	Use case #1: Control plane optimization in 5G data connectivity domain charging

5.1.1.1 	Description

The Control Plane CIoT 5GS Optimisation is used to exchange user data between the UE and the SMF as payload of a NAS message in both uplink and downlink directions, avoiding the establishment of a user plane connection for the PDU Session as described in TS 23.501 [3].

When the Control Plane CIoT 5GS Optimisation feature is used and the PDU session type is unstructured, the SMF selects either NEF or UPF based on information in the UE's subscription as described in TS 23.501 [3].

During the PDU Session Establishment procedure the AMF indicates to the SMF that Control Plane CIoT 5GS Optimisation is available for data transmission. If a PDU session shall only use Control Plane CIoT 5GS Optimisation, the AMF provides a Control Plane Only Indicator to the SMF during the PDU session establishment. A UE and SMF receiving the Control Plane Only Indicator for a PDU session shall always use the Control Plane CIoT 5GS Optimisation for this PDU session.

In 5G data connectivity domain converged charging architecture, the SMF embedding the CTF, generates charging events towards the CHF for PDU connectivity converged charging or offline only charging as described in TS 32.255 [6]. The Control Plane CIoT 5GS Optimization indicator can be included in charging data generated by CTF, the CHF CDR may include Control Plane CIoT 5GS Optimization indicator when this PDU session is negotiated and used for Control Plane CIoT 5GS Optimisation.

5.1.x	Use case #x

This clause is used to provide general use case for charging enhancement for 5GS CIoT. It is recommended to provide additional use case for the particular charging domain(s) (e.g., 5GS data connectivity charging domain, control plane data transfer charging etc.) since those charging aspects are already described in charging specs.

5.2 	Potential charging requirements

REQ-3GPPCH-5GSCIoT-01	The charging aspect of 5G data connectivity domain charging shall support charging of Control Plane CIoT 5GS Optimisation.

5.x	Key issues

5.x.2	Key issues

This clause is used to provide key issue for charging enhancement(s) for 5GS CIoT. It is recommended to provide key issues regarding the particular charging domain(s) (e.g., 5GS data connectivity charging domain, control plane data transfer charging etc.) since those charging aspects are already described in charging specs.

5.x.2.1	Description

5.x.2.2	Potential requirements

6	Solutions

Annex A:
Title

Annex X:
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2020-09 | SA5#133e | S5-205030 |  |  |  | Initial skeleton. | 0.0.0 |
| 2020-10 | SA5#133e | S5-205465 S5-205466 S5-205229 |  |  |  | pCR 28.816 Add scope pCR 28.816 Add introduction pCR 28.816 Add proposal for skeleton for TR 28.816 | 0.1.0 |
| 2020-11 | SA5#134e | S5-206431 S5-206432 S5-206433 S5-206118 |  |  |  | pCR 28.816 add background for data connectivity charging pCR 28.816 add background for control plane data transfer charging pCR 28.816 add background for monitoring event charging pCR 28.816 add CP optimization for data connectivity charging | 0.2.0 |