3GPP TSG-SA5 Meeting #134e 	S5-206183

# e-meeting 16th - 25th November 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.536 | CR | 0014 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update SLS Assurance Procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | COSLA |  |  |  |  |  | Date: |  |  | 2020-11-2 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | AssuranceControlLoopGoal may be derived from the ServiceProfile or SliceProfile. For the former case in the SLS assurance procedure, Entities_Participating_in_loop may respond the AssuranceControlLoop_consumer with assurance fulfilment (including QoE fulfilment) information. The AssuranceControlLoop_consumer may use this information to modify ServiceProfile and update the AssuranceControlLoopGoal if its QoE fulfilment is not satisfied. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Split the SLS Assurance Procedure into two separate procedures for ServiceProfile and SliceProfile. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The differences in SLS assurance procedures for cross domain and domain cannot be handled. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.1.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st of changes

4.1.3.1	SLS Assurance Procedure for AssuranceControlLoopGoals from ServiceProfile

Figure 4.1.3.1.1 SLS assurance procedure for Cross Domain

1. CrossDomain_AssuranceControlLoop_consumer derives AssuranceControlLoopGoal from the ServiceProfile.

2. CrossDomain_AssuranceControlLoop_consumer provides the AssuranceControlLoopGoal to CrossDomain_Entities_Participating_in_loop by utilizing the provision management services defined in as defined in clause 11.1.1.3 of TS 28.532 [7].

NOTE 1:	In this case, the AssuranceControlLoopGoal is the attribute(s) of the ServiceProfile.

3. CrossDomain_Entities_Participating_in_loop subscribes the related performance data (e.g., the packet delay related measurements), fault data, QoE data (e.g., buffer level) and MDT data from respective sources by utilizing the Operation establishStreamingConnection as defined in clause 6.2.1 of TS 28.550 [3].

4. CrossDomain_Entities_Participating_in_loop, optionally, subscribes the related analytical data from MDAS or network functions, e.g., NWDAF. In case of NWDAF as a provider, Nnwdaf_EventsSubscription Service as defined in clause 4.2 is used.

5. CrossDomain_Entities_Participating_in_loop collects the related performance, fault, QoE and MDT data (e.g., the packet delay related measurements), fault data, QoE data (e.g., buffer level) and MDT data from respective sources by utilizing the Operation establishStreamingConnection as defined in clause 6.2.1 of TS 28.550 [3].

6. CrossDomain_Entities_Participating_in_loop, optionally, collects the related analytical data from MDAS or network functions, e.g., NWDAF. In case of NWDAF as a provider, Nnwdaf_EventsSubscription Service as defined in clause 4.2 of TS 29.520 [11] is used.

7. CrossDomain_Entities_Participating_in_loop assesses if the AssuranceControlLoopGoal has been fulfilled.

8. CrossDomain_Entities_Participating_in_loop assesses if and which action to take in case the AssuranceControlLoopGoal has not been fulfilled.

9. As per the mitigation action (e.g., scale out) resources are changed, the generic provisioning management service as defined in clause 11.1 of TS 28.532 [7] is utilized for the same.

10. Action completed.

NOTE 2:	The CrossDomain_Entities_Participating_in_loop continues to monitor and analyse the performance and perform the adjustment until the attribute(s) of ServiceProfile is assured.

11. CrossDomain_AssuranceControlLoop_consumer receives the confirmation of assurance fulfilment (including QoE fulfilment) from CrossDomain_Entities_Participating_in_loop by utilizing the provision management services defined in clause 11.1.1.3 of TS 28.532 [7].

12. CrossDomain_AssuranceControlLoop_consumer may update ServiceProfile based on QoE fulfilment information.

4.1.3.2	SLS Assurance Procedure for AssuranceControlLoopGoals from SliceProfile

Figure 4.1.3.1.2 SLS assurance procedure for Domain

1. Domain_AssuranceControlLoop_consumer derives AssuranceControlLoopGoal from the SliceProfile.

2. Domain_AssuranceControlLoop_consumer provides the AssuranceControlLoopGoal to Domain_Entities_Participating_in_loop by utilizing the provision management services defined in as defined in clause 11.1.1.3 of TS 28.532 [7].

NOTE 1:	In this case, the AssuranceControlLoopGoal is the attribute(s) of the SliceProfile.

3. Domain_Entities_Participating_in_loop subscribes the related performance data (e.g., the packet delay related measurements), fault data, QoE data (e.g., buffer level) and MDT data from respective sources by utilizing the Operation establishStreamingConnection as defined in clause 6.2.1 of TS 28.550 [3].

4. Domain_Entities_Participating_in_loop, optionally, subscribes the related analytical data from MDAS or network functions, e.g., NWDAF. In case of NWDAF as a provider, Nnwdaf_EventsSubscription Service as defined in clause 4.2 is used.

5. Domain_Entities_Participating_in_loop collects the related performance, fault, QoE and MDT data (e.g., the packet delay related measurements), fault data, QoE data (e.g., buffer level) and MDT data from respective sources by utilizing the Operation establishStreamingConnection as defined in clause 6.2.1 of TS 28.550 [3].

6. Domain_Entities_Participating_in_loop, optionally, collects the related analytical data from MDAS or network functions, e.g., NWDAF. In case of NWDAF as a provider, Nnwdaf_EventsSubscription Service as defined in clause 4.2 of TS 29.520 [11] is used.

7. Domain_Entities_Participating_in_loop assesses if the AssuranceControlLoopGoal has been fulfilled.

8. Domain_Entities_Participating_in_loop assesses if and which action to take in case the AssuranceControlLoopGoal has not been fulfilled.

9. As per the mitigation action (e.g., scale out) resources are changed, the generic provisioning management service as defined in clause 11.1 of TS 28.532 [7] is utilized for the same.

10. Action completed.

NOTE 2:	The Domain_Entities_Participating_in_loop continues to monitor and analyse the performance and perform the adjustment until the attribute(s) of SliceProfile is assured.

11. Domain_AssuranceControlLoop_consumer receives the confirmation of assurance fulfilment from Domain_Entities_Participating_in_loop by utilizing the provision management services defined in clause 11.1.1.3 of TS 28.532 [7].

end of changes