3GPP TSG-SA5 Meeting #134-e	S5-206248

# Online, , 16th Nov 2020 - 25th Nov 2020

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.251 | CR | 0517 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of PGW supporting Nchf interface |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile Com. Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_NIESGU |  |  |  |  |  | Date: |  |  | 2020-11-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | For operators deploying a combined node serving as both SMF+PGW-C and PGW-C to provide network services for 2G/3G/4G/5G users, it is beneficial to use N40 interface for charging. Based on the above background, PGW should support Nchf interface and consume Nchf_ ConvergedCharging service exposed by CHF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Specify the PGW enhancement to support Nchf interface for charging upon 2G/3G accesses. Add corresponding references, symbols and abbreviations. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The charging mechanism is not clear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 3.2, 3.3, Annex X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TS 32.240: "Telecommunication management; Charging management; Charging architecture and principles".

[2] - [9]	Void.

[10]	3GPP TS 32.250: "Telecommunication management; Charging management; Circuit Switched (CS) domain charging".

[11] – [19]	Void.

[20]	3GPP TS 32.260: "Telecommunication management; Charging management; IP Multimedia Subsystem (IMS) charging".

[21] - [29]	Void.

[30]	3GPP TS 32.270: "Telecommunication management; Charging management; Multimedia Messaging Service (MMS) charging".

[31]	3GPP TS 32.271: "Telecommunication management; Charging management; Location Services (LCS) charging".

[32]	void.

[33]	3GPP TS 32.273: "Telecommunication management; Charging management; Multimedia Broadcast and Multicast Service (MBMS) charging"

[34] 	3GPP TS 32.274: "Telecommunication management; Charging management; Short Message Service (SMS) charging".

[35] - [49]	Void.

[50]	3GPP TS 32.299: "Telecommunication management; Charging management; Diameter charging application".

[51]	3GPP TS 32.298: "Telecommunication management; Charging management; Charging Data Record (CDR) encoding rules description".

[52]	3GPP TS 32.297: "Telecommunication management; Charging management; Charging Data Records (CDR) file format and transfer".

[53]	3GPP TS 32.296: "Telecommunication management; Charging management; Online Charging System (OCS) applications and interfaces".

[54]	3GPP TS 32.295: "Telecommunication management; Charging management; Charging Data Record (CDR) transfer".

[55]	3GPP TS 32.255: " Telecommunication management; Charging management; 5G system; 5G data connectivity domain charging; stage 2".

[56]	3GPP TS 32.291: " Telecommunication management; Charging management; 5G system; Charging service, stage 3".

[57] - [99]	Void.

[100]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[101]	3GPP TS 22.115: "Service aspects; Charging and billing".

[102] 	3GPP TS 22.011: "Service accessibility".

[103]	3GPP TS 23.003: "Numbering, Addressing and Identification".

[104] - [199]	Void.

[200]	3GPP TS 22.060: "General Packet Radio Service (GPRS); Service description; Stage 1".

[201]	3GPP TS 23.060: "General Packet Radio Service (GPRS); Service description; Stage 2".

[202]	3GPP TS 29.078: "Customized Applications for Mobile network Enhanced Logic (CAMEL) 
Phase X;CAMEL Application Part (CAP) specification".

[203]	3GPP TS 29.060: "General Packet Radio Service (GPRS); GPRS Tunnelling Protocol (GTP) across the Gn and Gp interface".

[204]	Void.

[205]	3GPP TS 29.061: "Interworking between the Public Land Mobile Network (PLMN) supporting packet based services and Packet Data Networks (PDN)".

[206]	3GPP TS 23.078: "Customized Applications for Mobile network Enhanced Logic (CAMEL); 
Phase 4; Stage 2".

[207]	3GPP TS 23.246: "Multimedia Broadcast/Multicast Service (MBMS); Architecture and functional description".

[208]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) acces".

[209]	3GPP TS 23.402: "Architecture enhancements for non-3GPP accesses".

[210]	3GPP TS 29.274: "3GPP Evolved Packet System (EPS); Evolved General Packet Radio Service (GPRS) Tunnelling Protocol for Control plane (GTPv2-C); Stage 3".

[211]	3GPP TS 29.275: "Proxy Mobile IPv6 (PMIPv6) based Mobility and Tunnelling protocols; Stage 3".

[212]	3GPP TS 23.261: "IP flow mobility and seamless Wireless Local Area Network (WLAN) offload; Stage 2".

[213]	3GPP TS 23.272: "Circuit Switched (CS) fallback in Evolved Packet System (EPS); Stage 2".

[214]	Void.

[215] 	3GPP TS 23.203: "Policy and charging control architecture".

[216]	3GPP TS 29.212: "Policy and Charging Control (PCC); Reference points".

[217]	3GPP TS 29.281: "General Packet Radio System (GPRS) Tunnelling Protocol User Plane (GTPv1-U)".

[218] - [241]	Void.

[242]	3GPP TS 23.161: "Network-based IP flow mobility and Wireless Local Area Network (WLAN) offload; Stage 2".

[243] - [399]	Void.

[400]	Void.

[401]	Void.

[402]	IETF RFC 4006 (2005): "Diameter Credit-Control Application".

[403] - [599]	Void.

[600]	Broadband Forum TR-146: "Internet Protocol (IP) Sessions".

[601]	Broadband Forum TR-134: "Broadband Policy Control Framework (BPCF)".

First change

3.2	Symbols

For the purposes of the present document the following symbols apply:

A	Interface between an MSC and a BSC

Bp	Reference point for the CDR file transfer from the Packet Switched CGF to the BD

C	Interface between a HLR and a SMSC

Nchf	Service based interface exhibited by CHF.

D	Interface between a MSC and a HLR

E	Interface between a MSC and a SMSC

Ga	Reference point between a CDF and the CGF for CDR transfer

Gb	Interface between an SGSN and a BSC

Gc	Interface between an GGSN and an HLR

Gd	Interface between an SMS-GMSC and an SGSN, and between a SMS-IWMSC and an SGSN

Ge	Interface between a SGSN and a CAMEL GSM SCF

Gf	Interface between an SGSN and an EIR

Gi	Interface between the Packet-Switched domain and an external packet data network

Gn	Interface between two GSNs within the same PLMN

Gp	Interface between two GSNs in different PLMNs

Gr	Interface between an SGSN and an HLR

Gs	Interface between an SGSN and an MSC/VLR.

Gx	Interface between a PCEF and a PCRF

Gxa	Interface between a HSGW and a PCRF

Gy	Online charging reference point between a PCEF and an OCS

Gyn	Online charging reference point between a TDF and an OCS

Gz	Offline charging reference point between a PCEF and anO FCS

Gzn	Offline charging reference point between a TDF and an OFCS

Iu	Interface between the 3G SGSN and the UTRAN

kbit/s	Kilobits per second. 1 kbit/s = 210 bits per second

Mbit/s	Megabits per second. 1 Mbit/s = 220 bits per second

R	Reference point between a non-ISDN compatible TE and MT. 
Typically this reference point supports a standard serial interface.

Rf	Offline charging reference point between a PCN network element and CDF

Ro	Online charging reference point between a PCN network element and the OCS

Um	Interface between the Mobile Station (MS) and the GSM fixed network part

Uu	Interface between the User Equipment (UE) and the UMTS fixed network part

First change

3.3	Abbreviations

For the purposes of the present document, the abbreviations defined in TR 21.905 [100] and the following apply:

3G	3rd Generation

ABC	Application Based Charging

ADC	Application Detection and Control

AoC	Advice of Charge

APN	Access Point Name

ARP	Allocation and Retention Priority

BD	Billing Domain

CAMEL	Customised Applications for Mobile network Enhanced Logic

CCA	Credit-Control-Answer

CCR	Credit-Control-Request

CDF	Charging Data Function

CDR	Charging Data Record

CG	Charging Gateway

CGF	Charging Gateway Function

CHF	Charging Function

CI	Cell Identity

CIoT	Cellular Internet of Things

CP	Control PlaneCS	Circuit Switched

CSE	CAMEL Service Environment

CSG	Closed Subscriber Group

CSG ID	Closed Subscriber Group Identity

CTF	Charging Trigger Function

DCCA	Diameter Credit-Control Application

EPC	Evolved Packet Core

ePDG	Evolved Packet Data Gateway

EPS	Evolved Packet System

ECUR	Event Charging with Unit Reservation

E-UTRAN	Evolved Universal Terrestrial Radio Access Network

FBC	Flow Based bearer Charging

FQPC	Fully Qualified Partial CDR

GERAN	GSM EDGE Radio Access Network

GGSN	Gateway GPRS Support Node

GPRS	General Packet Radio Service

GSM	Global System for Mobile communication

GTP	GPRS Tunnelling Protocol

GTP'	The GPRS protocol used for CDR transport. 
It is derived from GTP with enhancements to improve transport reliability necessary for CDRs.

HeNB	Home eNode B

HNB	Home Node B

H(e)NB	HNB and HeNB

HLR	Home Location Register

HPLMN	Home PLMN

HRPD	High Rate Packet Data

HSGW	HRPD Serving Gateway

IE	Information Element

IEC	Immediate Event Charging

IETF	Internet Engineering Task Force

IFOM	IP Flow Mobility

IMEI	International Mobile Equipment Identity

IMSI	International Mobile Subscriber Identity

IP	Internet Protocol

IP-CAN	IP Connectivity Access Network

IPv4	Internet Protocol version 4

IPv6	Internet Protocol version 6

ISDN	Integrated Services Digital Network

ITU-T	International Telecommunication Union - Telecommunications standardization sector

LAC	Location Area Code

LR	Location Request

M-CDR	Mobility management generated CDR

MAPCON	Multi Access PDN Connectivity

MCC	Mobile Country Code (part of IMSI)

ME	Mobile Equipment

MLC	Mobile Location Center

MME	Mobility Management Entity

MMS	Multimedia Messaging Service

MNC	Mobile Network Code (part of IMSI)

MO	Mobile Originated

MO-LR	Mobile Originated Location Request

MS	Mobile Station

MSISDN	Mobile Station ISDN number

MT	Mobile Terminated

MT-LR	Mobile Terminated Location Request

NBIFOM	Network-based IP flow mobility

NE	Network Element

NI	Network Identifier (part of the APN)

NI-LR	Network Induced - Location Request

NSWO	Non-Seamless WLAN Offload

OAM&P	Operation, Administration, Maintenance and Provisioning

OCF	Online Charging Function

OCS	Online Charging System

OI	Operator Identifier (part of the APN)

P-GW	PDN Gateway

PCC	Policy and Charging Control

PCEF	Policy and Charging Enforcement Function

PCN	Packet switched Core network Node (SGSN, GGSN, S–GW, P–GW, TDF)

PDN	Packet Data Network

PDP	Packet Data Protocol (e.g. IP)

PDU	Packet Data Unit

PGW-CDR	P-GW (enhanced by FBC) generated CDR

PLMN	Public Land Mobile Network

PMIP	Proxy Mobile IP

PPP	Point-to-Point Protocol

PRA	Presence Reporting Area

PS	Packet Switched

QCI	QoS Class Identifier

QoS	Quality of Service

RAB	Radio Access Bearer

RAC	Routing Area Code

RANAP	Radio Access Network Application Part

RG	Residential Gateway

RNC	Radio Network Controller

RPC	Reduced Partial CDR

SAC	Service Area Code

S-CDR	SGSN (IP-CAN bearer) generated CDR

S-GW	Serving Gateway

SCUR	Session Charging with Unit Reservation

SDF	Service Data Flow

SGSN	Serving GPRS Support Node

SGW-CDR	S-GW (IP-CAN bearer) generated CDR

SIPTO	Selected IP Traffic Offload

SMF	Session Management Function

SMS	Short Message Service

S-SMO-CDR	SGSN delivered Short message Mobile Originated CDR

S-SMT-CDR	SGSN delivered Short message Mobile Terminated CDR

T-PDU	Transport PDU

TDF	Traffic Detection Function

TDF-CDR	TDF generated CDR

TR	Technical Report

TS	Technical Specification

TWAG	Trusted WLAN Access Gateway

TWAN	Trusted WLAN Access Network

UMTS	Universal Mobile Telecommunications System

UP	User Plane

USIM	Universal Subscriber Identity Module

UTRAN	UMTS Terrestrial Radio Access Network

UWAN	Untrusted Wireless Access Network

Next change

Annex X (normative):
2/3G Accesses

This Annex specifies the PGW enhancement to support Nchf interface for charging upon 2G/3G accesses.

To simplify the network architecture and network management when 5GS is deployed and 2G or 3G still exists in their network, operators would like to be able to deploy a combined node serving as both SMF+PGW-C and PGW-C, to simultaneously serve 2G/3G/4G/5G users.

For 2G and 3G accesses scenario, network operators can decide whether PGW uses Ga/Gy charging reference points or Nchf serviced based interface. 	The charging architecture and principle for PGW using Nchf serviced based interface is specified in TS 32.255 [55]. The Nchf_ ConvergedCharging service exposed by CHF and consumed by the PGW is specified in TS 32.291 [56].

End of change