3GPP TSG SA WG5 Meeting 135-e		TDoc S5-211124

# electronic meeting, online, 25 January - 3 February 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.254 | CR | 0013 | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on applicable scenarios and flows |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1_NEFCH |  |  |  |  |  | Date: |  |  | 2021-01-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Missing description related to NEF APIs in the scope and general principles The flows for NEF API Invocation and API Notification - Immediate Event Charging (IEC) are described with Post Event Charging (PEC) flavor Ro SCUR is mentioned from SCEF, however it is not applicable. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the NEF APIs in the scope and principles descripitons Correct the flows on NEF API Invocation/API Notification IEC and add the PEC scenario Remove reference to SCUR for Ro from SCEF |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect charging for NEF APIs |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 1, 2, 3.3, 5.1.0, 5.1.1, 5.3.1.1, 5.4.1.1, 5.4.2.2, 5.4.2.4, 5.4.2.x (New), 5.4.2.y (New), 5.4.3.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

# 1	Scope

The present document is part of a series of documents that specify charging functionality and charging management in 3GPP networks. The 3GPP core network charging architecture and principles are specified in TS 32.240 [1], which provides an umbrella for other charging management TSs that specify:

-	the content of the CDRs per domain / subsystem / service (offline charging);

-	the content of real-time charging messages per domain / subsystem / service (online charging);

-	the functionality of online and offline charging for those domains / subsystems / services;

-	the interfaces that are used in the charging framework to transfer the charging information (i.e. CDRs or charging events).

The complete document structure for these TSs is defined in TS 32.240 [1].

The present document specifies the offline, online and converged charging description for Northbound Application Program Interfaces (API), based on the functional stage 2 description in TS 23.682 [243] for transaction over T8 reference point between SCEF and SCS/AS and in TS 23.501 [200] for Network Exposure Function (NEF). This charging description includes the offline, online and converged charging architecture and scenarios specific to Northbound Application Program Interfaces (API), as well as the mapping of the common 3GPP charging architecture specified in TS 32.240 [1] onto the Northbound Application Program Interfaces (API). It further specifies the structure and content of the CDRs for offline charging. The present document is related to other 3GPP charging TSs as follows:

-	The common 3GPP charging architecture is specified in TS 32.240 [1].

-	The parameters, abstract syntax and encoding rules for the CDRs are specified in TS 32.298 [51].

-	A transaction based mechanism for the transfer of CDRs within the network is specified in TS 32.295 [54].

-	The file based mechanism used to transfer the CDRs from the network to the operator's billing domain (e.g. the billing system or a mediation device) is specified in TS 32.297 [52].

-	The 3GPP Diameter application that is used for Northbound Application Program Interfaces (API) offline and online charging is specified in TS 32.299 [50].

-	The services, operations and procedures of charging, using Service Based Interface are specified in TS 32.290 [57].

-	The charging service of 5G system is specified in TS 32.291 [58].

Next change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TS 32.240: "Telecommunication management; Charging management; Charging architecture and principles".

[2] – [49]	Void.

[50]	3GPP TS 32.299: "Telecommunication management; Charging management; Diameter charging application".

[51]	3GPP TS 32.298: "Telecommunication management; Charging management; Charging Data Record (CDR) parameter description".

[52]	3GPP TS 32.297: "Telecommunication management; Charging management; Charging Data Record (CDR) file format and transfer".

[53]	3GPP TS 32.296: "Telecommunication management; Charging management; Online Charging System (OCS): Applications and interfaces".

[54]	3GPP TS 32.295: "Telecommunication management; Charging management; Charging Data Record (CDR) transfer".

[55] – [56]	Void.

[57]	3GPP TS 32.290: "Telecommunication management; Charging management; 5G system; Services, operations and procedures of charging using Service Based Interface (SBI)".

[58]	3GPP TS 32.291: "Telecommunication management; Charging management; 5G system; Charging service, stage 3".

[59] – [99]	Void.

[100]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[101] – [199]	Void

[200]	3GPP TS 23.501:"System Architecture for the 5G System".

[201]	3GPP TS 23.502:"Procedures for the 5G System".

[202] – [229]	Void

[230]	3GPP TS 29.122: "T8 reference point for northbound Application Programming Interfaces (APIs)".

[231] – [242]	Void

[243]	3GPP TS 23.682: "Architecture enhancements to facilitate communications with packet data networks and applications".

[244] – [299]	Void

[300] – [399]	Void.

Next change

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [100] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [100].

AS	Application Server

BD	Billing Domain

CDF	Charging Data Function

CGF	Charging Gateway Function

CTF	Charging Trigger Function

ECUR	Event Charging with Unit Reservation

IE	Information Element

IEC	Immediate Event Charging

NEF	Network Exposure Function

PEC	Post Event Charging

SCEF	Service Capability Exposure Function

SCS	Services Capability Server

SGSN	Serving GPRS Support Node

RCAF	RAN Congestion Awareness Function

Next change

### 5.1.0	General

The following are high level charging requirements for northbound API for Exposure Function which are specified in TS 23.682 [243] and TS 23.501 [200]:

For northbound API invocation/notification, the SCEF or NEF shall collect the following charging information:

-	invocations/notifications count of the northbound APIs.

-	identification of the SCS/AS or AF and the associated northbound API invocation/notification.

-	timestamp of the northbound API invocation/notification.

-	northbound API related information, e.g. location.

Next change

### 5.1.1	Northbound API procedures

All procedures that operate across the T8 reference point, as specified in 3GPP TS 23.682 [243] and TS 29.122 [230], are covered, which are the following:

-	Monitoring

-	Resource management of Background Data Transfer

-	Changing the chargeable party at session set up or during the session

-	Non-IP Data Delivery

-	Device Triggering

-	Group Message Delivery

-	Reporting of Network Status

-	Communication Pattern Parameters Provisioning

-	PFD Management

-	Enhanced Coverage Restriction Control

-	Network Parameter Configuration

-	Setting up an AS session with required QoS

-	MSISDN-less Mobile Originated SMS

The following clauses 5.2 and 5.3 describe the trigger conditions and simplified message flows for Event Based Charging (IEC/ECUR), with interfaces specified in 3GPP TS 32.299 [50].

The Northbound APIs supported by the NEF via the set of exposed services defined in 3GPP TS 23.502 [201] are covered for converged charging, with the trigger conditions and message flows defined in clause 5.4.

Next change

#### 5.3.1.1	General

Northbound API online charging is performed by the SCEF using the common Ro based Credit-Control application specified in TS 32.299 [50]. In order to provide the data required for the management activities outlined in TS 32.240 [1], the SCEF shall be able to perform online charging for the following:

-	Charging data related to northbound API invocation;

-	Charging data related to northbound API notification.

Event based online charging IEC and ECUR are applicable scenarios for the SCEF.

Next change

#### 5.4.1.1	General

Converged charging may be performed by the NEF interacting with CHF using Nchf specified in TS 32.290 [57] and TS 32.291 [58]. In order to provide the data required for the management activities outlined in TS 32.240 [1] (Credit-Control, accounting, billing, statistics etc.), the NEF shall be able to perform converged charging for the northbound API access.

The NEF shall be able to perform convergent charging by interacting with CHF, for charging data related to Northbound API access. The Charging Data Request and Charging Data Response are exchanged between the NEF and the CHF, based on PEC, IEC or ECUR scenarios specified in TS 32.290 [57]. The Charging Data Request is issued by the NEF towards the CHF when certain conditions (chargeable events) are met.

Converged charging uses centralized or decentralized unit determination and centralized rating scenarios for event based convergent charging specified in TS 32.290 [57].

The contents and purpose of each charging event that triggers interaction with CHF, as well as the chargeable events that trigger them, are described in the following clauses.

A detailed formal description of the converged charging parameters defined in the present document is to be found in TS 32.291 [58].

A detailed formal description of the CDR parameters defined in the present document is to be found in TS 32.298 [51].

The selection of the CHF can be configured in the NEF but may also rely on NRF.

Next change

#### 5.4.2.2	API Invocation - IEC

Figure 5.4.2.2.1 describes the scenario where there is an API invocation request at the NEF for IEC mode

Figure 5.4.2.2.1: API Invocation Request to NEF using IEC

1.	NEF receives an API invocation Request from an AF.

1ch-a. The NEF sends Charging Data Request [Event] to CHF for the received API Invocation.

1ch-b. The CHF creates a CDR for this API Invocation.

1ch-c. The CHF acknowledges and grants authorization by sending Charging Data Response [Event] to the NEF.

2.	NEF performs the actions needed to fulfil the API invoked.

3.	If authorized, the NEF continues the API invocation processing and sends the API Invocation Response.

Next change

#### 5.4.2.4	API Notification - IEC

Figure 5.4.2.4.1 describes the scenario where an API Notification is delivered from the NEF for IEC mode

Figure 5.4.2.4.1 API Notification from NEF using IEC

1.	The NEF receives a notification from an NF.

1ch-a. The NEF sends Charging Data Request [Event] to CHF for the Notification.

1ch-b. The CHF creates a CDR for this Notification.

1ch-c. The CHF acknowledges and grant authorization by sending Charging Data Response [Event] to the NEF.

2.	The NEF sends the notification to AF.

3.	The NEF receives acknowledgement for the notification.

Next change

#### 5.4.2.x	API Invocation - PEC

Figure 5.4.2.x.1 describes the scenario where there is an API invocation request at the NEF for PEC mode

Figure 5.4.2.x.1: API Invocation Request to NEF using PEC

1.	NEF receives an API invocation Request from an AF.

2.	NEF performs the actions needed to fulfil the API invoked.

3.	If authorized, the NEF continues the API invocation processing and sends the API Invocation Response.

3ch-a. The NEF sends Charging Data Request [Event] to CHF for the received API Invocation.

3ch-b. The CHF creates a CDR for this API Invocation.

3ch-c. The CHF acknowledges by sending Charging Data Response [Event] to the NEF.

Next change

#### 5.4.2.y	API Notification - PEC

Figure 5.4.2.y.1 describes the scenario where an API Notification is delivered from the NEF for PEC mode

Figure 5.4.2.y.1 API Notification from NEF using PEC

1.	The NEF receives a notification from an NF.

2.	The NEF sends the notification to AF.

2ch-a. The NEF sends Charging Data Request [Event] to CHF for the Notification.

2ch-b. The CHF creates a CDR for this Notification.

2ch-c. The CHF acknowledges by sending Charging Data Response [Event] to the NEF.

3.	The NEF receives acknowledgement for the notification.

Next change

##### 5.4.3.2.1	General

A Northbound API charging CHF CDR is used to collect charging information related to northbound API invocation and notifications chargeable events for PEC, IEC and ECUR.

End of changes