3GPP TSG-SA5 Meeting #135e 	S5-211157

# e-meeting 25 January - 3 February 2021

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.554 | CR | 0072 | rev | - | Current version: | 17.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Resource Consumption KPI for NF and 5GC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | EE5GPLUS |  |  |  |  |  | Date: |  |  | 2021-01-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The resource consumption of the virtualized network function reflects the its energy consumption. However, the resource consumption related KPIs are not defined. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Define KPI for the resoucrce consumption of a network function and the 5GC. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The resource consumption of a network function and the 5GC would remain undefined and the energy consumption estimation may not be accurate without the information provided by these KPIs. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.7.X (New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | N | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st change

6.7	Energy Efficiency (EE) KPI

6.7.x Resource Consumption KPI

6.7.x.1 NF Resource Consumption KPI

a)

b) 	A KPI that shows the resource consumption to 5GC NF.  This KPI is obtained by summing up the resource allocated to each of the VNF instance(s) that compose the NF. The resource allocated to a VNF instance is estimated by summing up all the resource allocated to each of its constituent VNF Component instance(s) (VNFC).

The types of the resource considered are Virtualization Container (VC) resources including CPU resource and memory resource. Resources allocated to VNFC are measured through performance monitoring, which could be external monitoring tools or monitoring functions provided by NFV infrastructure.

The summation is made with respect to each of the aforesaid types of resource separately.

, where:

- ‘vCPU Resource Usage’ is calculated as the average Process Utilization of the VNFC instance during a given time period multiplied by cpu clock speed and number of cores of the NFVI node where the VNFC is located,

- Process Utilization is defined in ETSI GS NFV-TST 008 clause 6.6 as the ratio of the total time that one or more compute resources (according to the defined scope) execute instructions in the specified execution context during the measurement interval to the time in the measurement interval, expressed as a percentage,

- ‘Memory Usage’ is the amount of memory used by VNFC (cf. the metric ‘Memory Used’ defined in ETSI GS NFV-TST 008 clause 8.6).

In this version of the document, it is assumed that:

- the resource allocated to a NF/VNF/VNFC other than CPU and Memory are not included.

- each type of the resource is one dimension of the NF, VNF and VNFC resource consumption, when one or multiple types of resource are considered, the calculation is made with respect to aforesaid type(s) of resource separately,

- each VNFC instance pertains to one and only one VNF instance,

- each VNF instance pertains to one and only one Network Function,

- each VNFC instance utilizes virtual CPUs pertaining to one and only one NFVI node.

c）

d)	ManagedFunction

6.7.x.2 5GC Resource Consumption KPI

a)

b)	A KPI that shows the resource consumption to 5GC.  This KPI is obtained by summing up the resource consumed by each of the NFs that compose the 5GC. The resource consumption of the NF is defined in 6.7.X.1

The types of the resource considered are Virtualization Container (VC) resources including CPU resource and memory resource.

The summation is made with respect to each of the aforesaid types of resource separately.

c)

d)	SubNetwork

End of changes