3GPP TSG-SA5 Meeting #135-e	S5-211191

# Online, , 25th Jan 2021 - 3rd Feb 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.541 | CR | 0438 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | YANG NRM for Network Slicing |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Cisco Systems Belgium |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNRM |  |  |  |  |  | Date: |  |  | 2021-01-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Missing Network Slice NRM stage 3 YANG. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding missing stage 3 YANG for the Network Slice NRM. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Mismatch between stage 2 and 3. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Stage 3 YANG for Network Slice NRM. Checked locally with pyang --3gpp Not present in Forge at the time of submission, but will be added to Forge soon. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

# E.6	Modules

# E.6.1	module _3gpp-ns-nrm-networkslice.yang

module _3gpp-ns-nrm-networkslice {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-ns-nrm-networkslice;

prefix ns3gpp;

import _3gpp-ns-nrm-networkslicesubnet { prefix nss3gpp; }

import _3gpp-common-subnetwork { prefix subnet3gpp; }

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-common-top { prefix top3gpp; }

include _3gpp-ns-nrm-serviceprofile;

organization "3GPP SA5";

description "A network slice instance in a 5G network.";

//FIXME: Add contact? No other _3gpp modules seem to have a contact stmt.

reference "3GPP TS 28.541

Management and orchestration;

5G Network Resource Model (NRM);

Information model definitions for network slice NRM (chapter 6)

";

revision 2020-01-15 {

description "Introduction of YANG definitions for network slice NRM";

reference "CR-0438";

}

revision 2018-08-09 {

description "Initial revision";

reference "CR-XXXXXX"; //FIXME: Fill in actual CR number

}

grouping NetworkSliceGrp {

uses subnet3gpp:SubNetworkGrp;   // Inherits from SubNetwork

leaf operationalState {

description "The operational state of the network slice instance.

It describes whether or not the resource is physically installed

and working.";

config false;

type types3gpp:OperationalState;

}

leaf administrativeState {

description "The administrative state of the network slice instance.

It describes the permission to use or prohibition against

using the instance, imposed through the OAM services.";

type types3gpp:AdministrativeState;

}

list serviceProfileList {

description "A list of service profiles supported by the network

slice instance.";

key serviceProfileId;

uses ServiceProfileGrp;

}

leaf networkSliceSubnetRef {

type leafref {  //FIXME: Or use type types3gpp:DistinguishedName; ?

path /nss3gpp:NetworkSliceSubnet/nss3gpp:id;

}

description "The NetworkSliceSubnet that the NetworkSliceSubnet is

associated with.";

//FIXME: Do we mean "The NetworkSliceSubnet that this NetworkSlice

//is associated with."?

}

}

list NetworkSlice {

description "Represents the properties of a network slice instance in

a 5G network.";

key id;

container attributes {

uses NetworkSliceGrp;

leaf-list parents {

description "Reference to all containg NetworkSlice instances

in strict order from the root NetworkSlice down to the immediate

parent NetworkSlice.

If NetworkSlices form a containment hierarchy this is

modeled using references between the child NetworkSlice and the

parent NetworkSlices.

This reference MUST NOT be present for the top level NetworkSlice

and MUST be present for other NetworkSlices.";

ordered-by user;

type leafref {

path "../../../NetworkSlice/id";

}

//FIXME: This construct is not going to work well in practice

//FIXME: The list SubNetwork in _3gpp-common-subnetwork will not get

//updated even if this class inherits from it

//FIXME: add a must statement for consistency checks?

//FIXME: separate CR for refactoring this into a common construct

//defined in _3gpp-common-subnetwork?

}

leaf-list containedChildren{

description "Reference to all directly contained NetworkSlice

instances.  If NetworkSlices form a containment hierarchy this is

modeled using references between the child and the parent

NetworkSlice.";

type leafref {

path "../../../NetworkSlice/id";

}

//FIXME: This construct is not going to work well in practice

//FIXME: The list SubNetwork in _3gpp-common-subnetwork will not get

//updated even if this class inherits from it

//FIXME: add a must statement for consistency checks?

//FIXME: separate CR for refactoring this into a common construct

//defined in _3gpp-common-subnetwork?

}

}

uses top3gpp:Top_Grp;

}

}

## E.6.2	module _3gpp-ns-nrm-networkslicesubnet.yang

module _3gpp-ns-nrm-networkslicesubnet {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-ns-nrm-networkslicesubnet;

prefix nss3gpp;

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-common-subnetwork { prefix subnet3gpp; }

import _3gpp-common-measurements { prefix meas3gpp; }

import _3gpp-common-top { prefix top3gpp; }

include _3gpp-ns-nrm-sliceprofile;

organization "3GPP SA5";

description "This IOC represents the properties of a network slice subnet

instance in a 5G network.";

//FIXME: Add contact? No other _3gpp modules seem to have a contact stmt.

reference "3GPP TS 28.541

Management and orchestration;

5G Network Resource Model (NRM);

Information model definitions for network slice NRM (chapter 6)

";

revision 2020-01-15 {

description "Introduction of YANG definitions for network slice NRM";

reference "CR-0438";

}

revision 2019-06-07 {

description "initial revision";

reference "Based on

3GPP TS 28.541 V15.X.XX";

}

feature MeasurementsUnderNetworkSliceSubnet {

description "The MeasurementSubtree shall be contained under

NetworkSliceSubnet.";

}

typedef ETSI-GS-NFV-Identifier {

type string;

reference "ETSI GS NFV-IFA 013";

}

grouping NsInfoGrp {

description "The NsInfo of the NS instance corresponding to the network

slice subnet instance.";

//suport condition: It shall be supported if the NSS instance is

//realized in the virtualized environment.

// Otherwise this attribute shall be absent.

reference "ETSI GS NFV-IFA 013 clause 8.3.3.2.2, which can be found at

https://www.etsi.org/deliver/etsi_gs/NFV-IFA/001_099/013

/03.04.01_60/gs_NFV-IFA013v030401p.pdf page 123-124";

leaf nSInstanceId {

description "Uniquely identifies the NS instance.";

config false;

type ETSI-GS-NFV-Identifier;

}

leaf nsName {

description "Human readable name of the NS instance.";

type string;

config false;

}

leaf description {

description "Human readable description of the NS instance.";

config false;

type string;

}

}

grouping NetworkSliceSubnetGrp {

uses subnet3gpp:SubNetworkGrp;

leaf operationalState {

description "The operational state of the network slice instance.

It describes whether or not the resource is physically installed

and working.";

mandatory true;

config false;

type types3gpp:OperationalState;

}

leaf administrativeState {

description "The administrative state of the network slice instance.

It describes the permission to use or prohibition against

using the instance, imposed through the OAM services.";

mandatory true;

type types3gpp:AdministrativeState;

}

//FIXME: This is multiplicity 1, no need for a list?

uses NsInfoGrp;

//FIXME: Or do we need a container?

//container nsInfo {

//  uses NsInfoGrp;

//}

//FIXME: Or is a list with max-instances 1 required?

//list nsInfo {

//  key nSInstanceId;

//  key "nSInstanceId nsName description";

//FIXME: Maybe even with all leafs as keys?

//  max-instances 1;

//  uses NsInfoGrp;

//}

list sliceProfileList {

description "List of SliceProfiles supported by the network slice

subnet instance";

key sliceProfileId;

uses SliceProfileGrp;

}

list managedFunctionRef {

description "The managed functions that the NetworkSliceSubnet is

associated with.";

key aggregatedManagedFunction;

leaf aggregatedManagedFunction {

type instance-identifier;

}//FIXME: is this mapping right? is the type ok?

}

leaf-list networkSliceSubnetRef {

type leafref {

path ../../id; //FIXME: Use absolute path (/NetworkSliceSubnet/id)

//instead? Easier to read.

}

description "Lists the NetworkSliceSubnet instances associated with

this NetworkSliceSubnet.";

}

}

list NetworkSliceSubnet {

description "Represents the properties of a network slice subnet

instance in a 5G network.";

key id;

container attributes {

uses NetworkSliceSubnetGrp;

leaf-list parents {

description "Reference to all containg NetworkSliceSubnet instances

in strict order from the root NetworkSliceSubnet down to the

immediate parent NetworkSliceSubnet.

If NetworkSliceSubnets form a containment hierarchy this is

modeled using references between the child NetworkSliceSubnet

and the parent NetworkSliceSubnet.

This reference MUST NOT be present for the top level

NetworkSliceSubnet and MUST be present for other

NetworkSliceSubnets.";

ordered-by user;

type leafref {

path "../../../NetworkSliceSubnet/id"; //FIXME: Use absolute

//path (/NetworkSliceSubnet/id) instead? Easier to read.

}

//FIXME: This construct is not going to work well in practice

//FIXME: The list SubNetwork in _3gpp-common-subnetwork will not get

//updated even if this class inherits from it

//FIXME: add a must statement for consistency checks?

//FIXME: separate CR for refactoring this into a common construct

//defined in _3gpp-common-subnetwork?

}

leaf-list containedChildren {

description "Reference to all directly contained NetworkSliceSubnet

instances.  If NetworkSliceSubnets form a containment hierarchy

this is modeled using references between the child

NetworkSliceSubnet and the parent NetworkSliceSubnet.";

type leafref {

path "../../../NetworkSliceSubnet/id"; //FIXME: Use absolute

//path (/NetworkSliceSubnet/id) instead? Easier to read.

}

//FIXME: This construct is not going to work well in practice

//FIXME: The list SubNetwork in _3gpp-common-subnetwork will not get

//updated even if this class inherits from it

//FIXME: add a must statement for consistency checks?

//FIXME: separate CR for refactoring this into a common construct

//defined in _3gpp-common-subnetwork?

}

}

uses top3gpp:Top_Grp;

uses meas3gpp:MeasurementSubtree {

if-feature MeasurementsUnderNetworkSliceSubnet;

}

}

}

## E.6.3	module _3gpp-ns-nrm-perfreq.yang

module _3gpp-ns-nrm-perfreq {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-ns-nrm-perfreq;

prefix perf3gpp;

organization "3GPP SA5";

description "The performance requirements for the NSI in terms of the

scenarios defined in the 3GPP TS 22.261, such as experienced data rate,

area traffic capacity (density) information of UE density.";

//FIXME: Add contact? No other _3gpp modules seem to have a contact stmt.

reference "3GPP TS 28.541

Management and orchestration;

5G Network Resource Model (NRM);

Information model definitions for network slice NRM (chapter 6)

";

revision 2020-01-15 {

description "Introduction of YANG definitions for network slice NRM";

reference "CR-0438";

}

revision 2018-08-10 {

description "Initial revision";

reference "CR-XXXXXX"; //FIXME: Fill in actual CR number

}

//FIXME: Possibly move these types to a common module

typedef data-rate {

type uint32;

units kbits/s;

}

typedef integer-percentage {

type uint8 {

range 0..100;

}

units percent;

}

typedef reliability-string {

description "Mean time between failures.

E.g. '1 day', or '3 months'";

type string {

pattern "[0-9]+ (day|week|month|year)s?";

}

reference "3GPP TS 22.104 clause 5.2-5.5";

}

typedef message-size-string {

description "Message size in bytes.

E.g. '80', or '250-2000'";

type string {

pattern '[0-9]+(-[0-9]+)?';

}

units bytes;

reference "3GPP TS 22.104 clause 5.2-5.5";

}

typedef transfer-interval-string {

description "Transfer interval time.  If multiple values are given,

the first value is the application requirement, the other values are

the requirement with multiple transmission of the same information

two or three times, respectively).

E.g. '40ms', or '0ms-5ms,0ms-2.5ms,0ms-1.7ms'";

type string {

pattern '[0-9]+(\.[0-9]+)?m?s-[0-9]+(\.[0-9]+)?m?s' +

'(,[0-9]+(\.[0-9]+)?m?s-[0-9]+(\.[0-9]+)?){0,2}';

}

reference "3GPP TS 22.104 clause 5.2-5.5";

}

typedef survival-time-string {

description "Survival time in milliseconds (ms) or in multiples of

the transfer interval (x).  If multiple values are given,

the first value is the application requirement, the other values are

the requirement with multiple transmission of the same information

two or three times, respectively).

E.g. '12ms', or '0x,2x'";

type string {

pattern '[0-9]+(x|ms)(,[0-9]+(x|ms)){0,2}';

}

reference "3GPP TS 22.104 clause 5.2-5.5";

}

grouping PerfReqGrp {//FIXME: actively complain about stage 2 design. in cr

// eMBB leafs, SST = 1

leaf expDataRateDL {

description "User experienced data rate over downlink";

//FIXME when 'somepath/sST = 1';

type data-rate;

reference "3GPP TS 22.261 clause 7.1, table 7.1-1";

}

leaf expDataRateUL {

description "User experienced data rate over uplink";

//FIXME when 'somepath/sST = 1';

type data-rate;

reference "3GPP TS 22.261 clause 7.1, table 7.1-1";

}

leaf areaTrafficCapDL {

description "Area traffic capacity over downlink";

//FIXME when 'somepath/sST = 1';

type data-rate;

units kbits/s/km2;

reference "3GPP TS 22.261 clause 7.1, table 7.1-1";

}

leaf areaTrafficCapUL {

description "Area traffic capacity over uplink";

//FIXME when 'somepath/sST = 1';

type data-rate;

units kbits/s/km2;

reference "3GPP TS 22.261 clause 7.1, table 7.1-1";

}

leaf overallUserDensity {

description "Overall user density";

//FIXME when 'somepath/sST = 1';

type uint32;

units users/km2;

reference "3GPP TS 22.261 clause 7.1, table 7.1-1";

}

leaf activityFactor {

description "Percentage value of the amount of simultaneous active

UEs to the total number of UEs where active means the UEs are

exchanging data with the network.";

//FIXME when 'somepath/sST = 1';

type integer-percentage;

reference "3GPP TS 22.261 clause 7.1, table 7.1-1";

}

// uRLLC leafs, SST = 2

leaf cSAvailabilityTarget {

description "Reliability uptime target";

//FIXME when 'somepath/sST = 2';

type decimal64 {

fraction-digits 6; // E.g. 99.999999

range 0..100;

}

reference "3GPP TS 22.104 clause 5.2-5.5";

}

leaf cSReliabilityMeanTime {

description "Mean time between failures";

//FIXME when 'somepath/sST = 2';

type reliability-string;

}

leaf expDataRate {

description "User experienced data rate";

//FIXME when 'somepath/sST = 2';

type data-rate;

reference "3GPP TS 22.104 clause 5.2-5.5";

}

leaf msgSizeByte {

description "PDU size";

//FIXME when 'somepath/sST = 2';

type message-size-string;

}

leaf transferIntervalTarget {

description "Time difference between two consecutive transfers

of application data from an application via the service interface

to 3GPP system";

//FIXME when 'somepath/sST = 2';

type transfer-interval-string;

}

leaf survivalTime {

description "The time that an application consuming a communication

service may continue without an anticipated message";

//FIXME when 'somepath/sST = 2';

type survival-time-string;

}

}

}

## E.6.4	module _3gpp-ns-nrm-serviceprofile.yang

submodule _3gpp-ns-nrm-serviceprofile {

yang-version 1.1;

belongs-to _3gpp-ns-nrm-networkslice { prefix ns3gpp; }

import _3gpp-common-yang-types { prefix types3gpp; }

organization "3GPP SA5";

description "A network slice instance in a 5G network.";

//FIXME: Add contact? No other _3gpp modules seem to have a contact stmt.

reference "3GPP TS 28.541

Management and orchestration;

5G Network Resource Model (NRM);

Information model definitions for network slice NRM (chapter 6)

";

revision 2020-01-15 {

description "Introduction of YANG definitions for network slice NRM";

reference "CR-0438";

}

revision 2019-06-23 {

description "Initial revision";

reference "3GPP TS 28.541 V15.X.XX";

}

//FIXME: Move typedefs to one of the types modules?

typedef availability-percentage {

description "

percentage value of the amount of time the end-to-end communication

service is delivered according to an agreed QoS, divided by the amount

of time the system is expected to deliver the end-to-end service

according to the specification in a specific area.";

reference "3GPP TS 22.261 3.1";

type decimal64 {

fraction-digits 4; // E.g. 99.9999

range 0..100;

}

}

typedef Category-enum {

type enumeration {

enum character;

enum scalability;

}

}

typedef Tagging-enum {

type enumeration {

enum performance;

enum function;

enum operation;

}

}

typedef Exposure-enum {

type enumeration {

enum API;

enum KPI;

}

}

typedef Support-enum {

type enumeration {

enum NOT_SUPPORTED;

enum SUPPORTED;

}

}

grouping ServAttrComGrp {

leaf category {

description "This attribute specifies the category of a service

requirement/attribute of GST";

type Category-enum;

}

leaf-list tagging {

description "This attribute specifies the tagging of a service

requirement/attribute of GST in character category";

when "../category = 'character'";

type Tagging-enum;

}

leaf exposure {

description "This attribute specifies exposure mode of a service

requirement/attribute of GST";

type Exposure-enum;

}

}

typedef DeterminCommAvailability {

type Support-enum;

}

grouping DLThptGrp {

container servAttrCom {

config false;

uses ServAttrComGrp;

}

leaf guaThpt {

description "This attribute describes the guaranteed data rate.";

type uint64; //FIXME: 28.541 says real, but I think that is to get a wide range. This is the YANG way to do that.

units kbits/s;

}

leaf maxThpt {

description "This attribute describes the maximum data rate.";

type uint64; //FIXME: 28.541 says real, but I think that is to get a wide range. This is the YANG way to do that.

units kbits/s;

}

}

typedef V2XMode-enum {

type enumeration {

enum NOT_SUPPORTED;

enum SUPPORTED_BY_NR;

}

}

grouping ServiceProfileGrp {

leaf serviceProfileId {

description "Service profile identifier.";

type types3gpp:DistinguishedName;

}

leaf-list sNSSAIList {

description "The S-NSSAI list to be supported by the new NSI to be

created or the existing NSI to be re-used.";

min-elements 1;

type types3gpp:SNssai;

}

list pLMNIdList {

description "List of PLMN IDs.";

min-elements 1;

key "mcc mnc";

ordered-by user;

uses types3gpp:PLMNId;

}

leaf maxNumberofUEs {

description "The maximum number of UEs that may simultaneously

access the network slice instance.";

mandatory true;

type uint64;

}

leaf-list coverageArea {

min-elements 1;

description "A list of TrackingAreas where the NSI can be selected.";

type types3gpp:Tac;

}

leaf latency {

description "The packet transmission latency (milliseconds) through

the RAN, CN, and TN part of 5G network, used to evaluate utilization

performance of the end-to-end network slice instance.";

reference "3GPP TS 28.554 clause 6.3.1";

mandatory true;

type uint16;

units milliseconds;

}

leaf uEMobilityLevel {

description "The mobility level of UE accessing the network slice

instance.";

reference "3GPP TS 22.261 clause 6.2.1";

type types3gpp:UeMobilityLevel;

}

leaf resourceSharingLevel {

description "Specifies whether the resources to be allocated to the

network slice instance may be shared with another network slice

instance(s).";

type types3gpp:ResourceSharingLevel;

}

leaf sST {

description "Specifies the slice/service type. See 3GPP TS 23.501

for defined values.";

mandatory true;

type uint32;

reference "3GPP TS 23.501 5.15.2.2";

}

leaf availability {

description "The availability requirement for a network slice

instance, expressed as a percentage.";

type availability-percentage;

}

//FIXME: delayTolerance has multiplicity 1 => container?

container delayTolerance {

description "An attribute specifies the properties of service delivery

flexibility, especially for the vertical services that are not

chasing a high system performance.";

reference "TS 22.104 clause 4.3";

config false;

//FIXME: servAttrCom has multiplicity 1 => container?

container servAttrCom {

uses ServAttrComGrp;

}

leaf support {

description "An attribute specifies whether or not the network

slice supports service delivery flexibility, especially for the

vertical services that are not chasing a high system performance.";

type Support-enum;

}

}

//FIXME: Not defined in 28.541 chapter 6, but I guess they mean determinComm

//FIXME: Guessing: deterministicComm has multiplicity 1 => container?

container deterministicComm {

//FIXME: servAttrCom has multiplicity 1 => container?

container servAttrCom {

config false;

uses ServAttrComGrp;

}

leaf availability {

//FIXME: Defined differently in 28.541 chapter 6, but XML uses DeterminCommAvailability

config false;

type DeterminCommAvailability;

}

leaf periodicityList {

//FIXME: Not defined in 28.541 chapter 6. XML and YAML says "string".

type string;

}

}

//FIXME: dLThptPerSlice has multiplicity 1 => container?

container dLThptPerSlice {

description "This attribute defines achievable data rate of the

network slice in downlink that is available ubiquitously across

the coverage area of the slice";

uses DLThptGrp;

}

//FIXME: dLThptPerUE has multiplicity 1 => container?

container dLThptPerUE {

description "This attribute defines data rate supported by the network

slice per UE";

uses DLThptGrp;

}

//FIXME: uLThptPerSlic has multiplicity 1 => container?

container uLThptPerSlic {

description "This attribute defines achievable data rate of the

network slice in uplink that is available ubiquitously across

the coverage area of the slice";

uses DLThptGrp;

}

//FIXME: uLThptPerUE has multiplicity 1 => container?

container uLThptPerUE {

description "This attribute defines data rate supported by the

network slice per UE";

uses DLThptGrp;

}

//FIXME: maxPktSize has multiplicity 1 => container?

container maxPktSize {

config false;

description "This parameter specifies the maximum packet size

supported by the network slice";

//FIXME: servAttrCom has multiplicity 1 => container?

container servAttrCom {

uses ServAttrComGrp;

}

leaf maxSize {

//FIXME: Not defined in 28.541, so I'm guessing

type uint32;

units bytes;

}

}

//FIXME: maxNumberofPDUSessions has multiplicity 1 => container?

container maxNumberofPDUSessions {

description "Represents the maximum number of

concurrent PDU sessions supported by the network slice";

config false;

container servAttrCom {

uses ServAttrComGrp;

}

leaf nOofPDUSessions {

//FIXME: Not defined in 28.541, so I'm guessing

type uint32;

}

}

//FIXME: kPIMonitoring has multiplicity 1 => container?

container kPIMonitoring {

description "Represents performance monitoring";

config false;

container servAttrCom {

uses ServAttrComGrp;

}

leaf kPIList {

description "An attribute specifies the name list of KQIs and KPIs

available for performance monitoring";

type string; //FIXME: Unusable unless some format is specified

}

}

//FIXME: userMgmtOpen has multiplicity 1 => container?

container userMgmtOpen {

description "An attribute specifies whether or not the network slice

supports the capability for the NSC to manage their users or groups

of users’ network services and corresponding requirements.";

config false;

container servAttrCom {

uses ServAttrComGrp;

}

leaf support {

type Support-enum;

}

}

//FIXME: v2XCommModels has multiplicity 1 => container?

container v2XCommModels {

description "An attribute specifies whether or not the V2X

communication mode is supported by the network slice.";

config false;

container servAttrCom {

uses ServAttrComGrp;

}

leaf v2XMode {

type V2XMode-enum;

}

}

//FIXME: termDensity has multiplicity 1 => container?

container termDensity {

description "An attribute specifies the overall user density over

the coverage area of the network slice";

config false;

container servAttrCom {

uses ServAttrComGrp;

}

leaf density {

type uint32;

units users/km2;

}

}

leaf activityFactor {

description "An attribute specifies the percentage value of the

amount of simultaneous active UEs to the total number of UEs where

active means the UEs are exchanging data with the network";

reference "TS 22.261 Table 7.1-1";

//FIXME: This is modeled as writable/config true, but that the stage 2 intent??

type decimal64 {

fraction-digits 1;

}

}

leaf uESpeed {

description "An attribute specifies the maximum speed (in km/hour)

supported by the network slice at which a defined QoS can be

achieved";

//FIXME: This is modeled as writable/config true, but that the stage 2 intent??

type uint32;

units km/h;

}

leaf jitter {

description "An attribute specifies the deviation from the desired

value to the actual value when assessing time parameters";

reference "TS 22.104 clause C.4.1";

//FIXME: This is modeled as writable/config true, but that the stage 2 intent??

type uint32;

units microseconds; //FIXME: is this a reasonable unit for packet jitter?

}

leaf survivalTime {

description "An attribute specifies the time that an application

consuming a communication service may continue without an

anticipated message.";

reference "TS 22.104 clause 5";

type string; //FIXME: Use typedef survival-time-string from _3gpp-ns-nrm-perfreq?

}

leaf reliability {

description "An attribute specifies in the context of network layer

packet transmissions, percentage value of the amount of sent

network layer packets successfully delivered to a given system

entity within the time constraint required by the targeted service,

divided by the total number of sent network layer packets.";

reference "TS 22.261, TS 22.104";

type string; //FIXME: Use typedef reliability-string from _3gpp-ns-nrm-perfreq?

}

//FIXME: Not defined in 28.541. XML and YAML says "string".

leaf maxDLDataVolume {

type string;

}

//FIXME: Not defined in 28.541. XML and YAML says "string".

leaf maxULDataVolume {

type string;

}

//FIXME: nBIoT has multiplicity 1 => container?

container nBIoT {

description "An attribute specifies whether NB-IoT is supported in

the RAN in the network slice";

config false;

container servAttrCom {

uses ServAttrComGrp;

}

leaf support {

description "An attribute specifies whether NB-IoT is supported

in the RAN in the network slice";

type Support-enum;

}

}

}

}

## E.6.5	module _3gpp-ns-nrm-sliceprofile.yang

submodule _3gpp-ns-nrm-sliceprofile {

yang-version 1.1;

belongs-to _3gpp-ns-nrm-networkslicesubnet { prefix nss3gpp; }

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-ns-nrm-perfreq { prefix perf3gpp; }

organization "3GPP SA5";

description "Represents the properties of network slice subnet related

requirement that should be supported by the network slice subnet

instance in a 5G network.";

//FIXME: Add contact? No other _3gpp modules seem to have a contact stmt.

reference "3GPP TS 28.541

Management and orchestration;

5G Network Resource Model (NRM);

Information model definitions for network slice NRM (chapter 6)

";

revision 2020-01-15 {

description "Introduction of YANG definitions for network slice NRM";

reference "CR-0438";

}

revision 2019-05-27 {

description "initial revision.";

reference "Based on

3GPP TS 28.541 V15.X.XX";

}

grouping SliceProfileGrp {

leaf sliceProfileId {

description "A unique identifier of the property of network slice

subnet related requirement should be supported by the network

slice subnet instance.";

type types3gpp:DistinguishedName;

}

leaf-list sNSSAIList {

description "List of S-NSSAIs the managed object is capable of

supporting. (Single Network Slice Selection Assistance Information)

An S-NSSAI has an SST (Slice/Service type) and an optional SD

(Slice Differentiator) field.";

type types3gpp:SNssai;

}

list pLMNIdList {

description "List of at most six entries of PLMN Identifiers, but at

least one (the primary PLMN Id).  The PLMN Identifier is composed

of a Mobile Country Code (MCC) and a Mobile Network Code (MNC).";

min-elements 1;

max-elements 6;

key "mcc mnc";

ordered-by user;

uses types3gpp:PLMNId;

}

//FIXME: See 28.541 clause 6.4.1 regarding perfReq:

// "Depending on the sST value, the list of perfReq will be ..."

// but there is no sST value in SliceProfile.

// Including all variants until resolved

list perfReq {

description "The performance requirements for the NSI in terms of the

scenarios defined in the 3GPP TS 22.261, such as experienced data

rate, area traffic capacity (density) information of UE density.";

key idx; //this list uses a grouping/choice and has no obvious key

leaf idx { type uint32; }

uses perf3gpp:PerfReqGrp;

}

leaf maxNumberofUEs {

description "Specifies the maximum number of UEs may simultaneously

access the network slice instance.";

//optional support

mandatory true;

type uint64;

}

leaf-list coverageAreaTAList {

description "A list of TrackingAreas where the NSI can be selected.";

//optional support

min-elements 1;

type types3gpp:Tac;

}

leaf latency {

description "The packet transmission latency (milliseconds) through

the RAN, CN, and TN part of 5G network, used to evaluate

utilization performance of the end-to-end network slice instance.";

reference "3GPP TS 28.554 clause 6.3.1";

//optional support

mandatory true;

type uint16;

units milliseconds;

}

leaf uEMobilityLevel {

description "The mobility level of UE accessing the network slice

instance.";

//optional support

type types3gpp:UeMobilityLevel;

}

leaf resourceSharingLevel {

description "Specifies whether the resources to be allocated to the

network slice subnet instance may be shared with another network

slice subnet instance(s).";

//optional support

type types3gpp:ResourceSharingLevel;

}

}

}

Next Change

End of Change