3GPP TSG SA WG5 Meeting 135-e		S5-211226

# electronic meeting, online, 25 January - 3 February 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.313 | CR | 0014 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update procedures for plug and connect to management sysstem |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | S5 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Huawei,China Telecom |  |  |  |  |  |  |  |  |  |
| Work item code: | eSON_5G |  |  |  |  |  | Date: |  |  | 2020.1.15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - In Clause 8.3.2.1, the sub procedures ("High-level plug-and-connect", "Initial IP Autoconfiguration" procedure, "Certificate Enrolment","Establishing Secure Connection" and "Establishing Connection to MnF") refers to TS 32.508, however, such procedure in TS 32.508 is specific for eNB. So contribution S5-211227 proposes to update such procedures to be applicable for generic RAN NE which can be applicable for both eNB and gNB; - The procedure “for RAN NE plug and connect to management system” is similar as procedure “High-level plug-and-connect” expect the actor. However, in contribution S5-211227 proposed to upate the procedure “High-level plug-and-connect” to be applicable for generic RAN NE which can be applicable for both eNB and gNB. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the clause 8.3.2.1 to refers to TS 32.508. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3.2.1,A.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This contribution depends on the approval of S5-211227 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

8.3.2.1	Procedures for RAN NE plug and connect to management system

The procedure flows defined in clause 5 in TS 32.508[15] can be used for RAN NEplug and connect to management system. The RAN NE described in there can be gNB in non-split scenario and gNB-DU in split scenario.

Note 1: 	The NE within virtualization is not addressed.

2nd Change

A.1.1	Void

End of Change