3GPP TSG SA WG5 Meeting 135-e		TDoc S5-211298

# electronic meeting, online, 25 January - 3 February 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.275 | CR | 0079 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduce Post Event Charging (PEC) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSIMSCH |  |  |  |  |  | Date: |  |  | 2021-01-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The Post Event Charging (PEC) mode is applicable and needs to be introduced. The current flows for OIP/OIR cover the PEC mode and they can be replaced by existing flows on offline charging for the simplification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce PEC and replace the OIP/OIR flows by referring to existing flows for offline. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear how the PEC mode is achieved |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.1.1, 5.4.2.2, 5.4.2.3, 5.4.2.x (New), 5.4.2.y(New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

#### 5.4.1.1	General

Converged charging may be performed by the MMTel AS interacting with CHF using Nchf specified in TS 32.290 [57] and TS 32.291 [58]. To provide the data required for the management activities outlined in TS 32.240 [1] (Credit-Control, accounting, billing, statistics etc.), the MMTel AS shall be able to perform converged charging for each of the MMTel supplementary service.

The CTFs implementing the MMTel converged charging functionality may apply:

-	Immediate Event Charging (IEC) with Charging Data Request [Event] generated, or

-	Session based charging SCUR, with Charging Data Request [Initial, Update, Termination], ot

-	Post Event Charging (PEC) with Charging Data Request [Event].

The circumstances on which IEC, SCUR or PEC are applicable, depend on the MMTel supplementary service and/or operator's policy.

Editor's note:	The use of ECUR is FFS.

Converged charging uses centralized or decentralized unit determination and centralized rating scenarios for event based convergent charging specified in TS 32.290 [57].

The contents and purpose of each charging event that triggers interaction with CHF, as well as the chargeable events that trigger them, are described in the following clauses.

A detailed formal description of the converged charging parameters defined in the present document is to be found in TS 32.291 [58].

A detailed formal description of the CDR parameters defined in the present document is to be found in TS 32.298 [51].

The selection of the CHF can be configured in the MMTel AS.

Editor's note:	The CHF selection mechanism using other means, e.g. SIP and user profile, is FFS.

Next change

#### 5.4.2.2	Originating Identification Presentation (OIP) charging

The PEC mode for Originating Identification Presentation (OIP) converged charging is described in figure 5.2.2.1.1.1 with the CDF replaced by CHF.

Next change

#### 5.4.2.3	Originating Identification Restriction (OIR) charging

The PEC mode for Originating Identification Restriction (OIR) converged charging is described in figure 5.2.2.1.2.1 with the CDF replaced by CHF.

Next change

#### 5.4.2.x	Terminating Identification Presentation (TIP) charging

The PEC mode for Terminating Identification Presentation (TIP) converged charging is described in figure 5.2.2.1.3.1 with the CDF replaced by CHF.

Next change

#### 5.4.2.y	Terminating Identification Restriction (TIR) charging

The PEC mode for Terminating Identification Restriction (TIR) converged charging is described in figure 5.2.2.1.4.1 with the CDF replaced by CHF.

End of changes