3GPP TSG-SA5 Meeting #135e 	S5-211033rev1

- e-meeting, 25 January - 3 February 2021

- Source:	Orange

- Title:	Update clause 4.4 to align with already approved CR

- Document for:	Approval

Agenda Item:	6.5.2

# 1	Decision/action requested

The group is asked to discuss and agree on the proposal.

# 2	References

[1]	TR 28.813 0.3.0: Study on new aspects of Energy Efficiency (EE) for 5G

# 3	Rationale

At SA#90e, S5-206370 (Rel-17 CR TS 28.554 Add EE KPI definitions for network slices) has been approved. The present contribution proposes to align the text in clause 4.4 of TR 28.813 with the text in S5-206370.

# 4	Detailed proposal

This document proposes the following changes in TR 28.813 [1].

1st Change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "5G; System Architecture for the 5G System".

[3]	3GPP TS 32.130: "Telecommunication management; Network sharing; Concepts and requirements".

[4]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[5]	3GPP TS 28.554: "Management and orchestration; 5G end to end Key Performance Indicators (KPI)".

[6]	ETSI ES 203 228 v1.2.1: "Environmental Engineering (EE); Assessment of mobile network energy efficiency".

[7]	S5-201169/S2-1912770: LS on analytics support for energy saving

[8]	3GPP TS 28.622: "Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

[9]	Draft ETSI GR NFV-IFA 015 V3.4.1 (2020-05): "Network Functions Virtualisation (NFV) Release 3; Management and Orchestration; Report on NFV Information Model".

[10]	ETSI GR NFV 003 V1.5.1 (2020-01): "Network Functions Virtualisation (NFV); Terminology for Main Concepts in NFV".[11]	3GPP TS 28.809: "Management and orchestration; Study on enhancement of Management Data Analytics (MDA)".

[12]	3GPP TS 28.310: "Management and orchestration; Energy efficiency of 5G".

[13]	IETF RFC 5136: "Defining Network Capacity".

[14]	ETSI GS NFV-TST 008 V3.3.1: "Network Functions Virtualisation (NFV) Release 3; Testing; NFVI Compute and Network Metrics Specification".

[x]	S5-206370: "Rel-17 CR TS 28.554 Add EE KPI definitions for network slices".

Next Change

## 4.4	Key Issue #4: EE KPI for network slice types

### 4.4.1	Description

Besides defining EE KPIs for NG-RAN and 5GC, there is a need to define EE KPIs for standardized network slice types. A Generic Network Slice EE KPI could be expressed as follows:

where:

- ‘Performance of network slice’ (Pns) is defined per type of network slice. For example:

- For eMBB, ‘Performance of network slice’ could be the Data Volume (DV) carried through network slices of type eMBB. The higher the Data Volume carried by an eMBB network slice is, the higher its Performance of network slice is;

- For URLLC, ‘Performance of network slice’ could be related to the network slice end-to-end latency or reliability, or a combination of both. The lower the latency or higher the reliability within a URLLC network slice is, the higher its Performance of network slice is;

- For MIoT, ‘Performance of network slice’ could be the number of registered subscribers, the number of active UEs, etc.. The higher the number of registered subscribers or active UEs supported by a MIoT network slice is, the higher its Performance of network slice is;

- For V2X, ‘Performance of network slice’ is still FFS.

In clause 4.4.2, potential solutions to define the Performance of network slice of aforementioned types of network slice are proposed.

- ECns is the Energy Consumption of the network slice.

Editor’s note: the definition of ECns is FFS.

### 4.4.2	Potential solutions

#### 4.4.2.1	Potential solution #1: Performance of network slice for eMBB

##### 4.4.2.1.1	Introduction

In this potential solution, the Performance of network slice (Pns) for eMBB type of network slice is obtained by summing up UL and DL traffic volumes at N3 interface, on a per S-NSSAI basis.

##### 4.4.2.1.2	Description

The Performance of network slice (Pns) for eMBB type of network slice is obtained by summing up UL and DL traffic volumes at N3 interface, on a per S-NSSAI basis:

The final Network Slice EE KPI definition, based on Data Volume, for eMBB type of slice, would be defined as follows:

The following measurements, defined in TS 28.552 [4], are used:

- GTP.InDataOctN3UPF.SNSSAI: Number of octets of incoming GTP data packets on the N3 interface, from (R)AN to UPF), where SNSSAI identifies the S-NSSAI, as defined in TS 28.552 [4] clause 5.4.1.3;

- GTP.OutDataOctN3UPF.SNSSAI: Number of octets of outgoing GTP data packets on the N3 interface, from (R)AN to UPF), where SNSSAI identifies the S-NSSAI, as defined in TS 28.552 [4] clause 5.4.1.4.

#### 4.4.2.2	Potential solution #2: Performance of network slice for URLLC

##### 4.4.2.2.1	Introduction

This potential solution focuses on the Low Latency (LL) characteristic of the URLLC type of network slice. The ‘Ultra Reliable’ (UR) characteristic of the URLLC type of network slice is not addressed by this potential solution.

In this potential solution, the Performance of network slice (Pns) for URLLC type of network slice is the inverse of the end-to-end User Plane (UP) latency of the network slice.

##### 4.4.2.2.2	Description

The Performance of network slice (Pns) for URLLC type of network slice is the inverse of the end-to-end User Plane (UP) latency of the network slice:

, where Network Slice Latency is defined as:

The final Network Slice EE KPI definition, based on Latency, for URLLC type of slice, would be defined as follows:

The following KPIs, defined in TS 28.554 [5], are used to calculate Network Slice Latency:

- DelayE2EUlNs: Average e2e uplink delay for a network slice, defined in TS 28.554 [5] clause 6.3.1.8.1 as the average e2e UL packet delay between the PSA UPF and the UE for a network slice;

- DelayE2EDlNs: Average e2e downlink delay for a network slice, defined in TS 28.554 [5] clause 6.3.1.8.2 as the average e2e DL packet delay between the PSA UPF and the UE for a network slice.

Editor’s note: further study is needed to potentially add Data Volume to measure the performance of the network slice for URLLC.

#### 4.4.2.3	Potential solution #3: Performance of network slice for MIoT

##### 4.4.2.3.1	Solution based on the number of registered subscribers

###### 4.4.2.3.1.1	Introduction

This potential solution focuses on the number of registered subscribers to the MIoT type of network slice. It does not consider the number of active UEs.

In this potential solution, the Performance of network slice (NSPns) for MIoT type of network slice is obtained by the maximum number of registered subscribers per AMF, on a per S-NSSAI basis.

###### 4.4.2.3.1.2	Description

The Performance of network slice (Pns) for MIoT type of network slice, based on the maximum number of registered subscribers per AMF, on a per S-NSSAI basis, is defined as:

The following measurement, defined in TS 28.552 [4], is used:

- RM.RegisteredSubNbrMax.SNSSAI: maximum number of registered state subscribers per AMF, where SNSSAI identifies the S-NSSAI, as defined in TS 28.552 [4] clause 5.2.1.2.

The final Network Slice EE KPI definition, based on the maximum number of registered subscribers, for MIoT type of slice, would be defined as follows:

##### 4.4.2.3.2	Solution based on the number of active UEs

###### 4.4.2.3.2.1	Introduction

This potential solution focuses on the number of active UEs to the MIoT type of network slice.

In this potential solution, the Performance of network slice (Pns) for MIoT type of network slice is obtained by summing up the number of active UEs in the DL per cell and the number of active UEs in the UL per cell of the considered network elements over the measurement period, on a per S-NSSAI basis. The samples are aggregated at the NG-RAN node level.

###### 4.4.2.3.2.2	Description

The Performance of network slice (Pns) for MIoT type of network slice, based on the number of active UEs in the DL per cell and the number of active UEs in the UL per cell, on a per S-NSSAI basis, is defined as:

The following measurements, defined in TS 28.552 [4], are used:

- DRB.MeanActiveUeDl.SNSSAI, where SNSSAI identifies the S-NSSAI, as defined in TS 28.552 [4] clause 5.1.1.23.1.

- DRB.MeanActiveUeUl.SNSSAI, where SNSSAI identifies the S-NSSAI, as defined in TS 28.552 [4] clause 5.1.1.23.3.

The final Network Slice EE KPI definition, based on summing up the number of active UEs in the DL per cell and the number of active UEs in the UL per cell of the considered network elements over the measurement period, for MIoT type of slice, would be defined as follows:

### 4.4.3	Impact on normative work

The definitions of:

- Generic network slice EE KPI - see clause 4.4.1,

- EE KPI for eMBB based on Data Volume (𝐸𝐸𝑒𝑀𝐵𝐵,𝐷𝑉) - see clause 4.4.2.1.2,

- EE KPI for URLLC based on network slice latency (EEURLLC,Latency) - see clause 4.4.2.2.2,

- EE KPI for MIoT based on the maximum number of registered subscribers (EEMIoT,RegSubs) - see clause 4.4.2.3.1.2,

- EE KPI for MIoT based on the mean number of active UEs (EEMIoT,ActiveUEs) - see clause 4.4.2.3.2.2

have been introduced in TS 28.554 [5] via S5-206370 (Rel-17 CR TS 28.554 Add EE KPI definitions for network slices) [x].

End of change