3GPP TSG SA WG5 Meeting 135-e		TDoc S5-211067

- electronic meeting, online, 25 January - 3 February 2021

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.530 | CR | 0038 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add use cases for discovery of management services |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GDMS |  |  |  |  |  | Date: |  |  | 2021-01-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Use cases and requirements for Discovery of Management Services are moved from 28.533 because 28.530 is a more suitable document. Use cases are extended and rewritten. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add new use cases and requirements. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear use cases. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 5.1.4 (new), 5.4.16 (new), 5.4.17 (new), 5.4.18 (new), 5.4.19 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.261 "Service requirements for next generation new services and markets".

[3]	3GPP TS 23.501: " System Architecture for the 5G system".

[4]	3GPP TS 38.401 "NG-RAN; Architecture description".

[5]	3GPP TS 28.531: "Management and orchestration; Provisioning".

[6]	3GPP TS 28.541: "Management and orchestration ; 5G Network Resource Model (NRM); Stage 2 and stage3".

[x]	3GPP TS 28.533: " Management and orchestration; Architecture framework".

2nd Change

### 5.1.4	Discovery of Management Services

REQ-DMS-CON-X	The MnS discovery service producer shall provide capabilities allowing its authorized consumer to publish its MnS data for use by other authorized MnS discovery service consumers.

REQ-DMS-CON-Y	The MnS discovery service producer shall provide capabilities allowing its authorized consumer to retrieve the published MnS data.

3rd Change

### 5.4.16	Adding a new management service producer

| Use case stage | Evolution/Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal | Add a new MnS producer to a 3GPP management system. |  |
| Actors and Roles | Network operator. |  |
| Telecom resources | New MnS producer. MnS discovery service producer. |  |
| Assumptions | New MnS producer is authorized to publish the MnS data [x] for a management service to MnS discovery service producer. |  |
| Pre-conditions | - |  |
| Begins when | Network operator wishes to add a new MnS producer to the 3GPP management system. |  |
| Step 1 (M) | Network operator adds the new MnS producer to the 3GPP management system. |  |
| Step 2 (M) | Network operator activates the new MnS producer. |  |
| Step 3 (M) | New MnS producer sends a request to MnS discovery service producer to publish the MnS data. |  |
| Step 4 (M) | MnS discovery service producer stores the MnS data. |  |
| Step 5 (M) | MnS discovery service producer sends confirmation to the new MnS producer that the MnS data has been stored. |  |
| Ends when | All the steps identified above are successfully completed. |  |
| Exceptions | One of the mandatory steps fails. |  |
| Post-conditions | MnS discovery service producer has stored the MnS data. |  |
| Traceability | REQ-DMS-CON-X |  |

### 5.4.17	Removing a management service producer

| Use case stage | Evolution/Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal | Remove an existing MnS producer in a 3GPP management system. |  |
| Actors and Roles | Network operator. |  |
| Telecom resources | MnS producer. MnS discovery service producer. |  |
| Assumptions | - |  |
| Pre-conditions | MnS discovery service producer has stored the MnS data for the MnS producer. The network operator has decided that it is safe to remove the MnS producer. |  |
| Begins when | Network operator wishes to remove an existing MnS producer in the 3GPP management system. |  |
| Step 1 (M) | Network operator orders removal of the MnS producer. |  |
| Step 2 (M) | MnS producer sends a request to MnS discovery service producer to remove the MnS data related to the MnS Producer. |  |
| Step 3 (M) | MnS discovery service producer removes the MnS data related to the MnS Producer. |  |
| Step 4 (M) | MnS discovery service producer sends confirmation to the MnS producer that the MnS data has been removed. |  |
| Ends when | All the steps identified above are successfully completed. |  |
| Exceptions | One of the mandatory steps fails. |  |
| Post-conditions | MnS discovery service producer has removed the MnS data related to the MnS Producer. |  |
| Traceability | REQ-DMS-CON-X |  |

### 5.4.18	Adding a new management service consumer

| Use case stage | Evolution/Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal | Add a new MnS consumer of an MnS provided by a 3GPP management system. |  |
| Actors and Roles | Network operator. |  |
| Telecom resources | New MnS consumer. MnS discovery service producer. |  |
| Assumptions | New MnS consumer is authorized to obtain the MnS data [x] for the available a management service(s) from MnS discovery service producer. |  |
| Pre-conditions | MnS data exists in MnS discovery service producer. |  |
| Begins when | Network operator wishes to add a new MnS consumer of an MnS provided by a 3GPP management system. |  |
| Step 1 (M) | Network operator adds the new MnS consumer to the 3GPP management system or to an external system. |  |
| Step 2 (M) | Network operator configures the new MnS consumer with the address of the MnS discovery service producer. |  |
| Step 3 (M) | Network operator activates the new MnS consumer. |  |
| Step 4 (M) | For each MnS that the new MnS consumer wishes to consume, the following steps are repeated. |  |
| Step 4.1 (M) | New MnS consumer sends a request to MnS discovery service producer to read the MnS data for relevant management services. The request may optionally request that the MnS data is filtered by MnS type (e.g. provisioning MnS, fault supervision MnS, performance assurance MnS) and description of MnS components. |  |
| Step 4.2 (M) | MnS discovery service producer reads the stored MnS data of the management service(s) and decides if the MnS data of the management service(s) can satisfy the received request, then sends the relevant MnS data of the management service(s) to the new MnS Consumer. |  |
| Step 4.3 (M) | New MnS consumer reads the MnS data to check if a suitable version of the desired MnS is available. If more than one suitable version exists, the new MnS consumer selects the management service with the highest suitable version of the MnS. |  |
| Step 4.4 (M) | New MnS consumer reads the MnS data for the selected management service to find details of the MnS producer for example root URL for MnS operations and root DN for MnS objects. |  |
| Ends when | All the steps identified above are successfully completed. |  |
| Exceptions | One of the mandatory steps fails. |  |
| Post-conditions | New MnS consumer is ready to use the selected version for each needed MnS. |  |
| Traceability | REQ-DMS-CON-Y |  |

### management service consumer

End of changes