3GPP TSG SA WG5 Meeting 135-e	S5-211071

electronic meeting, online, 25 January - 3 February 2021

- Source:	Huawei Technologies Co.,Ltd.

- Title:	pCR 28.814 solution for ECS deployment

- Document for:	Approval

Agenda Item:	6.5.3

# 1	Decision/action requested

The group is asked to discuss and approve the proposals.

# 2	References

[1] 	TR 28.814 “Study on enhancements of edge computing management”

[2] 	3GPP TS 23.558: "Architecture for enabling Edge Applications;"

# 3	Rationale

Adds an potention solution for ECS deployment.

# 4	Detailed proposal

1st modified section

## 7.x 	ECS lifecycle management by PLMN operator

### 7.x.1	General

Figure 7.x.1-1 shows that the PLMN operator as the consumer requests the ECS lifecycle management via the provisioning MnS.

Figure 7.x.1-1: ECS lifecycle management via provisioning MnS

This solution is to support the ECS instantiation and termination use case in 6.2.5 and 6.2.6. To support ECS deployment via provisioning MnS, the ECSFunction IOCs should be defined with the following properties:

-	ECSFunction IOC ID: the identifier of ECS VNF instance.

-	End Point: The endpoint address (e.g. URI, IP address) of the ECS.

NOTE: End Point could also be defined as a separate IOC with relationship with ECSFunction IOC.

-	Serving EES: Information about EES(s) known to ECS including EES end point

NOTE: Serving EES could also be defined as a separate IOC with relationship with ECSFunction IOC.

### 7.x.2	ECS instantiation

The ECS instantiation procedure shall follow the NF instance creation procedure as described in 7.10 of [5].

### 7.x.3	ECS termination

The ECS termination procedure shall follow the NF instance deletion procedure as described in 7.12 of [5].

End of modified section

-