3GPP TSG SA WG5 Meeting 135-e		TDoc S5-211074

- electronic meeting, online, 25 January - 3 February 2021

- Source:	Huawei

- Title:	pCR 28.811 Add deployment of edge application service

- Document for:	Approval

Agenda Item:	6.5.5

# 1	Decision/action requested

The group is asked to approve the proposal.

# 2	References

[1]	3GPP TR 28.811 v0.2.0: “Management and orchestration; Network Slice Management Enhancement”

# 3	Rationale

This contribution adds a scenario related to deployment of edge application service and identifies possible problems.

# 4	Detailed proposal

This contribution proposes to make the following changes in [1].

1st change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TS 23.501 v16.6.0: "System architecture for the 5G System (5GS); Stage 2"

[2]	3GPP TS 28.541 v17.0.0: "5G Network Resource Model (NRM); Stage 2 and stage 3"

[3]	3GPP TS 28.531 v16.7.0: "Management and orchestration; Provisioning"

[4]	3GPP TS 28.530 v16.3.0: "Management and orchestration; Concepts, use cases and requirements"

[x]	3GPP TS 23.558 v1.2.0: "Architecture for enabling Edge Applications"

2nd change

## 5.x	Deployment of edge application service

#### 5.x.1	Description

NOP wishes to provide an edge application service which is to be deployed as a network slice.

NOP deploys a network slice for the new service, which includes Edge Application Server (EAS) and Edge Enablement Server (EES) as Core Network NFs. An S-NSSAI is allocated for this new network slice.

If NOP is the owner of the Edge Configuration Server (ECS), NOP configures ECS with the relevant S-NSSAI for the new edge application service. Otherwise, NOP informs the owner of the ECS of the relevant S-NSSAI and the owner of the ECS configures the ECS.

When a UE registers on the network, the UE sends a service provisioning request to ECS. The response from ECS includes the S-NSSAI for the edge application service (ref 23.558 [x] clause 8.3.3).

#### 5.x.2	Identified problems

NRM for Edge Configuration Server should include S-NSSAI for the edge application service.

Editor’s note: This clause may need to be updated depending on changes in 23.558 and 28.814.

End of changes