3GPP TSG-SA5 Meeting #135e 	S5-211094d2

electronic meeting, online, 25th January 2021 - 3rd February 2021

Title:	[DRAFT] Response to LS Reply LS on Enhancement of RAN Slicing

Response to:	LS Reply (R3-207236) on Enhancement of RAN Slicing

Release:	Release 17

Source:	3GPP SA5

To:	3GPP RAN3

Cc:	3GPP SA2

Contact Person:

#### Name:	 Jan Önnegren

Tel. Number:

E-mail Address:	Jan.onnegren@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

- Overall description:

SA5 thanks 3GPP RAN3 for the LS on “Response to LS Reply on Enhancement of RAN Slicing” and would like to provide the following feedback to the solutions for supporting scenario 1 and scenario 2 in TR 38.832-030.

- Regarding candidate solutions 6.2.1, 6.2.2 and 6.2.4, the need for and enforcement of remapping of S-NSSAI is outside the scope of SA5 and has not been discussed, but the management support needed can be provided as required.

- Regarding candidate solution 6.2.3 (Configuration Based Solution), the concept of RRMPolicyRatio is available as defined by SA5. It may be dynamically modified to accommodate for local traffic situations. Shared resources are always available for contention. Resources with priority for certain slices are shared when not used, but these certain slices could pre-empt the used prioritized resources from other slices outside the assigned group of slices. Un-used, dedicated resources cannot currently be shared outside the assigned group of slices. A study in SA5 may be needed if further capabilities are deemed required by RAN. Pre-emption is primarily a question for RAN3, where SA5 will provide management capabilities as required. RRMPolicy defined in TS 28.541 can therefore be useful for scenario 1 (Slice resource shortage in case of Intra-RA mobility and Inter-RA mobility) without needing remapping between different S-NSSAIs but is not useful for scenario 2 (Non-supported slice in case of Inter-RA mobility).

- Regarding candidate solution 6.2.5 (Slice resource re-partitioning), the concept of RRMPolicyRatio is available as defined by SA5. It may be dynamically modified to accommodate for local traffic situations, therefore re-partitioning resources (a.k.a reconfiguring ratios) between groups of network slices is possible. Shared resources are always available for contention. Resources with priority for certain slices are shared when not used, i.e. making the partition soft. A study in SA5 may be needed if further capabilities are deemed required by RAN.

- Regarding solution 6.2.6, the concept of RRMPolicyRatio is configurable per cell, but not per frequency, as defined by SA5. Setting up DC or CA is outside the scope of SA5.

- Regarding solution 6.2.7 and 6.2.8, they are considered outside the scope of SA5 work.

# 2. Actions:

To 3GPP RAN3

ACTION:

- SA5 requests RAN3 to consider the information provided above.

# 3. Dates of next TSG SA WG 5 meetings

SA5#136-e	1st March 2021 - 10th March 2021	electronic meeting

SA5#137-e	10th - 19th May 2021		electronic meeting