3GPP TSG-SA5 Meeting #135e 	S5-211105rev3

25 January to 03 February 2021, E-meeting

- Source:	Intel, NEC

- Title:	New WID Enhancements of Management Data Analytics Service

- Document for:	Approval

Agenda Item:	6.2

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

# Title: 	Enhancements of Management Data Analytics Service

## Acronym: eMDAS

## Unique identifier: 	{A number to be provided by MCC at the plenary}

Potential target Release: {Rel-17}.

Note that this field above indicates the proposed Release at the time of submission of the WID to TSG approval. It can later be changed without a need to revise the WID. The updated target Release is indicated in the Work Plan.

## 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  |  | X | X |  |
| No |  | X |  |  |  |
| Don't know | X |  |  |  | X |

## 2	Classification of the Work Item and linked work items

### 2.1	Primary classification

This work item is a

| X | Feature |
| --- | --- |
|  | Building Block |
|  | Work Task |
|  | Study Item |

### 2.2	Parent Work Item

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| FS_eMDAS | SA5 | 850028 | Study on enhancement of Management Data Analytics Service |

### 2.3	Other related Work Items and dependencies

| Other related Work Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 880028 | MADCOL | Dependency on the solutions in this WI for handling historical data related to MDA (including MDA input and output) |

## 3	Justification

The MDA (Management Data Analytics), in conjunction with Artificial Intelligence (AI) and Machine Learning (ML) techniques, brings intelligence and assists automation to the network and service management & orchestration. MDA can identify the ongoing issues impacting the performance of the network and service, discover in advance the potential issues that would cause failure and/or performance degradation, and provide recommendations to solve or mitigate these issues, e.g., mobility issues, energy efficiency, etc. The MDA can also assist to predict the network and service demand to enable the timely resource provisioning and deployments which would allow fast time-to-market network and service deployment.

The MDAS (Management Data Analytics Service) is defined in TS 28.550 and TS 28.533 and is used as part of management loop achieving service assurance as described in TS 28.535 and TS 28.536. However, the detailed MDA reports and MDA management aspects are still yet to be defined.

The MDAS can be consumed by various consumers, for instance the MFs (i.e., MnS service producers/consumers for network and service management), NFs (e.g., NWDAF), SON functions, network and service optimization tools/functions, SLS assurance functions, human operators, etc.

SA5 has conducted a study on enhancements of MDAS in TR 28.809, which described the MDA functionality and service framework, MDA process, MDA role in the management loop and interaction between MDAS producer and network functions/entities including e.g., NWDAF and gNB. The TR also identified and documented a wide range of  use cases and corresponding potential requirements for management data analytics, as well as developing and evaluating the corresponding possible solutions.

As recommended by the study, the MDAS needs to be enhanced in normative work.

## 4	Objective

To describe:

- MDA service framework illustrating the service relations between MDAS producer, potential MDAS consumers and other network/management functions/services, including:

  - relation between MDAS producer and NWDAF, gNB and other 5G NFs and entities if necessary;

  - relation between MDAS producer with other management functions/services;

  - relation between cross domain MDAS producer and domain specific MDAS producer;

  - relation between MDAS producer/consumer and O-RAN/ONAP entities where relevant.

- MDA process and MDA role in the management loop (including Closed Loop SLS Assurance (COSLA)).

And, to specify:

- MDA input and report (output):

  - Common information elements of MDA report (output);

  - MDA input data (including new performance measurements, NRMs, MDT data, etc.) and MDA output attributes (as extension to the common attributes of MDA report) pertaining to the specific MDA use cases, based on the use cases studied, evaluated and concluded in TR 28.809 and the further prioritization to be addressed in this work item.

  - Mechanisms to address different use cases by a consolidated MDA report.

- Requirements and mechanisms on MDA report ask/subscription and reporting, with the generic file reporting MnS and generic streaming data reporting MnS taken into consideration;

- Requirements and mechanisms on retrieval of historical data related to MDA (including MDA input and output), by coordination with work item MADCOL (UID: 880028) and possibly other work items if relevant;

- Support for AI/ML for MDA, e.g., support for ML model training for MDA.

## 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |
| TS | 28.xyz | Management and orchestration; Management Data Analytics | Sep 2021 (SA#93) | Dec 2021 (SA#94) | Yizhi Yao, Intel, yizhi<dot>yao<at>intel<dot>com |

{Note 1: Only TSs may contain normative provisions. Study Items shall create or impact only TRs.
"Internal TR" is intended for 3GPP internal use only whereas "External TR" may be transposed by OPs.}

{Note 2: The first listed Rapporteur is the specification primary Rapporteur. Secondary Rapporteur(s) are possible for particular aspect(s) of the TS/TR. In this case, their responsibility has to be provided as "Remarks".}

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| 28.552 | Enhancement of 5G performance measurements for MDAS | Dec 2021 (SA#94) |  |
| 32.425 | Enhancement of performance measurements for ng-eNB for MDAS | Dec 2021 (SA#94) |  |
| 28.554 | Enhancement of 5G KPIs for MDAS | Dec 2021 (SA#94) |  |
| 28.541 | Enhancement of 5G NRMs for MDAS | Dec 2021 (SA#94) |  |
| 28.533 | Enhancement of 5G management architecture framework for MDAS | Dec 2021 (SA#94) |  |

## 6	Work item Rapporteur(s)

Yao, Yizhi, Intel (yizhi<dot>yao<at>intel<dot>com): primary rapporteur and rapporteur for the new TS;

Alkanani, Hassan, NEC (Hassan<dot>Alkanani<at>EMEA<dot>NEC<dot>COM): rapporteur for all CRs to affected existing TSs, e.g., CRs for enhancements of 5G management architecture framework, performance measurements, KPIs and NRMs.

## 7	Work item leadership

SA WG5.

## 8	Aspects that involve other WGs

## 9	Supporting Individual Members

Supporting IM name

Intel

NEC

China Unicom

KPN

Orange

China Telecom

China Mobile

CATT

ZTE

Samsung

Lenovo

Alibaba Group

HUAWEI?

Nokia?

AT&T

Pivotal Commware

P.I. Works

Deutsche Telekom