3GPP TSG SA WG5 Meeting 135-e		TDoc <S5-211134>

- electronic meeting, online, 25 January - 3 February 2021

- Source:	Huawei, Samsung

- Title:	Update the use case of traffic projection

- Document for:	Approval

Agenda Item:	6.5.4

# 1	Decision/action requested

This contribution is proposed to be approved.

# 2	References

[1]	3GPP TR 28.809: "Management and orchestration; Study on enhancement of Management Data Analytics (MDA)".

# 3	Rationale

This contribution proposes to update the required input data of network slice traffic projection analysis. The input data defined currently is not sufficient. The locations of the end users and the corresponding measurement reports including RSRP, RSRQ and SINR are also needed for the slice traffic projection analysis to help the MDAS producers to have a more detailed knowledge of the network traffic.

Besides, the evaluation of this use case is also provided.

# 4	Detailed proposal

It is proposed to apply the following changes in TR 28.809 [1].

1st change

REQ-TRA_CON_CON -3: The slice traffic analytics report providing traffic projection for the slice may include the following information:

-	Projected uplink and downlink throughput on each User Plane Function instance (UPF) present in the slice.

-	Projected number of Packet Data Unit (PDU) session for each Session Management Function (SMF) instance present in the slice.

-	Projected number of UE or Registered subscriptions for each AMF instance present in the slice.

-	Projected maximum packet size for each UPF instance present in the slice.

-	Projected UE uplink and downlink throughput on each gNodeB (gNB) instance present in the slice.

-	Projected number of UE for each gNB/NR cell instance present in the slice.

2nd change

##### 6.11.1.3.2 	Data required

The following table shows the data required to generate the traffic analytics report.

| Data Category | Required Data |
| --- | --- |
| Performance Measurements/KPIs |  |
|  | From each UPF instance in the slice - Current Uplink throughput. See 5.4.1.3 in TS 28.552. - Current Downlink throughput. See 5.4.1.4 TS 28.552 - Current Maximum packet size |
|  | From each gNB instance in the slice - Current Uplink UE throughput. See 5.1.1.3 in TS 28.552. - Current Downlink UE throughput. See 5.1.1.3 in TS 28.552. |
|  | From each SMF instance in the slice - Current Number of PDU session. See 5.2.1 in TS 28.552. |
|  | From each AMF instance in the slice - Current Number of registered subscriptions. See 5.2.1 in TS 28.552. |
| MDT Data | UE measurements related to RSRP, RSRQ, SINR and UE location information, see TS 37.320 [12]. |

3rd change

##### 6.11.1.3.3 	Analytics report

The slice traffic analytics report contains the following:

| Analytics Report of slice traffic projection | Attribute Name | Description |
| --- | --- | --- |
|  | Slice Identifier | Identifier of the slice for which the report is provided |
|  | Projection Timestamp | Provide a particular time stamp for which the projections are provided |
|  | Projection Duration | Provides a time duration during which the average projections are provided |
|  | For each UPF in the slice | Projected Uplink throughput |
|  |  | Projected Downlink throughput |
|  |  | Projected Maximum packet size |
|  | For each gNB in the slice | Projected Uplink UE throughput. |
|  |  | Projected Downlink UE throughput |
|  |  | Projected Number of UE |
|  | For each SMF in the slice | Projected Number of PDU session |
|  | For each AMF in the slice | Projected Number of registered subscriptions |

2nd change

#### 6.11.1.4	Evaluation

Based on the potential solutions, the required inputs are listed in clause 6.11.1.3.2, where:

-	The performance measurements and KPIs are defined in TS 28.552 [8] and TS 28.554 [7].

-	The MDT Data defined in TS 37.320 [12] can be acquired in procedures in TS 32.422 [25].

With these analytics inputs which are already defined, the analytics output as described in 6.11.1.3.3 can be derived

Therefore, MDA is feasible to support the analyses and predictions of traffic projection.

End of the changes