3GPP TSG SA WG5 Meeting 135-e		TDoc S5-211166rev1

- electronic meeting, online, 25 January - 3 February 2021

- Source:	Huawei

- Title:	Add use case rate control of the user data in control plane data transfer charging

- Document for:	Approval

Agenda Item:	7.5.2

# 1	Decision/action requested

In this box give a very clear / short /concise statement of what is wanted.

# 2	References

[1]	3GPP TR 28.816

# 3	Rationale

This contribution is to add use case rate control of the user data in control plane data transfer charging.

# 4	Detailed proposal

(For pseudo CR, include the complete clause(s) or subclause(s) of the latest draft TS/TR to be modified, with clear clause and sub-clause headings included and all modifications shown with revision marks, unambiguously showing where the changes shall be made or inserted in the draft TS/TR. It is not sufficient to just state, for example, “add the following text to the draft TS/TR…”.)

The first chagne

### 5.1.x3	Use case #x3: Support of Serving PLMN Rate Control in control plane data transfer domain charging

As described in TS 23.501 [3], the support of rate control of user data in 5GS CIoT is described as two 5GS CIoT features, which are Serving PLMN Rate control and Small Data Rate Control.

Serving PLMN Rate Control as a 5GS CIoT feature allows the serving PLMN to protect AMF and the Signalling Radio Bearers in the NG-RAN from the load generated by NAS Data PDUs. The use case begins during PDU session establishment and PDU session modification, SMF can indicate the support of PDU Session Serving PLMN Rate Control information to UE and UPF/NEF. That indicated information of Serving PLMN Rate Control is used for NAS Data PDUs.

During a PDU session, the UE can limit the rate to generate uplink NAS Data PDUs comply with the Serving PLMN policy, the UPF or NEF can limit the rate to generate downlink NAS Data PDU comply with the rate indicated for the PDU Session.

The indication information of Serving PLMN Rate Control, the change of Serving PLMN policy and enforcement of limit per PDU Session may be collected as charging information and CDRs regading the use of this 5GS CIoT feature.

In control plane data transfer domain charging, the cooresponding 5GC NFs those invole the Serving PLMN Rate control in a PDU connection to NEF may distinguish the indication information of Serving PLMN Rate Control and generate charging information during PDU session establishment and PDU session modification.

The potential charging requirements of control plane data transfer domain charging for this use case are: REQ-3GPPCH-5GSCIoT-z1

### 5.1.x3	Use case #t3: Support of Small Data Rate Control in control plane data transfer domain charging

As described in TS 23.501 [3], the support of rate control of user data in 5GS CIoT is described as two 5GS CIoT features, which are Servming PLMN Rate control and Small Data Rate Control.

Small Data Rate Control allows HPLMN operator to offer customer services in form of 'number of packets per time unit'. The SMF may determine the Small Data Rate Control based on operator policy, subscription, DNN, S-NSSAI, RAT type etc. The SMF can send a Small Data Uplink Rate Control commond to the UE and inform the UPF or NEF of Small Data Rate Control.

During a PDU session, the UE may comply with this uplink rate control instruction. If the UE exceeds the uplink 'number of packets per time unit', the UE may still send uplink exception reports if allowed. When the PDU Session is released, the AMF can stores Small Data Rate Control Status so that can be used for a subsequent establishment of a new PDU session. The SMF may receive the stored Small Data Rate Control Status during PDU session establishment, then provides the parameters to the UE and UPF/NEF.

The use of Small Data Rate Control and enforcement of limit per PDU Session may be collected as charging information and CDRs regading the use of this 5GS CIoT feature.

In control plane data transfer domain charging, the cooresponding 5GC NFs those invole the Serving PLMN Rate control in a PDU connection to NEF may distinguish information of Small Data Rate Control l and generate charging information.

The potential charging requirements of control plane data transfer domain charging for this use case are: REQ-3GPPCH-5GSCIoT-n1

Second change

## 5.2 	Potential charging requirements

REQ-3GPPCH-5GSCIoT-01	The charging aspect of 5G data connectivity domain charging shall support charging of Control Plane CIoT 5GS Optimisation.

REQ-3GPPCH-5GSCIoT-z1  the charging aspect of control plane data transfer domain should support collecting charging information related to Serving PLMN rate control for 5GS CIoT based on 5G system architecture.

REQ-3GPPCH-5GSCIoT-n1 The charging aspect of control plane data transfer should support collecting charging information of Small Data Rate Control for 5GS CIoT based on 5G system architecture.

End of change