3GPP TSG SA WG5 Meeting 135-e		TDoc S5-211282rev1

- electronic meeting, online, 25 January - 3 February 2021

- Source:	Ericsson

- Title:	pCR 28.811 Update to end to end network slice concept

- Document for:	Approval

Agenda Item:	6.5.5

# 1	Decision/action requested

The group is asked to discuss and agree on the proposal.

# 2	References

[1]	3GPP TR 28.811 v 0.2.0 : "Management and orchestration; Network Slice Management Enhancement"

# 3	Rationale

The existing text in section 4.3 introduces the notion of an “end-to-end network slice”, which is not necessary in 3GPP since a network slice always is “end to end”, i.e. across multiple technical domains (a.k.a network domains), e.g. RAN, Core, Transport. The objective in the SID for FS_NSMEN states “Investigate and propose the potential new management capabilities to support end to end network slicing” with reference to e.g. ETSI ZSM and GSMA 5GJA. In ZSM “end-to-end network slicing” is synonymous with management of network slices across multiple technical domains.

Section 4.3 seems to address the multi-operator scenario defined in section 4.1 but should then be rephrased as “End to end management of network slices” and defined as management of network slices across multiple administrative management domains.

The detailed proposed update of 4.3, is based on the text in SA5-206066rev5 (below, previously removed) but modified in accordance with the above rationale.

End to end network slicing is the set of management and orchestration activities that allow the deployment and operation of complete network slices. According to 28.530 [4], a network slice is complete in the sense that it includes all the network function instances, with their supporting resources, to provide a certain set of communication services to serve a certain business purpose.

# 4	Detailed proposal

Start of 1st Change

## 4.3	End to end management of network slice

Management of network slices is the set of management and orchestration activities that allow the deployment and operation of network slices across multiple administrative and network management domains. According to 28.530 [4], a network slice includes all the network function instances, with their supporting resources, to provide a certain set of communication services to serve a certain business purpose.

End of Change