3GPP TSG-SA5 Meeting #135-e 	S5-211442

- January 25 – Feburary 3, 2021, e-Meeting				       Revision of S5-211361

Title:	LS on the clarification of HO and/or reselection parameters ranges required by MLB

Response to:

Release:	3GPP Rel-17

Source:	3GPP SA5

To:	3GPP RAN3

Cc:

Contact Person:

#### Name:	Chou, Joey

E-mail Address:	Joey <dot> Chou <at> Intel <dot> com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1. Overall Description:

SA5 is working on the Rel-17 “Self-Organizing Networks (SON) for 5G networks” WI to support RAN3’s Rel-16 NR_SON_MDT WI “SON (Self-Organising Networks) and MDT (Minimization of Drive Tests) support for NR”.

SA5 notices that the Support for Mobility Load Balancing in TS 38.300 requires OAM to provide the ranges of HO and/or reselection parameters, as shown in the highlighted text below. However, no description is provided as to which parameters need configuration.

15.5.1.4	Adapting handover and/or reselection configuration

This function enables requesting of a change of handover and/or reselection parameters at target cell. The source cell that initialized the load balancing estimates if it is needed to change mobility configuration in the source and/or target cell. If the amendment is needed, the source cell initializes mobility negotiation procedure toward the target cell.

The source cell informs the target cell about the new mobility setting and provides cause for the change (e.g. load balancing related request). The proposed change is expressed by the means of the difference (delta) between the current and the new values of the handover trigger. The handover trigger is the cell specific offset that corresponds to the threshold at which a cell initialises the handover preparation procedure. Cell reselection configuration may be amended to reflect changes in the HO setting. The target cell responds to the information from the source cell. The allowed delta range for HO trigger parameter may be carried in the failure response message. The source cell should consider the responses before executing the planned change of its mobility setting.

All automatic changes on the HO and/or reselection parameters must be within the range allowed by OAM.

SA5 would like to request RAN3 to clarify which HO and/or reselection parameters SA5 should provide ranges to.

2. Actions:

To RAN3

ACTION: 	 SA5 would like to request RAN3 to clarify which HO and/or reselection parameters SA5 should provide ranges to.

3. Date of Next TSG-SA WG5 Meetings:

SA5#136-e	01 - 09 March 2021	e-meeting

SA5#137-e	10 - 19 May 2021	e-meeting