3GPP TSG-SA5 Meeting #135-e	S5-211052

# Online,  25th Jan 2021 - 3rd Feb 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0284 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-17 CR TS 28.552 Add PLMN granularity for number of active UEs measurements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecommunications |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | MANS |  |  |  |  |  | Date: |  |  | 2021-01-13 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The number of the active UEs for each mapped 5QI per PLMN ID and for each S-NSSAI per PLMN ID is a valuable measurement for operators to know how many DRBs are running with buffered data per cell, especially in multi-operator RAN sharing scenario. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding PLMN granularity for number of active UEs measurements. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Imcomplete granularity potentially affects the diagnosis of network problems. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.1.23, A.60 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st change

5.1.1.23	Number of Active Ues

5.1.1.23.1	Number of Active UEs in the DL per cell

a)	This measurement provides the mean number of active DRBs for UEs in an NRCellDU. The measurement is optionally split into subcounters per PLMN ID, and for each PLMN ID the measurement is optionally further split into subcounters per QoS level (mapped 5QI or/and QCI in NR option 3) and subcounters per S-NSSAI.

b)	DER (n=1).

c)	This measurement is defined according to measurement "Mean number of Active UEs in the DL per QoS level per cell" in TS 38.314 [29]. Separate counters are optionally maintained for each PLMN ID, and for each mapped 5QI (or/and QCI for option 3) per PLMN ID and for each S-NSSAI per PLMN ID.

d)	Each measurement is a single integer value.

e)	The measurement name has the form DRB.MeanActiveUeDl, 
DRB.MeanActiveUeDl.PLMN, where PLMN identifies the PLMN ID, and
DRB.MeanActiveUeDl.PLMN.QOS where QOS identifies the target quality of service class, and
DRB.MeanActiveUeDl.PLMN.SNSSAI, where SNSSAI identifies the S-NSSAI.

f)	NRCellDU.

g)	Valid for packet switched traffic.

h)	5GS.

i)	One usage of this measurement is for performance assurance within integrity area (user plane connection quality).

5.1.1.23.2	Max number of Active UEs in the DL per cell

a)	This measurement provides the max number of active DRBs for UEs in an NRCellDU.  The measurement is optionally split into subcounters per PLMN ID, and for each PLMN ID the measurement is optionally further split into subcounters per QoS level (mapped 5QI or/and QCI in NR option 3) and subcounters per S-NSSAI.

b)	DER (n=1).

c)	This measurement is defined according to measurement "Max number of Active UEs in the DL per QoS level per cell" in TS 38.314 [29]. Separate counters are optionally maintained for each PLMN ID, and for each mapped 5QI (or/and QCI for option 3) per PLMN ID and for each S-NSSAI per PLMN ID.

d)	Each measurement is a single integer value.

e)	The measurement name has the form DRB.MaxActiveUeDl, 
DRB.MaxActiveUeDl.PLMN, where PLMN identifies the PLMN ID, and
DRB.MaxActiveUeDl.PLMN.QOS where QOS identifies the target quality of service class, and
DRB.MaxActiveUeDl.PLMN.SNSSAI, where SNSSAI identifies the S-NSSAI.

f)	NRCellDU.

g)	Valid for packet switched traffic.

h)	5GS.

i)	One usage of this measurement is for performance assurance within integrity area (user plane connection quality).

5.1.1.23.3	Number of Active UEs in the UL per cell

a)	This measurement provides the mean number of active DRBs for UEs in an NRCellDU.  The measurement is optionally split into subcounters per PLMN ID, and for each PLMN ID the measurement is optionally further split into subcounters per QoS level (mapped 5QI or/and QCI in NR option 3) and subcounters per S-NSSAI.

b)	DER (n=1)

c)	This measurement is defined according to measurement "Mean number of Active UEs in the UL per QoS level per cell" in TS 38.314 [29]. Separate counters are optionally maintained for each PLMN ID, and for each mapped 5QI (or/and QCI for option 3) per PLMN ID and for each S-NSSAI per PLMN ID.

d)	Each measurement is a single integer value.

e)	The measurement name has the form DRB.MeanActiveUeUl, 
DRB.MeanActiveUeUl.PLMN, where PLMN identifies the PLMN ID, and
DRB.MeanActiveUeUl.PLMN.QOS where QOS identifies the target quality of service class, and
DRB.MeanActiveUeUl.PLMN.SNSSAI, where SNSSAI identifies the S-NSSAI.

f)	NRCellDU.

g)	Valid for packet switched traffic.

h)	5GS.

i)	One usage of this measurement is for performance assurance within integrity area (user plane connection quality).

5.1.1.23.4	Max number of Active UEs in the UL per cell

a)	This measurement provides the max number of active DRBs for UEs in an NRCellDU.  The measurement is optionally split into subcounters per PLMN ID, and for each PLMN ID the measurement is optionally further split into subcounters per QoS level (mapped 5QI or/and QCI in NR option 3) and subcounters per S-NSSAI.

b)	DER (n=1)

c)	This measurement is defined according to in RAN specification [x], measurement "Max number of Active UEs in the UL per QoS level per cell" in TS 38.314 [29]. Separate counters are optionally maintained for each PLMN ID, and for each mapped 5QI (or/and QCI for option 3) per PLMN ID and for each S-NSSAI per PLMN ID.

d)	Each measurement is a single integer value.

e)	The measurement name has the form DRB.MaxActiveUeUl, 
DRB.MaxActiveUeUl.PLMN, where PLMN identifies the PLMN ID, and
DRB.MaxActiveUeUl.PLMN.QOS where QOS identifies the target quality of service class, and
DRB.MaxActiveUeUl.PLMN.SNSSAI, where SNSSAI identifies the S-NSSAI.

f)	NRCellDU.

g)	Valid for packet switched traffic.

h)	5GS.

i)	One usage of this measurement is for performance assurance within integrity area (user plane connection quality).

Next change

A.60	Monitoring of the number of active UEs in NG-RAN

The number of the active UEs per direction in each cell is a valuable measurement for operators to know how many DRBs are running with buffered data per cell and QoS or S-NSSAI basis. For multi-operator RAN sharing scenario, PLMN basis is needed, too. This kind of information can help operators to tune the admission control parameters for the cell and to estimate load in neighbour cells, to ensure that the UEs admitted achieve the target QoS and that capacity is not over-estimated when distributing load between cells and gNBs.

End of changes