3GPP TSG-SA5 Meeting #136-e	S5-212068

# Online, , 1st Mar 2021 - 9th Mar 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.554 | CR | 0075 | rev | - | Current version: | 17.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-17 CR 28.554 Update the Accessibility KPI to cover RRC Resume |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson LM |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | ePM_KPI_5G |  |  |  |  |  | Date: |  |  | 2021-02-19 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The current definition of the KPI DRB Accessibility for UE services, in chapter 6.2.4 of TS 28.554, is not covering the accesses that are made using the RRC Resume procedure and the accesses made using the procedures of fallback to RRC Setup from RRC Resume or RRC Re-establishment procedures. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The existing KPI named “DRB Accessibility for UE services” (in 6.2.4) has been updated to include accesses that are made using the RRC Resume procedure and the accesses made using the procedures of fallback to RRC Setup from RRC Resume or RRC Re-establishment procedures.The updated KPI is a weighted sum of the different types of accesses. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The existing KPI named “DRB Accessibility for UE services” will not cover all accesses. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.2, 6.2.4, A.9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  | x |  | O&M Specifications |  |  |  | TS 28.552 CR 0291 |  |  |
| Other comments: |  | The proposed KPI update was Endorsed in S5-211093 in SA5#135e. The updated KPI relates to new measurements in S5-212067 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st modified section

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

EE	Energy Efficiency

kbit	kilobit (1000 bits)

RTT	Round Trip Time

RNA	RAN Notification Area

Next modified section

### 6.2.4	DRB Accessibility for UE services

a)	DRB Accessibility

b)	This KPI describes the DRBs setup success rate, including the success rate for setting up RRC connection and NG signalling connection. It is obtained as as the weigthed sum of success rate for the three different accessibility types: RRC resume, Initial DRB setup and Added DRB setup. Initial DRB setup success rate is obtained as the success rate for RRC connection setup multiplied by the success rate for NG signalling connection setup multiplied by the success rate for DRB setup. The success rate for RRC connection setup and for NG signalling connection setup shall exclude setups with establishment cause mo-Signalling [5]. The success rate for RRC resume shall exclude setups related to RNA update. It is a percentage. The KPI type is RATIO.

c)	DRBAccessibility  5QI = 100 * W_Resume * (RRC Resume success rate / 100) + W_Idle * Initial DRB setup success rate_5QI / 100) + W_Added * (Added DRB setup success rate_5QI / 100)

DRBAccessibility  SNSSAI = 100 * W_Resume * (RRC Resume success rate / 100) + W_Idle * Initial DRB  setup success rate_SNSSAI / 100) + W_Added * (Added DRB setup success rate_SNSSAI / 100)

Where:

RRC Resume success rate = 100* ∑RRC.ResumeSucc.cause /∑(RRC.ResumeAtt.cause - RRC.ResumeFallbackToSetupAtt.cause), where all but the causes related to RNA update shall be included.

Initial DRB setup success rate_5QI = 100* (∑(RRC.ConnEstabSucc.Cause + RRC.ResumeSuccByFallback.cause) + RRC.ReEstabSuccWithoutUeContext) /(∑(RRC.ConnEstabAtt.Cause + RRC.ResumeFallbackToSetupAtt.cause) + RRC.ReEstabFallbackToSetupAtt) * (∑UECNTXT.ConnEstabSucc.Cause/∑ UECNTXT.ConnEstabAtt.Cause)  * (DRB.EstabSucc.5QI - DRB.AddedSucc.5QI) /(DRB.EstabAtt.5QI - DRB.AddedAtt.5QI

Initial DRB setup success rate_SNSSAI = 100* (∑(RRC.ConnEstabSucc.Cause + RRC.ResumeSuccByFallback.cause) + RRC.ReEstabSuccWithoutUeContext) /(∑(RRC.ConnEstabAtt.Cause + RRC.ResumeFallbackToSetupAtt.cause) + RRC.ReEstabFallbackToSetupAtt) * (∑UECNTXT.ConnEstabSucc.Cause/∑ UECNTXT.ConnEstabAtt.Cause)  * (DRB.EstabSucc.SNSSAI - DRB.AddedSucc.SNSSAI) /(DRB.EstabAtt.SNSSAI - DRB.AddedAtt.SNSSAI)

Added DRB setup success rate_5QI = 100* DRB.AddedSucc.5QI / DRB.AddedAtt.5QI

Added DRB setup success rate_SNSSAI = 100* DRB.AddedSucc.SNSSAI / DRB.AddedAtt.SNSSAI

W_Resume = ∑RRC.ResumeAtt.cause / (∑RRC.ResumeAtt.cause  + ∑(RRC.ConnEstabAtt.Cause + RRC.ResumeFallbackToSetupAtt.cause) + RRC.ReEstabFallbackToSetupAtt + ∑DRB.AddedAtt.5QI)

W_Idle = (∑(RRC.ConnEstabAtt.Cause + RRC.ResumeFallbackToSetupAtt.cause) + RRC.ReEstabFallbackToSetupAtt) / (∑RRC.ResumeAtt.cause  + ∑(RRC.ConnEstabAtt.Cause + RRC.ResumeFallbackToSetupAtt.cause) + RRC.ReEstabFallbackToSetupAtt + ∑DRB.AddedAtt.5QI)

W_Added = ∑DRB.AddedAtt.5QI / (∑RRC.ResumeAtt.cause  + ∑(RRC.ConnEstabAtt.Cause + RRC.ResumeFallbackToSetupAtt.cause) + RRC.ReEstabFallbackToSetupAtt + ∑DRB.AddedAtt.5QI)

The sum over causes shall exclude the establishment cause mo-Signalling [5].

The sum over causes for RRC resume shall exclude the causes related to RNA update [5].

For KPI on SubNetwork level the measurement shall be the averaged over all NRCellCUs in the SubNetwork

d)	SubNetwork, NRCellCU.

Next modified section

# A.9	Use case for DRB accessibility-related KPI

In providing services to end-users, the first step is to get access to the service. First after access to the service has been performed, the service can be used.

The service provided by NG-RAN is the DRB. For the DRB to be successfully setup it is also necessary to setup an RRC connection and an NG signalling connection.

If an end user cannot access a service, it is hard to charge for the service. Also, if it happens often that an end-user cannot access the provided service, the end-user might change wireless subscription provider, i.e. loss of income for the network operator. Hence, to have a good accessibility of the services is important from a business point of view.

A DRB accessibility KPI requires the following 5 measurements:

-	RRC connection setup success rate

- 	RRC resume success rate

-	NG signalling connection setup success rate.

- 	DRB addition success rate

The success rate for RRC connection setup and for NG signalling connection setup shall exclude setups with establishment cause mo-Signalling, since these phases/procedures occur when there is no request to setup a DRB.

The success rate for RRC resume shall exclude the cases related to RNA update, since these procedures occur when there is no request to setup a DRB.

This KPI is available per mapped 5QI and per S-NSSAI, and it assists the network operator with information about the accessibility provided to their 5G network customers.

End of modified section