3GPP TSG SA WG5 Meeting 135-e		Tdoc S5-211301rev4

# electronic meeting, online, 25 January - 3 February 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.537 | CR | Draft CR | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-17 CR 28.537 Add management data management requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | MADCOL |  |  |  |  |  | Date: |  |  | 2021-01-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | 17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Management data management requirements are missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Management data management requirements and procedures are added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Management data management requirements are missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | X (New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Input to DraftCR to 28.537 related to the WI MADCOL |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First modification

# X	Management data management

## X.1	Overview

Management data of different kinds are collected from telecommunication networks by observing discrete events or analogue quantities. This data serves as input to different functions such as optimisation functions or functions processing the raw input data to produce higher level abstractions and statistics or predictions. Management data is also used for training Machine Learning (ML) models.

[…]

Many consumers can ask network functions to produce management data. There is no guarantee that a network function is not requested to produce the very same data at a given point in time multiple times (for example measurement X on BTS Y). Some requests for data may also be satisfied from already existing data production processes. For example, when an analogue quantity is already sampled with a granularity of 1s, a request to sample the same quantity with a granularity of 5s does not need to be forwarded to the network function, but can be handled at management level by sending to the requestor every 5th element of the data stream received from the network function. In other words, it is beneficial to coordinate data requests at the management level to offload network functions from management tasks and let them focus their capacity on their primary tasks. These considerations lead to REQ-MDM-FUN-2.

AI/ML models need input data collected over a certain period of time for training purposes. A specific set of collected data may serve different purposes and can therefore be input to multiple AI/ML functions. For example, data collected from an area with a train station may be used also for another area with a train station, because the scenarios are statistically similar. This reduces not only the amount of data to be produced by the network functions but enables to address areas with AI/ML for which no data is available or for which no data can be produced. Another use case for storing produced data is related to the fact that multiple sets of traning data from similar scenarios are typically required. For example, one set of data produced for the rush hour in a subway station on a single weekday is typically not enough for profiling. Many sets produced on many workdays are required. This calls for storing produced data (REQ-MDM-FUN-3).

Stored data is only useful when functions can discover which data has been produced to check if the needed data is already available or has to be produced. This requires a description of the stored data and means allowing data consumers to discover data that was stored and is available for retrieval (REQ-MDM-FUN-4, REQ-MDM-FUN-5).

[…]

## X.2	Specification level requirements

[…]

REQ-MDM-FUN-2: The 3GPP management system shall coordinate requests from data consumers to produce management data in order to avoid the same data is requested multiple times from a data producer by different data consumers.

Editor's note: It is tbc what exactly is "same data".

REQ-MDM-FUN-3: The 3GPP management system shall support storing produced management data.

REQ-MDM-FUN-4: The 3GPP management system shall support a capability enabling a consumer to discover stored management data.

REQ-MDM-FUN-5: The 3GPP management system shall support a capability enabling a consumer to retrieve stored management data.

[…]

## X.3	Procedures

Editor's note: To be added

End of modifications