3GPP TSG-SA5 Meeting #136-e	S5-212107

# Online, 1st – 9th March 2021

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.535 | CR | 00xx | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-16 CR 28.535 Clean up the business level use cases and requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | eCOSLA |  |  |  |  |  | Date: |  |  | 2021-02-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Some of the existing use cases introduce inaccurate requirements not related to closed loop assurance and inaccurate descriptions. The use case on obtaining resource requirements for a communication service does not suit the scope of eCOSLA. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Delete REQ-CSA_CSA-FUN-01, delete use case example in the description in 5.1.2, delete REQ-CSIA_CON-02, delete section 5.1.3 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Stage 1 of eCOSLA would not be aligned with Stage 2 and Stage 3 of eCOSLA |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 4.2.1, 4.2.2, 4.3.X (new), 4.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  | x |  | O&M Specifications |  |  |  | TS 28.623 CR … |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Modified Section

# 5	Business level use cases and requirements

## 5.1	Use cases

### 5.1.1	Communication service assurance

The CSP needs to meet the CSC expectations on automation as well as internal goals on CAPEX and OPEX efficiency.

The CSP has access to capabilities, procedures and tools that can address both CAPEX and OPEX in the provisioning and management of communication services to their customers (CSC). The CSC expects the CSP to offer a variety of communication services including business critical communication services that allow the CSC (e.g. Enterprise) to run their applications in a predictable manner [2]. Hence automation of the on-boarding of the CSC application, which will use communication services provided by the CSP, on a 5GS, is a requirement to meet the following needs:

-	reduce the complexity for a CSC application to be on-boarded on a 5GS;

- 	assure the tsrget goals for a CSC;

-	reduce the cost ownership through automation.

During the operation of the communication service the CSP provides assurance of service quality expectation and CSP meets the CSC expectations on automation as well as internal goals on CAPEX and OPEX efficiency.

REQ-CSA_CSA-FUN-02 The 5GS shall have the capabilities to monitor, and report to CSP the fulfilment of committed CS requirements and actions taken to adjust for deviations.

REQ-CSA_CSA-FUN-03 The 5GS shall have the capabilities to monitor and report to CSP actions taken to adjust deviations on committed CS requirements.

REQ-CSA_CSA-FUN-04 The 5GS shall have the capability to provide in-operation assurance of service quality expectation.

REQ-CSA_CSA-FUN-05 The 5GS shall have the capability to ensure the service quality requirements during the service operations.

### 5.1.2	Communication service assurance for shared resources

In this scenario, it is assumed that the SLA's for the two communication services will allow them to share resources, for example RAN resources.

The management systems CS-Assurance service receives the request from Order Care and using a MDAS CS preparation assistance service, explores and evaluates communication service realisation and impact on other communication services, if any.

Once the Order Care has committed to an SLA with a CSC, the management system activates the communication service.

As the communication service operates, a management service for communication service assurance, CSA, continuously monitors the SLA fulfilment using MDAS, PM assurance services [3] including and core network NWDAF QoE analytics service, if available.

Based on goals for SLA fulfilment, or other KPIs, the CSA service may initiate an action when SLA goals are not met, be that over- or under fulfilment. The CSA service may use an MDAS to assist in selecting proper action and how to best execute the action.

The CSA service triggers the action by using provisioning service [4] towards RAN, transport and core network and monitors the effect of the change.

REQ-CSIA_CON-01: The 3GPP management system shall have the capability providing a management service for assisting in assessing (evaluating) a target SLA for a requested communication service.

REQ-CSIA_CON-02: The 3GPP management system shall have the capability providing a management service for assisting in asserting an agreed SLA for a requested communication service.

### 5.1.4	Use case for interaction with core network for service assurance

The goal is to enable the 3GPP management system to take early action to prevent service degradation.

The 3GPP management system configures the control plane functions (e.g. NWDAF) so as to report potential service degradation according to the SLS. Service load can be determined by considering both NF(s) load in 5GC and resource utilization in access network. If the service degradation occurs or predicted when the resources are scaled down, resources could be scaled up to solve the issue. Therefore, it is necessary for the 3GPP management system to configure the 5GC functions such that in the event that a potential service degradation or overloading is predicted, that is sent to the 3GPP management system. This can be done by properly configuring the overloading conditions (e.g. triggering parameters) in the 5GC functions of a selected service. The 3GPP management system could configure the 5GC functions to trigger when the service load is increased or predicted to be increased beyond a certain threshold level. The 3GPP management system could then do resource scaling or use MDAS to find a proper solution.

Similarly, when the resources are underutilized the 3GPP management system could do scaling down or deactivation of resources.

REQ-CSA_RR-CON-01 The 3GPP management system shall be able to configure the 5GC functions to make them report of a potential service load increase beyond a certain threshold so that the 3GPP management system can do scaling up of resources in time without impacting the SLA.

REQ-CSA_RR-CON-02 The 3GPP management system shall be able to determine the service load thresholds that need to be used by the 5GC functions to report, so that a potential resource overprovisioning situation can be ascertained.

REQ-CSA_RR-CON-03 The 3GPP management system shall be able to perform scaling down of resources when a resource overprovisioning is detected, and the overprovisioning is not needed.

End of modifications