3GPP TSG-SA5 Meeting #136-e	S5-212109

# Online, 1st – 9th March 2021

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.535 | CR | 00xx | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-16 CR 28.535 Clean up the specification level use cases and requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | eCOSLA |  |  |  |  |  | Date: |  |  | 2021-02-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | NWDAF use case is already included in the core network use case and some of the technical requirements are not specific for the closed loop assurance. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Delete section 6.1.2 and delete requirements REQ-CSA-CON-02, REQ-CSA-CON-03, REQ-CSA-CON-04, REQ-CSA-CON-05, REQ-CSA-CON-06, REQ-CSA-CON-08, REQ-CSA-CON-10, REQ-CSA-CON-11 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Requirement introduced are not in the scope of the closed cloop assurance and not aligned with the sage 2 and stage 3 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 4.2.1, 4.2.2, 4.3.X (new), 4.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  | x |  | O&M Specifications |  |  |  | TS 28.536 CR … |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Modified Section

# 6	Technical Specification level use cases and requirements

## 6.1	Use cases

### 6.1.1	Communication service quality assurance and optimization of communication services

The goal of the use case is to enable communication service quality assurance and optimization for the set of services provided by the network to certain group (category) of UEs. For example, the set can include the communication services provided via certain NSI(s) or to IoT devices in a certain area.

The group of NG-RAN and 5GC nodes (deployed and active), which are essential for the set of E2E services, provide provisioning and PM management services. It is assumed that the providers of the related NSI / NSSI provisioning and PM management services are deployed and active.

The management system is consuming aforementioned management services either directly or through proxy nodes that re-expose the management services; the management system is aware of the performance requirements imposed on the set of communication services.

The management system is collecting the service experience information and monitoring the key performance indicators, KPIs, related to the targeted services. Analytics hosted by the MDAF may be utilized for processing of the network data to derive and analyse the KPIs. If the service quality assurance and optimization function detects performance degradation the 3GPP management system may continuously modify the configuration parameters in the corresponding NG-RAN and 5GC nodes and NSI(s)/NSSI(s), to satisfy the SLA requirement(s). In case that changes of communication service SLA/SLS are needed, those changes may result as input to the 3GPP management system.

If the network performance does not recover or improve, the management system may further adjust the network configuration or roll back to the previous configuration.

### 6.1.3	5G Core assisted SLS communication service Assurance

The goal of this use case is to describe 5G Core management to assure compliance to SLSs (Service Level Specifications) for a communication service in 3GPP management system.

3GPP management system receives the SLS requirements that required by CSP or NOP. 3GPP management system is capable to translate e2e SLS goal and set the 5GC goal(s) of SLS related to 5GC and activate a closed control loop for service assurance goal(s). To fulfill the SLS requirements, 3GPP management system is capable to configure the management resource and 5GC network functions (e.g. AMF, SMF, NWDAF) to monitor measurements and fault alarms that are relevant to the SLS. Since, for example, a network slice for eMBB can provide multiple communications services, one or multiple closed control loops for service assurance goals are set, and the network resource and performance measurements which are relevant to the SLS.

During the process of service assurance of 5GC, the 5GC domain MDAS provider can be used to provide analysis of 5GC related network resources, virtual resources and performance assurance related to SLS in 5GC. The 5GC domain analysis report may be provided to 3GPP management system as part of the analysis result(s) of 5GC SLS.

Together with the report from NWDAF, performance measurements and fault alarms related to 5GC NFs are also available for analysis of any potential service degradation.

### 6.1.4	Communication service SLS assurance control

The goal of this use case is to enable the MnS consumer to control the communication service SLS assurance closed control loop(s) (e.g. specify the SLS to be assured, enable/disable the SLS assurance, specify the assurance time for certain SLS) and obtain the SLS fulfil information provided by MnS producer. It is assumed that the MnS producer maintains SLS assurance closed control loops for multiple SLSs. The detailed SLSs for network slice assurance are captured in ServiceProfile (e.g. latency, Throughput) associated to network slice and the detailed SLS for network slice subnet assurance are captured in SliceProfile (e.g. latency, Throughput) associated to network slice subnet.

When an MnS producer receives an SLS assurance closed control loops(s) creation request with SLS assurance requirements for certain managed Entity (i.e. network slice, network slice subnet) from an MnS consumer, the SLS assurance requirements may include information of which SLS should be assured (e.g. latency should be assured), the SLS assurance granularity (e.g. per UE, per Network Slice, per S-NSSAI), SLS assurance condition (e.g. SLS assurance duration time, SLS assurance fulfilment requirements (e.g. the ratio of the SLS assurance time during the whole service usage time) ), the MnS producer create SLS closed control loop managed object instance contained by the specified managed Entity (i.e. NetworkSlice, NetworkSliceSubnet) and configures the received SLS assurance requirements in the created SLS closed control loop managed object instances. The MnS producer performs the network and/or service management to satisfy the SLS assurance requirements by adjusting the network (e.g. adjust the network topology, configure RRM policy) to satisfy the required SLS assurance requirements.

During the SLS assurance closed control loop operation phase, the MnS consumer may request MnS producer to enable/disable the corresponding SLS assurance or update the SLS assurance requirements if needed, then MnS producer update corresponding the SLS assurance closed control loop managed object instance to ensure the MnS producer perform the SLS assurance closed control loop based on the new request.

During the SLS assurance closed control loop operation phase, the MnS producer may report the SLS assurance closed control loop progress information and fulfilment information (e.g. SLS assurance requirements is satisfied or not) to the MnS consumer.

## 6.2	Requirements

REQ-CSA-CON-01 The 3GPP management system shall have the capability to take actions for a set of communication services serving certain group of UEs based on the target SLS.

REQ-CSA-CON-07	The 3GPP management system shall have the capability to ascertain SLS breach.

REQ-CSA-CON-09	The 3GPP management system shall have the capability to take corrective actions to assure the target goal.

REQ-CSA-CON-12	The 3GPP management system shall have the capability to allow its authorized consumer to control the SLS assurance (e.g. specify the SLS to be assured, enable/disable, specify the assurance time and update the SLS assurance requirements).

REQ-CSA-CON-13	The 3GPP management system shall have the capability to allow its authorized consumer to obtain the SLS assurance progress information and fulfil information.

NOTE 2:	The management system refers to the producer of management service for SLS assurance.

End of modifications