3GPP TSG-SA5 Meeting #136-e	S5-212276

# Online, , 1st Mar 2021 - 9th Mar 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.532 | CR | 0170 | rev | - | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-16 CR 28.532 Correct some minor errors in the Fault MnS definition (REST SS) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16 |  |  |  |  |  | Date: |  |  | 2021-02-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | There are some smaller isssues in the Fault MnS REST SS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct and clarify some smaller isssues in the Fault MnS REST SS, note there is no impact on the OpenAPI definitions. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The REST SS might be ambiguous, and some data type names not aligned to the nmaes in the OpenAPI definition. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 12.2.1.1.2, 12.2.1.1.3, 12.2. 12.2.1.2.1, 1.1.4, 12.2.1.1.5, 12.2.1.1.6, 12.2.1.1.7, 12.2.1.1.8, 12.2.1.1.9, 12.2.1.2.4, 12.2.1.2.5, 12.2.1.2.7, 12.2.1.2.8, 12.2.1.2.10, 12.2.1.2.11, 12.2.1.3.2.2.3.1, 12.2.1.3.2.2.3.1, 12.2.1.3.2.4.3.1, 12.2.1.3.2.6.3.1, 12.2.1.3.2.7.3.1, 12.2.1.3.2.8.3.1, 12.2.1.4.4.3, 12.2.1.4.4.4, 12.2.1.4.4.5, 12.2.1.4.4.6, 12.2.1.4.4.7, 12.2.1.4.4.8, 12.2.1.4.4.9, 12.2.1.4.4.10 12.2.1.4.1a.22 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First modification

## 12.2	Generic fault supervision management service

### 12.2.1	RESTful HTTP-based solution set

#### 12.2.1.1	Mapping of operations

##### 12.2.1.1.1	Introduction

The IS operations are mapped to SS equivalents according to table 12.2.1.1.1-1.

Table 12.2.1.1.1-1: Mapping of IS operations to SS equivalents

| IS operation | HTTP Method | Resource URI | S |
| --- | --- | --- | --- |
| getAlarmList | GET | /alarms | M |
| getAlarmCount | GET | /alarms/alarmCount | O |
| acknowledgeAlarms | PATCH | /alarms | M |
|  | PATCH | /alarms/{alarmId} | M |
| unacknowledgeAlarms | PATCH | /alarms | M |
|  | PATCH | /alarms/{alarmId} | M |
| clearAlarms | PATCH | /alarms | M |
|  | PATCH | /alarms/{alarmId} | M |
| setComment | POST | /alarms/{alarmId}/comment | O |
| subscribe | POST | /subscriptions | M |
| unsubscribe | DELETE | /subscriptions/{subscriptionId} | M |

##### 12.2.1.1.2	Operation getAlarmList

The IS operation parameters are mapped to SS equivalents according to table 12.2.1.1.2-1 and table 12.2.1.1.2-2.

Table 12.2.1.1.2-1: Mapping of IS operation input parameters to SS equivalents (HTTP GET)

| ISparameter name | SS parameter location | SS parameter name | SS parameter type | S |
| --- | --- | --- | --- | --- |
| alarmAckState | query | alarmAckState | AlarmAckState- | O |
| baseObjectClass baseObjectInstance | query | baseObjectInstance | Dn | O |
| filter | query | filter | Filter | O |

Table 12.2.1.1.2-2: Mapping of IS operation output parameters to SS equivalents (HTTP GET)

| IS parameter name | SS parameter location |  | SS parameter name | SS parameter type | S |  |
| --- | --- | --- | --- | --- | --- | --- |
| alarmInformationList | response body |  | n/a | map(NotificationHeader, AlarmRecord, comments(map(Comment))) |  | M |
| status |  | response status codes | n/a | n/a | M |  |
|  |  | response body | error | ErrorResponse | O |  |

The message flow is as follows:

1. The MnS consumer sends a HTTP GET request to the MnS producer.

- The URI identifies the "…/alarms" collection resource.

- The querycomponent may contain three optional parameters: "alarmAckstate", "baseObjectInstance" and "filter". Absence of the query component means all alarms shall be returned.

- The request message body shall be empty.

2. The MnS producer sends a HTTP GET response to the MnS consumer.

- On success "200 OK" shall be returned. The response message body shall contain the queried alarm records, the notification headers of the last "notifyNewAlarm", "notifyChangedAlarm" or "notifyClearedAlarm" notifications, which were sent related to the alarms, and the comments associated to the alarms.

- On failure, an appropriate error code shall be returned. The response message body may carry additional error information.

##### 12.2.1.1.3	Operation getAlarmCount

The IS operation parameters are mapped to SS equivalents according to table 12.2.1.1.3-1 and table 12.2.1.1.3-2.

Table 12.2.1.1.3-1: Mapping of IS operation input parameters to SS equivalents (HTTP GET)

| IS parameter name | SS parameter location | SS parameter name | SS parameter type | S |
| --- | --- | --- | --- | --- |
| alarmAckState | query | alarmAckState | AlarmAckState- | O |
| filter | query | filter | string | O |

Table 12.2.1.1.3-2: Mapping of IS operation output parameters to SS equivalents (HTTP GET)

| IS parameter name | SS parameter location | SS parameter name |  | SS parameter type |  | S |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| criticalCount, majorCount, minorCount, warningCount, indeterminateCount, clearedCount | response body | n/a |  | AlarmCount |  | M |  |
| status | response status codes |  | n/a |  | n/a |  | M |
|  | response body |  | error |  | ErrorResponse |  | O |

The message flow is as follows:

1. The MnS consumer sends a HTTP GET request to the MnS producer.

- The URI identifies the "…/alarms/alarmsCount" collection resource.

- The query component may contain two optional parameters: "alarmAckstate" and "filter". Absence of the query component means all alarms shall be counted.

- The request message body shall be empty.

2. The MnS producer sends a HTTP GET response to the MnS consumer.

- On success "200 OK" shall be returned. The response message body shall carry the alarm count for all perceived severity values. The response format is defined by "AlarmsCount".

- On failure, an appropriate error code shall be returned. The response message body may carry additional error information.

##### 12.2.1.1.4	Operation setComment

In case a comment shall be added to a single alarm the IS operation parameters are mapped to SS equivalents according to table 12.2.1.1.4-1 and table 12.2.1.1.4-2.

Table 12.2.1.1.4-1: Mapping of IS operation input parameters to SS equivalents (HTTP POST)

| IS parameter name | SS parameter location | SS parameter name | SS parameter type | S |
| --- | --- | --- | --- | --- |
| alarmInformationReferenceList | path | /alarms/{alarmId}/comment | string | M |
| commentUserId | request body | commentUserId | commentUserId | M |
| commentSystemId | request body | commentSystemId | commentSystemId | O |
| commentText | request body | commentText | commentText | M |

Table 12.2.1.1.4-2: Mapping of IS operation output parameters to SS equivalents (HTTP POST)

| IS parameter name |  | SS parameter location |  | SS parameter name |  | SS parameter type |  | S |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| badAlarmInformationReferenceList |  | response body |  | n/a |  | ErrorResponse |  | M |
| status |  | response status codes |  | n/a |  | n/a |  | M |  |
|  |  | response body |  | error |  | ErrorResponse |  | O |  |

The message flow for adding a comment to a single alarm is as follows:

1. The MnS consumer sends a HTTP POST request to the MnS producer.

- The URI identifies the "…/alarms/{alarmId}/comment" alarm resource the comment shall be added to.

- The query component shall be absent.

- The request message body shall contain a JSON object with "commentUserId" and "commentText" properties. In addition to that the request object may contain the "commentSystemId" property. .

2. The MnS producer sends a HTTP POST response to the MnS consumer.

- On success "201 Created " shall be returned. The response message body shall carry the representation of the created comment resource. The Location header shall be present and carry the URI of the created comment resource.

- On failure, an appropriate error code shall be returned. The response message body may carry additional error information.

The stage 3 solution does not support adding a comment to multiple alarms.

##### 12.2.1.1.5	Operation acknowledgeAlarms

In case a single alarm shall be acknowledged the IS operation parameters are mapped to SS equivalents according to table 12.2.1.1.5-1 and table 12.2.1.1.5-2.

Table 12.2.1.1.5-1: Mapping of IS operation input parameters to SS equivalents (HTTP PATCH)

| IS parameter name | SS parameter location | SS parameter name | SS parameter type | S |
| --- | --- | --- | --- | --- |
| alarmInformationAndSeverityReferenceList | path | /{alarmId} | string | M |
| ackUserId | request body | ackUserId | AckUserId | M |
| ackSystemId | request body | ackSystemId | AckSystemId | O |

The perceived severity is not mapped in the present documet.

Table 12.2.1.1.5-2: Mapping of IS operation output parameters to SS equivalents (HTTP PATCH)

| IS parameter name |  | SS parameter location |  | SS parameter name |  | SS parameter type | S |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| badAlarmInformationReferenceList |  | response body |  | n/a |  | errorResponse | M |  |
| status |  | response status codes |  | n/a |  | n/a |  |  | M |
|  |  | response body |  | error |  | ErrorResponse |  |  | O |

The message flow for acknowledging a single alarm is as follows:

1. The MnS consumer sends a HTTP PATCH request to the MnS producer.

- The URI identifies the "…/alarms/{alarmId}" alarm resource to be acknowledged.

- The query component is absent..

- The request message body contains a merge patch document. The document shall patch the "ackState" and "ackUserId" property of the identified alarm resource, and may patch the "ackSystemId" property. The patch document is defined by "MergePatchAcknowledgeAlarms".

2. The MnS producer sends a HTTP PATCH response to the MnS consumer.

- On success "204 No Content" shall be returned. The response message body shall be empty.

- On failure, an appropriate error code shall be returned. The response message body shall return the alarmId, together with failure reason. The response message body may carry additional error information.

In case multiple alarms shall be acknowledged the IS operation parameters are mapped to SS equivalents according to table 12.2.1.1.5-3 and table 12.2.1.1.5-4.

Table 12.2.1.1.5-3: Mapping of IS operation input parameters to SS equivalents (HTTP PATCH)

| IS parameter name | SS parameter location | SS parameter name | SS parameter type | S |
| --- | --- | --- | --- | --- |
| alarmInformationAndSeverity ReferenceList | path request body | /alarms alarmId (key in map) | n/a string | M M |
| ackUserId | request body | ackUserId | ackUserIdType | M |
| ackSystemId | request body | ackSystemId | ackSystemIdType | O |

The perceived severity is not mapped in the present document.

Table 12.2.1.1.5-4: Mapping of IS operation output parameters to SS equivalents (HTTP PATCH)

| IS parameter name | SS parameter location | SS parameter name | SS parameter type | S |
| --- | --- | --- | --- | --- |
| badAlarmInformationReferenceList | response body | n/a | array(failedAlarm) | M |
| status | response status codes | n/a | n/a | M |

The message flow for acknowledging multiple alarms is as follows:

1. The MnS consumer sends a HTTP PATCH request to the MnS producer.

- The URI identifies the "…/alarms" collection resource.

- The query component is absent..

- The request message body contains a merge patch document. The document shall patch the "ackState" and "ackUserId" property of the identified alarm resources, and my patch the "ackSystemId" property. The patch document is defined by "MergePatchAcknowledgeAlarms".

2. The MnS producer sends a HTTP PATCH response to the MnS consumer.

- On success "200 OK" shall be returned. The response message body shall be empty.

- On failure, an appropriate error code shall be returned. The response message body shall return a list with the alarmId's that did not exist or were identifying alarms that could not be acknowledged, together with the failure reasons.

##### 12.2.1.1.6	Operation unacknowledgeAlarms

In case a single alarm shall be unacknowledged the IS operation parameters are mapped to SS equivalents according to table 12.2.1.1.6-1 and table 12.2.1.1.6-2.

Table 12.2.1.1.6-1: Mapping of IS operation input parameters to SS equivalents (HTTP PATCH)

| IS parameter name | SS parameter location | SS parameter name | SS parameter type | S |
| --- | --- | --- | --- | --- |
| alarmInformationReferenceList | path | /{alarmId} | string | M |
| ackUserId | request body | ackUserId | AckUserId | M |
| ackSystemId | request body | ackSystemId | AckSystemId | O |

Table 12.2.1.1.6-2: Mapping of IS operation input parameters to SS equivalents (HTTP PATCH)

| IS parameter name |  | SS parameter location |  | SS parameter name |  | SS parameter type | S |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| badAlarmInformationReferenceList |  | response body |  | n/a |  | errorResponse | M |  |
| status |  | response status codes |  | n/a |  | n/a |  |  | M |
|  |  | response body |  | error |  | ErrorResponse |  |  | O |

The message flow for unacknowledging a single alarm is as follows:

1. The MnS consumer sends a HTTP PATCH request to the MnS producer.

- The URI identifies the "…/alarms/{alarmId}" alarm resource to be acknowledged.

- The query component is absent.

- The request message body contains a merge patch document. The document shall patch the "ackState" and "ackUserId" property of the identified alarm resource, and may patch the "ackSystemId" property. The patch document is defined by "MergePatchAcknowledgeAlarms".

2. The MnS producer sends a HTTP PATCH response to the MnS consumer.

- On success "204 No Content" shall be returned. The response message body shall be empty.

- On failure, an appropriate error code shall be returned. The response message body may carry additional error information.

In case multiple alarms shall be unacknowledged the IS operation parameters are mapped to SS equivalents according to table 12.2.1.1.6-3 and table 12.2.1.1.6-4.

Table 12.2.1.1.6-3: Mapping of IS operation input parameters to SS equivalents (HTTP PATCH)

| IS parameter name | SS parameter location | SS parameter name | SS parameter type | S |
| --- | --- | --- | --- | --- |
| alarmInformationReferenceList | path request body | /alarms alarmId (key in map) | n/a string | M M |
| ackUserId | request body | ackUserId | AckUserId | M |
| ackSystemId | request body | ackSystemId | AckSystemId | O |

Table 12.2.1.1.6-4: Mapping of IS operation output parameters to SS equivalents (HTTP PATCH)

| IS parameter name | SS parameter location | SS parameter name | SS parameter type | S |
| --- | --- | --- | --- | --- |
| badAlarmInformationReferenceList | response body | error | array(failedAlarm) | M |
| status | response status codes | n/a | n/a | M |

The message flow for unacknowledging multiple alarms is as follows:

1. The MnS consumer sends a HTTP PATCH request to the MnS producer.

- The URI identifies the "…/alarms" collection resource.

- The query component is absent.

- The request message body contains a merge patch document. The document shall patch the "ackState" and "ackUserId" property of the identified alarm resources, and my patch the "ackSystemId" property. The patch document is defined by "MergePatchAcknowledgeAlarms".

2. The MnS producer sends a HTTP PATCH response to the MnS consumer.

- On success "200 OK" shall be returned. The response message body shall be empty.

- On failure, an appropriate error code shall be returned. The response message body shall return a list with the alarmId's that did not exist or were identifying alarms that could not be unacknowledged, together with the failure reasons.

##### 12.2.1.1.7	Operation clearAlarms

In case a single alarm shall be cleared the IS operation parameters are mapped to SS equivalents according to table 12.2.1.1.7-1 and table 12.2.1.1.7-2.

Table 12.2.1.1.7-1: Mapping of IS operation input parameters to SS equivalents (HTTP PATCH)

| IS parameter name | SS parameter location | SS parameter name | SS parameter type | S |
| --- | --- | --- | --- | --- |
| alarmInformationReferenceList | path | /{alarmId} | string | M |
| clearUserId | request body | clearUserId | ClearUserId | M |
| clearSystemId | request body | clearSystemId | ClearSystemId | O |

Table 12.2.1.1.7-2: Mapping of IS operation output parameters to SS equivalents (HTTP PATCH)

| IS parameter name |  | SS parameter location |  | SS parameter name |  | SS parameter type |  | S |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| badAlarmInformationReferenceList |  | response body |  | n/a |  | errorResponse |  | M |
| status |  | response status codes |  | n/a |  | n/a |  | M |  |
|  |  | response body |  | error |  | ErrorResponse |  | O |  |

The message flow for clearing a single alarm is as follows:

1. The MnS consumer sends a HTTP PATCH request to the MnS producer.

- The URI identifies the "…/alarms/{alarmId}" alarm resource.

- The query component is absent.

- The request message body contains a merge patch document. The document shall patch the "clearUserId" property, may patch the "clearSystemId" property and shall patch the "perceivedSeverity" property of the identified alarm resource represented by an "alarmRecord" object. The patch document is defined by "MergePatchClearAlarms".

2. The MnS producer sends a HTTP PATCH response to the MnS consumer.

- On success "204 No content" shall be returned. The response message body shall be empty.

- On failure, an appropriate error code shall be returned. The response message body shall return the alarmId that did not exist or was identifying an alarm that could not be cleared together with a failure reason. The JSON document carried in the response shall comply to  "FailedAlarms-Response" .

In case multiple alarms shall be cleared the IS operation parameters are mapped to SS equivalents according to table 12.2.1.1.7-3 and table 12.2.1.1.7-4.

Table 12.2.1.1.7-3: Mapping of IS operation input parameters to SS equivalents (HTTP PATCH)

| IS parameter name | SS parameter location | SS parameter name | SS parameter type | S |
| --- | --- | --- | --- | --- |
| alarmInformationReferenceList | path request body | /alarms alarmId (key in map) | n/a string | M M |
| clearUserId | request body | clearUserId | ClearUserId | M |
| clearSystemId | request body | clearSystemId | ClearSystemId | O |

Table 12.2.1.1.7-4: Mapping of IS operation output parameters to SS equivalents (HTTP PATCH)

| IS parameter name | SS parameter location | SS parameter name | SS parameter type | S |
| --- | --- | --- | --- | --- |
| badAlarmInformationReferenceList | response body | n/a | array(failedAlarm) | M |
| status | response status codes | n/a | n/a | M |

The message flow for clearing multiple alarms is as follows:

1. The MnS consumer sends a HTTP PATCH request to the MnS producer.

- The URI identifies the "…/alarms" collection resource.

- The query component is absent..

- The request message body contains a merge patch document. The document shall patch the "clearUserId" property, may patch the "clearSystemId" property and shall patch the "perceivedSeverity" property of the identified alarm resources . The patch document is defined by "patchClearAlarms-RequestType".

2. The MnS producer sends a HTTP PATCH response to the MnS consumer.

- On success "200 OK" shall be returned. The response message body shall be empty.

- On failure, an appropriate error code shall be returned. The response message body shall return a list with the alarmId's that did not exist or were identifying alarms that could not be cleared, together with the failure reasons.

##### 12.2.1.1.8	Operation subscribe

The IS operation parameters are mapped to SS equivalents according to table 12.2.1.1.8-1 and table 12.2.1.1.8-2.

Table 12.2.1.1.8-1: Mapping of IS operation input parameters to SS equivalents (HTTP POST)

| IS parameter name | SS parameter location | SS parameter name | SS parameter type | S |
| --- | --- | --- | --- | --- |
| consumerReference | request body | consumerReference | Uri | M |
| timeTick | request body | timeTick | integer | O |
| filter | request body | filter | Filter | O |

Table 12.2.1.1.8-2: Mapping of IS operation output parameters to SS equivalents (HTTP POST)

| IS parameter name |  | SS parameter location |  | SS parameter name |  | SS parameter type | S |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| subscriptionId |  | Location header |  | n/a |  | Uri | M |  |
| status | response status codes |  | n/a |  | n/a |  |  | M |
|  | response body |  | error |  | ErrorResponse |  |  | O |

The procedure for subscribing to notifications is as follows:

1. The MnS consumer  sends a HTTP POST request to the MnS producer.

- The URI identifies the "…/subscriptions" collection resource.

- The query component shall be absent.

- The request message body shall carry a data structure of type "Subscription". This data structure contains filtering criteria and a consumer side URI to which the provider will subsequently send notifications about events that match the filter.

2. The MnS producer creates a new subscription for notifications related to fault management, and a resource that represents this subscription.

3. The MnS producer sends a HTTP POST response to the MnS consumer.

- On success "201 Created " shall be returned. The response message body shall carry the representation of the created subscription resource. The Location header shall be present and carry the URI of the created subscription resource.

- On failure, an appropriate error code shall be returned. The response message body may carry additional error information.

##### 12.2.1.1.9	Operation unsubscribe

In case one subscription shall be cancelled the IS operation parameters are mapped to SS equivalents according to table 12.2.1.1.9-1 and table 12.2.1.1.9-2.

Table 12.2.1.1.9-1: Mapping of IS operation input parameters to SS equivalents (HTTP DELETE)

| IS parameter name | SS parameter location | SS parameter name | SS parameter type | S |
| --- | --- | --- | --- | --- |
| consumerReference | -- | -- | -- | -- |
| subscriptionId | path | /subscriptions/{subscriptionId} | string | M |

Table 12.2.1.1.9-2: Mapping of IS operation output parameters to SS equivalents (HTTP DELETE)

| IS parameter name |  | SS parameter location |  | SS parameter name |  | SS parameter type | S |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| status | response status codes |  | n/a |  | n/a |  |  | M |
|  | response body |  | error |  | ErrorResponse |  |  | O |

The consumer reference is not mapped in the present document.

The procedure for unsubscribing from one  subscription is as follows:

1. The MnS consumer sends a HTTP DELETE request to the MnS producer.

- The URI identifies the "…/subscriptions/{subscriptionId}" subscription resource.

- The querycomponent shall be absent.

- The request message body shall be empty.

2. The MnS producer sends a HTTP DELETE response to the MnS consumer.

- On success "204 No Content" shall be returned. The response message body shall be empty.

- On failure, an appropriate error code shall be returned. The response message body may carry an error object.

#### 12.2.1.2	Mapping of notifications

##### 12.2.1.2.1	Introduction

The IS notifications are mapped to SS equivalents according to table 12.2.1.2.1-1.

Table 12.2.1.2.1-1: Mapping of IS notifications to SS equivalents

| IS notification | HTTP Method | Resource URI | S |
| --- | --- | --- | --- |
| notifyNewAlarm | POST | {notificationTarget} | M |
| notifyAckStateChanged | POST | {notificationTarget} | M |
| notifyClearedAlarm | POST | {notificationTarget} | M |
| notifyAlarmListRebuilt | POST | {notificationTarget} | M |
| notifyChangedAlarm | POST | {notificationTarget} | M |
| notifyComments | POST | {notificationTarget} | M |
| notifyPotentialFaultyAlarmList | POST | {notificationTarget} | M |
| notifyCorrelatedNotificationChanged | POST | {notificationTarget} | M |
| notifyChangedAlarmGeneral | POST | {notificationTarget} | O |

##### 12.2.1.2.2	Notification notifyNewAlarm (non-security alarm)

The IS notification parameters are mapped to SS equivalents according to table 12.2.1.2.2-1.

Table 12.2.1.2.2-1: Mapping of IS notification parameters to SS equivalents

| IS parameter name | SS parameter location | SS parameter name | SS parameter type | S |
| --- | --- | --- | --- | --- |
| objectClass, objectInstance | request body | href | Uri | M |
| notificationId | request body | notificationId | NotificationId | M |
| notificationType | request body | notificationType | NotificationType | M |
| eventTime | request body | eventTime | DateTime | M |
| systemDN | request body | systemDN | SystemDN | M |
| alarmId | request body | alarmId | AlarmId | M |
| alarmType | request body | alarmType | AlarmType | M |
| probableCause | request body | probableCause | ProbableCause | M |
| specificProblem | request body | specificProblem | SpecificProblem | O |
| perceivedSeverity | request body | perceivedSeverity | PerceivedSeverity | M |
| correlatedNotifications | request body | correlatedNotifications | array(CorrelatedNotification) | O |
| backedUpStatus | request body | backedUpStatus | boolean | O |
| backUpObject | request body | backUpObject | Dn | O |
| trendIndication | request body | trendIndication | TrendIndication | O |
| thresholdInfo | request body | thresholdInfo | ThresholdInfo | O |
| correlatedNotifications | request body | correlatedNotifications | array(CorrelatedNotification) | O |
| stateChangeDefinition | request body | stateChangeDefinition | AttributeValueChangeSet | O |
| monitoredAttributes | request body | monitoredAttributes | AttributeNameValuePairSet | O |
| proposedRepairActions | request body | proposedRepairActions | string | O |
| additionalText | request body | additionalText | string | O |
| additionalInformation | request body | additionalInformation | AttributeNameValuePairSet | O |

##### 12.2.1.2.3	Notification notifyNewAlarm (security alarm)

The IS notification parameters are mapped to SS equivalents according to table 12.2.1.2.3-1.

Table 12.2.1.2.3-1: Mapping of IS notification parameters to SS equivalents

| IS parameter name | SS parameter location | SS parameter name | SS parameter type | S |
| --- | --- | --- | --- | --- |
| objectClass, objectInstance | request body | href | Uri | M |
| notificationId | request body | notificationId | NotificationId | M |
| notificationType | request body | notificationType | NotificationType | M |
| eventTime | request body | eventTime | DateTime | M |
| systemDN | request body | systemDN | SystemDN | M |
| alarmId | request body | alarmId | AlarmId | M |
| alarmType | request body | alarmType | AlarmType | M |
| probableCause | request body | probableCause | ProbableCause | M |
| perceivedSeverity | request body | perceivedSeverity | PerceivedSeverity | M |
| correlatedNotifications | request body | correlatedNotifications | array(CorrelatedNotification) | O |
| additionalText | request body | additionalText | string | O |
| additionalInformation | request body | additionalInformation | AttributeNameValuePairSet | O |
| rootCauseIndicator | request body | rootCauseIndicator | boolean | O |
| serviceUser | request body | serviceUser | string | M |
| serviceProvider | request body | serviceProvider | string | M |
| securityAlarmDetector | request body | securityAlarmDetector | string | M |

##### 12.2.1.2.4	Notification notifyAckStateChanged

The IS notification parameters are mapped to SS equivalents according to table 12.2.1.2.4-1.

Table 12.2.1.2.4-1: Mapping of IS notification parameters to SS equivalents

| IS parameter name | SS parameter location | SS parameter name | SS parameter type | S |
| --- | --- | --- | --- | --- |
| objectClass, objectInstance | request body | href | Uri | M |
| notificationId | request body | notificationId | NotificationId | M |
| notificationType | request body | notificationType | NotificationType | M |
| eventTime | request body | eventTime | DateTime | M |
| systemDN | request body | systemDN | SystemDN | M |
| alarmId | request body | alarmId | AlarmId- | M |
| alarmType | request body | alarmType | AlarmType | M |
| probableCause | request body | probableCause | ProbableCause | M |
| perceivedSeverity | request body | perceivedSeverity | PerceivedSeverity | M |
| ackState | request body | ackState | AckState | M |
| ackUserId | request body | ackUserId | string | M |
| ackSystemId | request body | ackSystemId | string | O |

##### 12.2.1.2.5	Notification notifyClearedAlarm

The IS notification parameters are mapped to SS equivalents according to table 12.2.1.2.5-1.

Table 12.2.1.2.5-1: Mapping of IS notification parameters to SS equivalents

| IS parameter name | SS parameter location | SS parameter name | SS parameter type | S |
| --- | --- | --- | --- | --- |
| objectClass, objectInstance | request body | href | Uri | M |
| notificationId | request body | notificationId | NotificationId | M |
| notificationType | request body | notificationType | NotificationType | M |
| eventTime | request body | eventTime | DateTime | M |
| systemDN | request body | systemDN | SystemDN | M |
| alarmId | request body | alarmId | AlarmId | M |
| alarmType | request body | alarmType | AlarmType | M |
| probableCause | request body | probableCause | ProbableCause | M |
| perceivedSeverity | request body | perceivedSeverity | PerceivedSeverity | M |
| correlatedNotifications | request body | correlatedNotifications | array(CorrelatedNotification) | O |
| clearUserId | request body | clearUserId | string | O |
| clearSystemId | request body | clearSystemId | string | O |

##### 12.2.1.2.6	Notification notifyAlarmListRebuilt

The IS notification parameters are mapped to SS equivalents according to table 12.2.1.2.6-1.

Table 12.2.1.2.6-1: Mapping of IS notification parameters to SS equivalents

| IS parameter name | SS parameter location | SS parameter name | SS parameter type | S |
| --- | --- | --- | --- | --- |
| objectClass, objectInstance | request body | href | Uri | M |
| notificationId | request body | notificationId | NotificationId | M |
| notificationType | request body | notificationType | NotificationType | M |
| eventTime | request body | eventTime | DateTime | M |
| systemDN | request body | systemDN | SystemDN | M |
| reason | request body | reason | string | M |
| alarmListAlignmentRequirement | request body | alarmListAlignmentRequirement | AlarmListAlignmentRequirement | O |

##### 12.2.1.2.7	Notification notifyChangedAlarm

The IS notification parameters are mapped to SS equivalents according to table 12.2.1.2.7-1.

Table 12.2.1.2.7-1: Mapping of IS notification parameters to SS equivalents

| IS parameter name | SS parameter location | SS parameter name | SS parameter type | S |
| --- | --- | --- | --- | --- |
| objectClass, objectInstance | request body | href | Uri | M |
| notificationId | request body | notificationId | NotificationId | M |
| notificationType | request body | notificationType | NotificationType | M |
| eventTime | request body | eventTime | DateTime | M |
| systemDN | request body | systemDN | SystemDN | M |
| alarmId | request body | alarmId | AlarmId | M |
| alarmType | request body | alarmType | AlarmType | M |
| probableCause | request body | probableCause | ProbableCause | M |
| perceivedSeverity | request body | perceivedSeverity | PerceivedSeverity | M |

##### 12.2.1.2.8	Notification notifyComments

The IS notification parameters are mapped to SS equivalents according to table 12.2.1.2.8-1.

Table 12.2.1.2.8-1: Mapping of IS notification parameters to SS equivalents

| IS parameter name | SS parameter location | SS parameter name | SS parameter type | S |
| --- | --- | --- | --- | --- |
| objectClass, objectInstance | request body | href | Uri | M |
| notificationId | request body | notificationId | NotificationId | M |
| notificationType | request body | notificationType | NotificationType | M |
| eventTime | request body | eventTime | DateTime | M |
| systemDN | request body | systemDN | SystemDN | M |
| alarmId | request body | alarmId | AlarmId | M |
| alarmType | request body | alarmType | AlarmType | M |
| probableCause | request body | probableCause | ProbableCause | M |
| perceivedSeverity | request body | perceivedSeverity | PerceivedSeverity | M |
| comments | request body | comments | map(Comment) | M |

##### 12.2.1.2.9	Notification notifyPotentialFaultyAlarmList

The IS notification parameters are mapped to SS equivalents according to table 12.2.1.2.9-1.

Table 12.2.1.2.9-1: Mapping of IS notification parameters to SS equivalents

| IS parameter name | SS parameter location | SS parameter name | SS parameter type | S |
| --- | --- | --- | --- | --- |
| objectClass, objectInstance | request body | href | Uri | M |
| notificationId | request body | notificationId | NotificationId | M |
| notificationType | request body | notificationType | NotificationType | M |
| eventTime | request body | eventTime | DateTime | M |
| systemDN | request body | systemDN | SystemDN | M |
| reason | request body | reason | string | M |

##### 12.2.1.2.10	Notification notifyCorrelatedNotificationChanged

The IS notification parameters are mapped to SS equivalents according to table 12.2.1.2.10-1.

Table 12.2.1.2.10-1: Mapping of IS notification parameters to SS equivalents

| IS parameter name | SS parameter location | SS parameter name | SS parameter type | S |
| --- | --- | --- | --- | --- |
| objectClass, objectInstance | request body | href | Uri | M |
| notificationId | request body | notificationId | NotificationId | M |
| notificationType | request body | notificationType | NotificationType | M |
| eventTime | request body | eventTime | DateTime | M |
| systemDN | request body | systemDN | SystemDN | M |
| alarmId | request body | alarmId | AlarmId | M |
| correlatedNotifications | request body | correlatedNotifications | array(CorrelatedNotification) | M |
| rootCauseIndicator | request body | rootCauseIndicator | boolean | O |

##### 12.2.1.2.11	Notification notifyChangedAlarmGeneral (non-security alarm)

The IS notification parameters are mapped to SS equivalents according to table 12.2.1.2.11-1.

Table 12.2.1.2.11-1: Mapping of IS notification parameters to SS equivalents

| IS parameter name | SS parameter location | SS parameter name | SS parameter type | S |
| --- | --- | --- | --- | --- |
| objectClass, objectInstance | request body | href | Uri | M |
| notificationId | request body | notificationId | NotificationId | M |
| notificationType | request body | notificationType | NotificationType | M |
| eventTime | request body | eventTime | DateTime | M |
| systemDN | request body | systemDN | SystemDN | M |
| alarmId | request body | alarmId | AlarmId | M |
| alarmType | request body | alarmType | AlarmType | M |
| probableCause | request body | probableCause | ProbableCause | O |
| specificProblem | request body | specificProblem | SpecificProblem | O |
| perceivedSeverity | request body | perceivedSeverity | PerceivedSeverity | O |
| backedUpStatus | request body | backedUpStatus | booleanbackedUpStatus | O |
| backUpObject | request body | backUpObject | Dn | O |
| trendIndication | request body | trendIndication | TrendIndication | O |
| thresholdInfo | request body | thresholdInfo | ThresholdInfo | O |
| correlatedNotifications | request body | correlatedNotifications | array(CorrelatedNotification) | O |
| stateChangeDefinition | request body | stateChangeDefinition | AttributeValueChangeSet | O |
| monitoredAttributes | request body | monitoredAttributes | AttributeNameValuePairSet | O |
| proposedRepairActions | request body | proposedRepairActions | string | O |
| additionalText | request body | additionalText | string | O |
| additionalInformation | request body | additionalInformation | AttributeNameValuePairSet | O |
| rootCauseIndicator | request body | rootCauseIndicator | boolean | O |
| changedAlarmAttributes | request body | changedAlarmAttributes | AttributeNameValuePairSet | O |

##### 12.2.1.2.12	Notification notifyChangedAlarmGeneral (security alarm)

The IS notification parameters are mapped to SS equivalents according to table 12.2.1.2.12-1.

Table 12.2.1.2.12-1: Mapping of IS notification parameters to SS equivalents

| IS parameter name | SS parameter location | SS parameter name | SS parameter type | S |
| --- | --- | --- | --- | --- |
| objectClass, objectInstance | request body | href | Uri | M |
| notificationId | request body | notificationId | NotificationId | M |
| notificationType | request body | notificationType | NotificationType | M |
| eventTime | request body | eventTime | DateTime | M |
| systemDN | request body | systemDN | SystemDN | M |
| alarmId | request body | alarmId | AlarmId | M |
| alarmType | request body | alarmType | AlarmType | M |
| probableCause | request body | probableCause | ProbableCause | O |
| perceivedSeverity | request body | perceivedSeverity | PerceivedSeverity | O |
| correlatedNotifications | request body | correlatedNotifications | array(CorrelatedNotification) | O |
| additionalText | request body | additionalText | string | O |
| additionalInformation | request body | additionalInformation | AttributeNameValuePairSet | O |
| rootCauseIndicator | request body | rootCauseIndicator | boolean | O |
| serviceUser | request body | serviceUser | string | M |
| serviceProvider | request body | serviceProvider | string | M |
| securityAlarmDetector | request body | securityAlarmDetector | string | M |
| changedAlarmAttributes | request body | changedAlarmAttributes | AttributeNameValuePairSet | M |

#### 12.2.1.3	Resources

##### 12.2.1.3.1	Resource structure

###### 12.2.1.3.1.1	Resource structure on the MnS producer

Figure 12.2.1.3.1.1-1 shows the resource structure of the Fault Supervision MnS on the MnS producer.

Figure 12.2.1.3.1-1: Resource URI structure of the Fault Supervision MnS on the MnS producer

Table 12.2.1.3.1.1-1 provides an overview of the resources and applicable HTTP methods.

Table 12.2.1.3.1.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method | Description |
| --- | --- | --- | --- |
| Alarms | .../alarms | GET | Retrieve all alarms or a filtered subset |
|  |  | PATCH | Clear, acknowledge or unacknowledge multiple alarms |
| Alarm Count | .../alarms/alarmCount | GET | Retrieve the alarm count per perceived severity |
| Alarm | .../alarms/{alarmId} | PATCH | Clear, acknowledge or unacknowledge an alarm |
| Comments | .../alarms/{alarmId}/comments | POST | Add a comment to an alarm |
| Subscriptions | .../subscriptions | POST | Create a subscription |
| Subscription | .../subscriptions/{subscriptionId} | DELETE | Delete a subscription |

###### 12.2.1.3.1.2	Resource structure on the MnS consumer

Figure 12.2.1.3.1.2-1 shows the resource structure of the Fault Supervision MnS on the MnS consumer.

Figure 12.2.1.3.1.2-1: Resource URI structure of the Fault Supervision MnS on the MnS consumer

Table 12.2.1.3.1.2-1 provides an overview of the resources and applicable HTTP methods.

Table 12.2.1.3.1.2-1: Resources and methods overview

| Resource name | Resource URI | HTTP method | Description |
| --- | --- | --- | --- |
| Notification Target | {notificationTarget} | POST | Send a notification to the notification target |

##### 12.2.1.3.2	Resource definitions

###### 12.2.1.3.2.1	Resource "…/alarms"

12.2.1.3.2.1.1	Description

This resource represents a collection of alarms.

12.2.1.3.2.1.2	URI

Resource URI: {MnSRoot}/FaultSupervisionMnS/{MnSVersion}/alarms

The resource URI variables are defined in table 12.2.1.3.2.1.2-1.

Table 12.2.1.3.2.1.2-1: URI variables

| Name | Definition |
| --- | --- |
| MnSRoot | See clause 4.4.2 of TS 32.158 [15] |
| MnSVersion | See clause 4.4.2 of TS 32.158 [15] |

12.2.1.3.2.1.3	HTTP methods

12.2.1.3.2.1.3.1	HTTP GET

This method shall support the URI query parameters specified in the following table.

Table 12.2.1.3.2.1.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | Description | S |
| --- | --- | --- | --- |
| alarmAckState | AlarmAckState |  | O |
| href | Dn |  | O |
| filter | string |  | O |

This method shall support the request data structures, the response data structures and response codes specified in the following table.

Table 12.2.1.3.2.1.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | Description | S |
| --- | --- | --- |
| n/a |  |  |

Table 12.2.1.3.2.1.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | Response codes | Description | S |
| --- | --- | --- | --- |
| GetAlarmsResponse | 200 OK | The alarms returned. | M |
| ErrorResponse | 4xx/5xx | Returned in case of an error | O |

12.2.1.3.2.1.3.2	Void

12.2.1.3.2.1.3.3	HTTP PATCH

This method shall support the URI query parameters specified in the following table.

Table 12.2.1.3.2.1.3.3-1: URI query parameters supported by the PATCH method on this resource

| Name | Data type | Description | S |
| --- | --- | --- | --- |
| n/a | n/a | n/a | n/a |

This method shall support the request data structures, the response data structures and response codes specified in the following table.

Table 12.2.1.3.2.1.3.3-2: Data structures supported by the PATCH Request Body on this resource

| Data type | Description | S |
| --- | --- | --- |
| map(MergePatchAcknowledgeAlarm) | Patch document for acknowledging one or multiple alarms | M |
| map(MergePatchClearAlarm) | Patch document for clearing one or multiple alarms | M |

Table 12.2.1.3.2.1.3.3-3: Data structures supported by the PATCH Response Body on this resource

| Data type | Response codes | Description | S |
| --- | --- | --- | --- |
| n/a | 204 No Content | In case of success the response body shall be empty. | M |
| FailedAlarms-Response | 4xx/5xx | In case of failure, the response body shall carry a JSON object described by the "FailedAlarmsResponse" format. | M |

###### 12.2.1.3.2.2	Resource "…/alarms /{alarmId}"

12.2.1.3.2.2.1	Description

This resource represents an alarm.

12.2.1.3.2.2.2	URI

Resource URI: {MnSRoot}/FaultSupervisionMnS/{MnSVersion}/alarms/{alarmId}

The resource URI variables are defined in table 12.2.1.3.2.2.2-1.

Table 12.2.1.3.2.2.2-1: URI variables

| Name | Definition |
| --- | --- |
| MnSRoot | See clause 4.4.2 of TS 32.158 [15] |
| MnSVersion | See clause 4.4.2 of TS 32.158 [15] |
| alarmId | Alarm identifier |

12.2.1.3.2.2.3	HTTP methods

12.2.1.3.2.2.3.1	HTTP PATCH

This method shall support the URI query parameters specified in the following table.

Table 12.2.1.3.2.2.3.1-1: URI query parameters supported by the PATCH method on this resource

| Name | Data type | Description | S |
| --- | --- | --- | --- |
| n/a | n/a | n/a | n/a |

This method shall support the request data structures, the response data structures and response codes specified in the following table.

Table 12.2.1.3.2.2.3.1-2: Data structures supported by the PATCH Request Body on this resource

| Data type | Description | S |
| --- | --- | --- |
| MergePatchAcknowledgeAlarm | Patch document for acknowledging an alarm | M |
| MergePatchClearAlarm | Patch document for clearing an alarm | M |

Table 12.2.1.3.2.2.3.1-3: Data structures supported by the PATCH Response Body on this resource

| Data type | Response codes | Description | S |
| --- | --- | --- | --- |
| n/a | 200 OK | In case of success the response body shall be empty. |  |
| ErrorResponse | 4xx/5xx | In case of failure, the response body shall carry a JSON object described by "ErrorResponse". |  |

###### 12.2.1.3.2.3	Resource "…/alarms/alarmCount"

12.2.1.3.2.3.1	Definition

This resource holds metadata about the /alarms collection resource like the alarm count per perceived severity.

12.2.1.3.2.3.2	URI

Resource URI: {MnSRoot}/FaultSupervisionMnS/{MnSVersion}/alarms/alarmCount

The resource URI variables are defined in table 12.2.1.3.2.3.2-1.

Table 12.2.1.3.2.3.2-1: URI variables

| Name | Definition |
| --- | --- |
| MnSRoot | See clause 4.4.2 of TS 32.158 [15] |
| MnSVersion | See clause 4.4.2 of TS 32.158 [15] |

12.2.1.3.2.3.3	HTTP methods

12.2.1.3.2.3.3.1	GET

This method shall support the URI query parameters specified in table 12.2.1.3.2.3.3.1-1.

Table 12.2.1.3.2.3.3.3-1: URI query parameters supported by the GET method on this resource

| Name | Data type | Description | S |
| --- | --- | --- | --- |
| alarmAckState | AlarmAckState | Allows to control which alarms are counted based on acknowledgement state | O |
| filter | string | Allows to control which alarms are counted based on a general filter applied to the alarm records. | O |

This method shall support the request data structures specified in table 12.2.1.3.2.3.3.1-2 and the response data structures and response codes specified in table 12.2.1.3.2.3.3.1-3.

Table 12.2.1.3.2.3.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | Description | S |
| --- | --- | --- |
| n/a | n/a | n/a |

Table 12.2.1.3.2.3.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | Response codes | Description | S |
| --- | --- | --- | --- |
| AlarmsCount | 200 OK | The alarm count per severity level returned. | M |
| ErrorResponse | 4xx/5xx | Returned in case of an error | O |

###### 12.2.1.3.2.4	Resource "…/alarms/{alarmId}/comments"

12.2.1.3.2.4.1	Definition

This resource is a collection resource for comments attached to an alarm.

12.2.1.3.2.4.2	URI

Resource URI: {MnSRoot}/FaultSupervisionMnS/{MnSVersion}/alarms/{alarmId}/comments

The resource URI variables are defined in table 12.2.1.3.2.4.2-1.

Table 12.2.1.3.2.4.2-1: URI variables

| Name | Definition |
| --- | --- |
| MnSRoot | See clause 4.4.2 of TS 32.158 [15] |
| MnSVersion | See clause 4.4.2 of TS 32.158 [15] |
| alarmId | Alarm identifier |

12.2.1.3.2.4.3	HTTP methods

12.2.1.3.2.4.3.1	POST

This method shall support the URI query parameters specified in the following table.

Table 12.2.1.3.2.4.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | Description | S |
| --- | --- | --- | --- |
| n/a | n/a | n/a | n/a |

This method shall support the request data structures, and the response data structures and response codes specified in the following tables.

Table 12.2.1.3.2.4.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | Description | S |
| --- | --- | --- |
| Comment | The representation of the comment to be added to an alarm. | M |

Table 12.2.1.3.2.4.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | Response codes | Description | S |
| --- | --- | --- | --- |
| Comment-ResponseType | 201 Created | In case of success, the response body shall be carry the representation of a comment. The "commentTime" shall be set by the MnS producer. | M |
| ErrorResponse | 4xx/5xx | In case of failure, the response body shall be described by "ErrorResponse". | M |

###### 12.2.1.3.2.5	Resource "…/comments/{commentId}"

12.2.1.3.2.5.1	Definition

This resource represents a comment attached to an alarm.

12.2.1.3.2.5.2	URI

Resource URI: {MnSRoot}/FaultSupervisionMnS/{MnSVersion}/alarms/{alarmId}/comments/{commentId}

The resource URI variables are defined in table 12.2.1.3.2.4.5-1.

Table 12.2.1.3.2.4.5-1: URI variables

| Name | Definition |
| --- | --- |
| MnSRoot | See clause 4.4.2 of TS 32.158 [15] |
| MnSVersion | See clause 4.4.2 of TS 32.158 [15] |
| alarmId | Alarm identifier |
| commentId | Comment identifier |

12.2.1.3.2.5.3	HTTP methods

None.

###### 12.2.1.3.2.6	Resource "…/subscriptions"

12.2.1.3.2.6.1	Description

This resource is a container resource for individual subscriptions.

12.2.1.3.2.6.2	URI

Resource URI: {MnSRoot}/FaultSupervisionMnS/{MnSVersion}/subscriptions

The resource URI variables are defined in table 12.2.1.3.2.6.2-1.

Table 12.2.1.3.2.6.2-1: URI variables

| Name |  | Definition |
| --- | --- | --- |
| MnSRoot |  | See clause 4.4.2 of TS 32.158 [15] |
| MnSVersion | See clause 4.4.2 of TS 32.158 [15] |  |

12.2.1.3.2.6.3	HTTP methods

12.2.1.3.2.6.3.1	POST

This method shall support the URI query parameters specified in table 12.2.1.3.2.6.3.1-1.

Table 12.2.1.3.2.6.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | Description | S |
| --- | --- | --- | --- |
| n/a | n/a | n/a | n/a |

This method shall support the request data structures specified in table 12.2.1.3.2.6.3.1-2 and the response data structures and response codes specified in table 12.2.1.3.2.6.3.1-3.

Table 12.2.1.3.2.6.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | Description | S |
| --- | --- | --- |
| Subscription | Details of the subscription to be created | M |

Table 12.2.1.3.2.6.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | Response codes | Description | S |
| --- | --- | --- | --- |
| Subscription | 201 Created | In case of success the representation of the created subscription is returned. | M |
| ErrorResponse | 4xx/5xx | In case of failure the error object is returned. | M |

12.2.1.3.2.6.3.2	Void

###### 12.2.1.3.2.7	Resource "…/subscriptions/{subscriptionId}"

12.2.1.3.2.7.1	Description

This resource represents a subscription.

12.2.1.3.2.7.2	URI

Resource URI: {MnSRoot}/FaultSupervisionMnS/{MnSVersion}/ subscriptions/{subscriptionId}

The resource URI variables are defined in table 12.2.1.3.2.7.2-1.

Table 12.2.1.3.2.7.2-1: URI variables

| Name | Definition |
| --- | --- |
| MnSRoot | See clause 4.4.2 of TS 32.158 [15] |
| MnSVersion | See clause 4.4.2 of TS 32.158 [15] |
| subscriptionId | Subscription identifier |

12.2.1.3.2.7.3	HTTP methods

12.2.1.3.2.7.3.1	DELETE

This method shall support the URI query parameters specified in table 12.2.1.3.2.7.3.1-1.

Table 12.2.1.3.2.7.3.1-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | Description | S |
| --- | --- | --- | --- |
| n/a | n/a | n/a | n/a |

This method shall support the request data structures specified in table 12.2.1.3.2.7.3.1-2 and the response data structures and response codes specified in table 12.2.1.3.2.7.3.1-3.

Table 12.2.1.3.2.7.3.1-2: Data structures supported by the DELETE Request Body on this resource

| Data type | Description | S |
| --- | --- | --- |
| n/a | n/a | n/a |

Table 12.2.1.3.2.7.3.1-3: Data structures supported by the DELETE Response Body on this resource

| Data type | Response codes | Description | S |
| --- | --- | --- | --- |
| n/a | 204 No Content | In case of success no message body is returned | M |
| ErrorResponse | 4xx/5xx | In case of failure the error object is returned. | M |

###### 12.2.1.3.2.8	Resource "{notificationTarget}"

12.2.1.3.2.8.1	Description

This resource represents a notification target on the MnS consumer.

12.2.1.3.2.8.2	URI

Resource URI: {notificationTarget}

The resource URI variables are defined in table 12.2.1.3.2.8.2-1.

Table 12.2.1.3.2.8.2-1: URI variables

| Name | Definition |
| --- | --- |
| notificationTarget | URI of the notification target on the MnS consumer, contained in the notification subscription |

12.2.1.3.2.8.3	HTTP methods

12.2.1.3.2.8.3.1	POST

This method shall support the URI query parameters specified in table 12.2.1.3.2.8.3.1-1.

Table 12.2.1.3.2.8.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | Description | S |
| --- | --- | --- | --- |
| n/a | n/a | n/a | n/a |

This method shall support the request data structures specified in table 12.2.1.3.2.8.3.1-2 and the response data structures and response codes specified in table 12.2.1.3.2.8.3.1-3.

Table 12.2.1.3.2.8.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type |  | Description |  | S |  |
| --- | --- | --- | --- | --- | --- |
| NotifyNewAlarm |  | Type for a notifyNewAlarm notification (non-security alarm) |  | M |  |
| NotifyNewSecurityAlarm- |  | Type for a notifyNewAlarm notification (security alarm) |  | M |  |
| NotifyAckStateChanged |  | Type for a notifyAckStateChanged notification |  | M |  |
| NotifyClearedAlarm |  | Type for a notifyClearedAlarm notification |  | M |  |
| NotifyAlarmListRebuilt |  | Type for a notifyAlarmListRebuilt notification |  | M |  |
| NotifyChangedAlarm |  | Type for a notifyChangedAlarm notification |  | M |  |
| NotifyComments |  | Type for a notifyComments notification |  | M |  |
| NotifyPotentialFaultyAlarmList |  | Type for a notifyPotentialFaultyAlarmList notification |  | M |  |
| NotifyCorrelatedNotificationChanged |  | Type for a notifyCorrelatedNotificationChanged notification (non-security alarm) |  | M |  |
| NotifyChangedAlarmGeneral |  | Type for a notifyChangedAlarmGeneral notification |  | M |  |
| NotifyChangedSecAlarmGeneral |  | Type for a notifyChangedAlarmGeneral notification (security alarm) |  | M |  |

Table 12.2.1.3.2.8.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | Response codes | Description | S |
| --- | --- | --- | --- |
| n/a | 204 No Content | In case of success no message body is returned | M |
| ErrorResponse | 4xx/5xx | In case of failure the error object is returned. | M |

#### 12.2.1.4	Data type definitions

##### 12.2.1.4.1	General

This clause defines the data types used by the Fault Supervision MnS. Table 12.2.1.4.1-1 specifies the data types defined in the present document and table 12.2.1.4.1-2 the data types imported.

Table 12.2.1.4.1-1: Data types defined in the present document

| Data type | Reference | Description |
| --- | --- | --- |
| AlarmAckState | 12.2.1.4.3.4 | Used in the query part of HTTP GET on /alarms to discriminate alarms to be returned or counted |
| AlarmId | 12.2.1.4.4.2 | Alarm identifier, see clause 11.2.2.1.5.1 |
| AlarmType | 12.2.1.4.4.6 | Alarm type as defined in ITU-T Rec. X. 733 [4] |
| ProbableCause | 12.2.1.4.4.7 | Probable cause of an alarm as defined in ITU-T Rec. X.733 [4] |
| PerceivedSeverity | 12.2.1.4.4.9 | Perceived severity of an alarm as defined in ITU-T Rec. X. 733 [4] |
| TrendIndication | 12.2.1.4.4.10 | Severity trend of the alarmed object as defined in ITU-T Rec. X. 733 [4] |
| ThresholdHysteresis | 12.2.1.4.1a.1 | Used in the definition of ThresholdInfo as defined in ITU-T Rec. X. 733 [4] |
| ThresholdLevelInd | 12.2.1.4.1a.2 | Used in the definition of ThresholdInfo as defined in ITU-T Rec. X. 733 [4] |
| ThresholdInfo | 12.2.1.4.1a.3 | Provides information for threshold crossing alarms as defined in ITU-T Rec. X. 733 [4] |
| CorrelatedNotification | 12.2.1.4.1a.4 | Describes the correlated notifications of a single source |
| AckState | 12.2.1.4.4.4 | Acknowledgement state, see clause 11.2.2.1.5.1 |
| AlarmNotificationTypes | 12.2.1.4.4.8 | Alarm notification types (notifyNewAlarm, etc.) |
| AlarmListAlignmentRequirement | 12.2.1.4.4.5 | Indicating if alarm list alignment is required or not |
| AlarmRecord | 12.2.1.4.1a.5 | Representation of an alarm resource |
| AlarmCount | 12.2.1.4.1a.6 | Representation of an alarmCout resource |
| Comment | 12.2.1.4.1a.7 | Representation of a comment resource |
| Subscription | 12.2.1.4.1a.8 | Representation of a subscription resource |
| MergePatchAcknowledgeAlarm | 12.2.1.4.1a.9 | Used in the request message body of HTTP PATCH to acknowledge or unacknowledge an alarm |
| MergePatchClearAlarm | 12.2.1.4.1a.10 | Used in the request body of HTTP PATCH to clear an alarm |
| FailedAlarm | 12.2.1.4.1a.11 | Used in the response body of multiple HTTP methods to indicate error reasons per alarm id |
| NotifyNewAlarm | 12.2.1.4.1a.12 | Used in the request body of HTTP POST for the notification type notifyNewAlarm |
| NotifyNewSecAlarm | 12.2.1.4.1a.13 | Used in the request body of HTTP POST for the notification type notifyNewAlarm |
| NotifyClearedAlarm | 12.2.1.4.1a.14 | Used in the request body of HTTP POST for the notification type notifyClearedAlarm |
| NotifyChangedAlarm | 12.2.1.4.1a.15 | Used in the request body of HTTP POST for the notification type notifyChangedAlarm |
| NotifyChangedAlarmGeneral | 12.2.1.4.1a.16 | Used in the request body of HTTP POST for the notification type notifyChangedAlarmGeneral |
| NotifyChangedSecAlarmGeneral | 12.2.1.4.1a.17 | Used in the request body of HTTP POST for the notification type notifyChangedAlarmGeneral |
| NotifyCorrelatedNotificationChanged | 12.2.1.4.1a.18 | Used in the request body of HTTP POST for the notification type notifyCorrelatedNotificationChanged |
| NotifyAckStateChanged | 12.2.1.4.1a.19 | Used in the request body of HTTP POST for the notification type notifyAckStateChanged |
| NotifyComments | 12.2.1.4.1a.20 | Used in the request body of HTTP POST for the notification type notifyComments |
| NotifyPotentialFaultyAlarmList | 12.2.1.4.1a.21 | Used in the request body of HTTP POST for the notification type notifyPotentialFaultyAlarmList |
| NotifyAlarmListRebuilt | 12.2.1.4.1a.22 | Used in the request body of HTTP POST for the notification type notifyAlarmListRebuilt |

Table 12.2.1.4.1-2: Data types imported

| Data type | Reference | Description |
| --- | --- | --- |
| DateTime | TS 28.623 [44] | Date and time |
| Float | TS 28.623 [44] | Float type |
| Dn | TS 28.623 [44] | DN type |
| SystemDN | TS 28.623 [44] | systemDN type |
| Uri | TS 28.623 [44] | URI type |
| AttributeNameValuePairSet | TS 28.623 [44] | Set of attribute name/value pairs |
| AttributeValueChangeSet | TS 28.623 [44] | Set of attribute names with their old and new values |
| Filter | TS 28.623 [44] | Filter type |
| NotificationId | TS 28.623 [44] | Notification identifier as defined in ITU-T Rec. X. 733 [4] |
| NotificationType | TS 28.623 [44] | Notification type |
| NotificationHeader | TS 28.623 [44] | Notification header |
| ErrorResponse | TS 28.623 [44] | Used in the response body of multiple HTTP methods in case of error |

##### 12.2.1.4.1a	Structured data types

###### 12.2.1.4.1a.1	Type ThresholdHysteresis

Table 12.2.1.4.1a.1-1: Definition of type ThresholdHysteresis

| Attribute name | Data type | Description | S |
| --- | --- | --- | --- |
| high | oneOf(integer, Float) | Higher value of a threshold with hysteris, the integer type is used for counter thresholds and the float type for gauge thresholds. | M |
| low | Float | Lower value of a threshold with hysteresis, applicable only to gauge thresholds. | M |

###### 12.2.1.4.1a.2	Type ThresholdLevelInd

Table 12.2.1.4.1a.2-1: Definition of type ThresholdLevelInd

| Attribute name (choice) | Data type | Description | S |
| --- | --- | --- | --- |
| up | ThresholdHysteresis | Indicates for counter and gauge thresholds that the threshold crossing occurred when going up. | M |
| down | ThresholdHysteresis | Indicates for gauge thresholds that the threshold crossing occurred when going down, applicable only to gauge thresholds. | M |

###### 12.2.1.4.1a.3	Type ThresholdInfo

Table 12.2.1.4.1a.3-1: Definition of type ThresholdInfo

| Attribute name | Data type | Description | S |
| --- | --- | --- | --- |
| observedMeasurement | string | The name of the monitored measurement that crossed the threshold and that caused the notification (Rec. ITU-T X. 733 [4]). | M |
| observedValue | oneOf(integer, Float) | The value of the gauge or counter which crossed the threshold. This may be different from the threshold value if, for example, the gauge may only take on discrete values. The integer type is used for counters and the float type for gauges (Rec. ITU-T X. 733 [4]). | M |
| thresholdLevel | ThresholdLevelInd | In the case of a gauge the threshold level specifies a pair of threshold values, the first being the value of the crossed threshold and the second, its corresponding hysteresis; in the case of a counter the threshold level specifies only the threshold value (Rec. ITU-T X. 733 [4]). | O |
| armTime | DateTime | For a gauge threshold, the time at which the threshold was last re-armed, namely the time after the previous threshold crossing at which the hysteresis value of the threshold was exceeded thus again permitting generation of notifications when the threshold is crossed. For a counter threshold, the later of the time at which the threshold offset was last applied, or the time at which the counter was last initialized (for resettable counters) (Rec. ITU-T X. 733 [4]). | O |

###### 12.2.1.4.1a.4	Type CorrelatedNotification

Table 12.2.1.4.1a.4-1: Definition of type CorrelatedNotification

| Attribute name | Data type | Description | S |
| --- | --- | --- | --- |
| sourceObjectInstance | Dn | Source oject instance of the notifications identified by notificationIds. The sourceObjectInstance shall be present if the sourceObjectInstance is not identical to the alarmed objectInstance of the alarmRecord | O |
| notificationIds | array(NotificationId) | Notification identifiers of notifications related to the sourceObjectInstance that are considered to be correlated to the alarmRecord | M |

###### 12.2.1.4.1a.5	Type AlarmRecord

Table 12.2.1.4.1a.5-1: Definition of type AlarmRecord

| Attribute name | Data type | Description | S |
| --- | --- | --- | --- |
| alarmId | key(AlarmId) | Alarm identifier, see clause 11.2.2.1.5.1. The alarmId is used as key in alarm record maps. | M |
| objectInstance | Dn | Alarmed object instance | M |
| notificationId | NotificationId | Notification identifier of the last notifyNewAlarm, notifyChangedAlarm or notifyClearedAlarm | M |
| alarmRaisedTime | DateTime | Date and time the alarm was raised the first time, see clause 11.2.2.1.5.1 | M |
| alarmChangedTime | DateTime | Date and time the perceived severity of the alarm changed the last time, see clause 11.2.2.1.5.1 | O |
| alarmClearedTime | DateTime | Date and time the alarm was cleared, see clause 11.2.2.1.5.1 | M |
| alarmType | AlarmType | Alarm type as defined in ITU-T Rec. X. 733 [4] | M |
| probableCause | ProbableCause | Probable cause of an alarm as defined in ITU-T Rec. X.733 [4] | M |
| specificProblem | oneOf(string, integer) | Refinements to the probable cause of the alarm as defined in ITU-T Rec. X. 733 [4] | O |
| perceivedSeverity | PerceivedSeverity | Perceived severity of the alarm as defined in ITU-T Rec. X. 733 [4] | M |
| backedUpStatus | boolean | Indicating if the object emitting the alarm has been backed up as defined in ITU-T Recommendation X. 733 [4] | O |
| backUpObject | Dn | Backup object of the alarmed object as defined in ITU-T Rec. X. 733 [4] | O |
| trendIndication | TrendIndication | Severity trend of the alarmed object as defined in ITU-T Rec. X. 733 [4] | O |
| thresholdInfo | ThresholdInfo | Additional information for threshold crossing alarms as defined in ITU-T Rec. X. 733 [4] | O |
| correlatedNotifications | array(CorrelatedNotification) | Set of all notifications to which this notification is considered to be correlated as defined in ITU-T Rec. X. 733 [4] | O |
| stateChangeDefinition | AttributeValueChangeSet | State transition associated to an alarm as defined in ITU-T Rec. X. 733 [4] | O |
| monitoredAttributes | AttributeNameValuePairSet | Attributes of the alarmed manged object and their corresponding values at the time of the alarm as defined in ITU-T Rec. X. 733 [4]. | O |
| proposedRepairActions | string | Proposed repair action, used if the cause is known and the system being managed can suggest one or more solutions to fix the problem causing the alarm as defined in ITU-T Rec. X. 733 [4] | O |
| additionalText | string | Allows a free form text description to be reported as defined in ITU-T Rec. X. 733 [4] | O |
| additionalInformation | AttributeNameValuePairSet | Allows the inclusion of a set of additional information in the event report as defined in ITU-T Rec. X. 733 [4] | O |
| rootCauseIndicator | boolean | Indicates if this event is the root cause of the events captured by the notifications whose identifiers are in the related correlatedNotifications attribute, see clause 11.2.2.1.5.1 | O |
| ackTime | DateTime | Time when the alarm has been acknowledged or unacknowledged the last time, see clause 11.2.2.1.5.1 | M |
| ackUserId | string | Identifier of a user acknowledging an alarm, see clause 11.2.2.1.5.1 | M |
| ackSystemId | string | Identifier of a system acknowledging an alarm, see clause 11.2.2.1.5.1 | O |
| ackState | AckState | Acknowledgement state, see clause 11.2.2.1.5.1 | M |
| clearUserId | string | Identifier of a system clearing an alarm, see clause 10.2.2.1.5.1 | O |
| clearSystemId | string | Identifier of a user clearing an alarm, see clause 11.2.2.1.5.1 | O |
| serviceUser | string | Identifies the service-user whose request for service provided by the serviceProvider led to the generation of the security alarm, see clause 11.2.2.1.5.1 | O |
| serviceProvider | string | Identifies the service-provider whose service is requested by the serviceUser and the service request provokes the generation of the security alarm, see clause 11.2.2.1.5.1 | O |
| securityAlarmDetector | string | Identity of the detector of the security alarm, see clause 11.2.2.1.5.1 | O |

###### 12.2.1.4.1a.6	Type AlarmCount

Table 12.2.1.4.1a.6-1: Definition of type AlarmCount

| Attribute name | Data type | Description | S |
| --- | --- | --- | --- |
| criticalCount | integer | Number of alarms with perceived severity equal to critical | M |
| majorCount | integer | Number of alarms with perceived severity equal to major | M |
| minorCount | integer | Number of alarms with perceived severity equal to minor | M |
| warningCount | integer | Number of alarms with perceived severity equal to warning | M |
| indeterminateCount | integer | Number of alarms with perceived severity equal to indeterminate | M |
| clearedCount | integer | Number of alarms with perceived severity equal to cleared | M |

###### 12.2.1.4.1a.7	Type Comment

Table 12.2.1.4.1a.7-1: Definition of type Comment

| Attribute name | Data type | Description | S |
| --- | --- | --- | --- |
| commentTime | DateTime | Time when the comment has been added to the alarm. | M |
| commentText | string | Comment in text form | M |
| commentUserId | string | Identifier of the user who makes the comment | M |
| commentSystemId | string | Identifier of the system which makes the comment | O |

###### 12.2.1.4.1a.8	Type Subscription

Table 12.2.1.4.1a.8-1: Definition of type Subscription

| Attribute name | Data type | Description | S |
| --- | --- | --- | --- |
| consumerReference | Uri | URI of the notification target on the MnS consumer | M |
| timeTick | integert | Time window within which the subscriber intends to subscribe again to confirm its subscription, see clause 11.2.2.2.5.1 | O |
| filter | Filter | Filter settings for this subscription, to define the subset of all notifications this subscription relates to. A notification is sent to the subscriber if the filter matches, or if there is no filter. | O |

###### 12.2.1.4.1a.9	Type MergePatchAcknowledgeAlarm

Table 12.2.1.4.1a.9-1: Definition of type MergePatchAcknowledgeAlarm

| Attribute name | Data type | Description | S |
| --- | --- | --- | --- |
| ackUserId | string | User acknowledging an alarm | M |
| ackSystemId | string | System acknowledging an alarm | O |
| ackState | AckState | Indicates the ackState shall be set to "ACKNOWLEDGED" or "UNACKNOWLEDGED" | M |

###### 12.2.1.4.1a.10	Type MergePatchClearAlarm

Table 12.2.1.4.1a.10-1: Definition of type MergePtchClearAlarm

| Attribute name | Data type | Description | S |
| --- | --- | --- | --- |
| clearUserId | clearUserId-Type | User clearing an alarm | M |
| clearSystemId | clearSystemId-Type | System clearing an alarm | O |
| perceivedSeverity | type string, enum "CLEARED" | Indicates the perceivedSeverity shall be set to "CLEARED" | M |

###### 12.2.1.4.1a.11	Type FailedAlarm

Table 12.2.1.4.1a.11-1: Definition of type FailedAlarm

| Attribute name | Data type | Description | S |
| --- | --- | --- | --- |
| alarmId | AlarmId | Indicating the alarms for which the action on the alarm could not be performed | M |
| failureReason | string | Indicating the reason why the action could not be performed | M |

###### 12.2.1.4.1a.12	Type NotifyNewAlarm

Table 12.2.1.4.1a.12-1: Definition of type NotifyNewAlarm

| Attribute name | Data type | Description | S |
| --- | --- | --- | --- |
| href | Uri | URI of the resource where the event (alarm) occurred | M |
| notificationId | NotificationId | Notification identifier as defined in ITU-T Rec. X. 733 [4] | M |
| notificationType | NotificationType | Notification type (notifyNewAlarm) | M |
| eventTime | DateTime | Event (alarm) occurrence time | M |
| systemDN | SystemDN | System DN | M |
| alarmId | AlarmId | Alarm identifier, see clause 11.2.2.1.5.1 | M |
| alarmType | AlarmType | Alarm type as defined in ITU-T Rec. X. 733 [4] | M |
| probableCause | ProbableCause | Probable cause of an alarm as defined in ITU-T Rec. X.733 [4] | M |
| specificProblem | SpecificProblem | Identifies further refinements to the Probable cause of the alarm as defined in ITU-T Rec. X. 733 [4] | O |
| perceivedSeverity | PerceivedSeverity | Perceived severity of an alarm as defined in ITU-T Rec. X. 733 [4] | M |
| backedUpStatus | boolean | Indicating if the object emitting the alarm has been backed up as defined in ITU-T Recommendation X. 733 [4] | O |
| backUpObject | Dn | Indicating the backup object of the alarmed object as defined in ITU-T Rec. X. 733 [4] | O |
| trendIndication | TrendIndication | Severity trend of the alarmed object as defined in ITU-T Rec. X. 733 [4] | O |
| thresholdInfo | ThresholdInfo | Provides additional information for threshold crossing alarms as defined in ITU-T Rec. X. 733 [4] | O |
| correlatedNotifications | array(CorrelatedNotification) | Set of all notifications to which this notification is considered to be correlated as defined in ITU-T Rec. X. 733 [4] | O |
| stateChangeDefinition | AttributeValueChangeSet | Indicates a state transition associated to an alarm as defined in ITU-T Rec. X. 733 [4] | O |
| monitoredAttributes | AttributeNameValuePairSet | Defines one or more attributes of the alarmed manged object and their corresponding values at the time of the alarm as defined in ITU-T Rec. X. 733 [4]. | O |
| proposedRepairActions | string | Used if the cause is known and the system being managed can suggest one or more solutions to fix the problem causing the alarm as defined in ITU-T Rec. X. 733 [4] | O |
| additionalText | string | Allows a free form text description to be reported as defined in ITU-T Rec. X. 733 [4] | O |
| additionalInformation | AttributeNameValuePairSet | Allows the inclusion of a set of additional information in the event report as defined in ITU-T Rec. X. 733 [4] | O |
| rootCauseIndicator | boolean | Indicates if this event is the root cause of the events captured by the notifications whose identifiers are in the related correlatedNotifications attribute, see clause 11.2.2.1.5.1 | O |

###### 12.2.1.4.1a.13	Type NotifyNewSecAlarm

Table 12.2.1.4.1a.13-1: Definition of type NotifyNewSecAlarm

| Attribute name | Data type | Description | S |
| --- | --- | --- | --- |
| href | Uri | URI of the resource where the event (alarm) occurred | M |
| notificationId | NotificationId | Notification identifier as defined in ITU-T Rec. X. 733 [4] | M |
| notificationType | NotificationType | Notification type (notifyNewAlarm) | M |
| eventTime | DateTime | Event (alarm) occurrence time | M |
| systemDN | SystemDN | System DN | M |
| alarmId | AlarmId | Alarm identifier, see clause 11.2.2.1.5.1 | M |
| alarmType | AlarmType | Alarm type as defined in ITU-T Rec. X. 733 [4] | M |
| probableCause | ProbableCause | Probable cause of an alarm as defined in ITU-T Rec. X.733 [4] | M |
| perceivedSeverity | PerceivedSeverity | Perceived severity of an alarm as defined in ITU-T Rec. X. 733 [4] | M |
| correlatedNotifications | array(CorrelatedNotification) | Set of all notifications to which this notification is considered to be correlated as defined in ITU-T Rec. X. 733 [4] | O |
| additionalText | string | Allows a free form text description to be reported as defined in ITU-T Rec. X. 733 [4] | O |
| additionalInformation | AttributeNameValuePairSet | Allows the inclusion of a set of additional information in the event report as defined in ITU-T Rec. X. 733 [4] | O |
| rootCauseIndicator | boolean | Indicates if this event is the root cause of the events captured by the notifications whose identifiers are in the related correlatedNotifications attribute, see clause 11.2.2.1.5.1 | O |
| serviceUser | string | Identifies the service-user whose request for service provided by the serviceProvider led to the generation of the security alarm, see clause 11.2.2.1.5.1 | M |
| serviceProvider | string | Identifies the service-provider whose service is requested by the serviceUser and the service request provokes the generation of the security alarm, see clause 11.2.2.1.5.1 | M |
| securityAlarmDetector | string | Identity of the detector of the security alarm, see clause 11.2.2.1.5.1 | M |

###### 12.2.1.4.1a.14	Type NotifyClearedAlarm

Table 12.2.1.4.1a.14-1: Definition of type NotifyClearedAlarm

| Attribute name | Data type | Description | S |
| --- | --- | --- | --- |
| href | Uri | URI of the resource where the event (alarm) occurred | M |
| notificationId | NotificationId | Notification identifier as defined in ITU-T Rec. X. 733 [4] | M |
| notificationType | NotificationType | Notification type (notifyClearedAlarm) | M |
| eventTime | DateTime | Event occurrence time | M |
| systemDN | SystemDN | System DN | M |
| alarmId | AlarmId | Alarm identifier, see clause 11.2.2.1.5.1 | M |
| alarmType | AlarmType | Alarm type as defined in ITU-T Rec. X. 733 [4] | M |
| probableCause | ProbableCause | Probable cause of an alarm as defined in ITU-T Rec. X.733 [4] | M |
| perceivedSeverity | PerceivedSeverity | Perceived severity of an alarm as defined in ITU-T Rec. X. 733 [4] | M |
| correlatedNotifications | array(correlatedNotification) | Set of all notifications to which this notification is considered to be correlated as defined in ITU-T Rec. X. 733 [4] | O |
| clearUserId | string | Identifier of a user clearing an alarm, see clause 11.2.2.1.5.1 | O |
| clearSystemId | string | Identifier of a system clearing an alarm, see clause 11.2.2.1.5.1 | O |

###### 12.2.1.4.1a.15	Type NotifyChangedAlarm

Table 12.2.1.1a.14.15-1: Definition of type NotifyChangedAlarm

| Attribute name | Data type | Description | S |
| --- | --- | --- | --- |
| href | Uri | URI of the resource where the event (alarm) occurred | M |
| notificationId | NotificationId | Notification identifier as defined in ITU-T Rec. X. 733 [4] | M |
| notificationType | NotificationType | Notification type (notifyChangedAlarm) | M |
| eventTime | DateTime | Event occurrence time | M |
| systemDN | SystemDN | System DN | M |
| alarmId | AlarmId | Alarm identifier, see clause 11.2.2.1.5.1 | M |
| alarmType | AlarmType | Alarm type as defined in ITU-T Rec. X. 733 [4] | M |
| probableCause | ProbableCause | Probable cause of an alarm as defined in ITU-T Rec. X.733 [4] | M |
| perceivedSeverity | PerceivedSeverity | Perceived severity of an alarm as defined in ITU-T Rec. X. 733 [4] | M |

###### 12.2.1.4.1a.16	Type NotifyChangedAlarmGeneral

Table 12.2.1.4.1a.16-1: Definition of type NotifyChangedAlarmGeneral

| Attribute name | Data type | Description | S |
| --- | --- | --- | --- |
| href | Uri | URI of the resource where the event (alarm) occurred | M |
| notificationId | NotificationId | Notification identifier as defined in ITU-T Rec. X. 733 [4] | M |
| notificationType | NotificationType | Notification type (notifyChangedAlarmGeneral) | M |
| eventTime | DateTime | Event occurrence time | M |
| systemDN | SystemDN | System DN | M |
| alarmId | AlarmId | Alarm identifier, see clause 11.2.2.1.5.1 | M |
| alarmType | AlarmType | Alarm type as defined in ITU-T Rec. X. 733 [4] | M |
| probableCause | ProbableCause | Probable cause of an alarm as defined in ITU-T Rec. X.733 [4] | O |
| specificProblem | SpecificProblem | Identifies further refinements to the Probable cause of the alarm as defined in ITU-T Rec. X. 733 [4] | O |
| perceivedSeverity | PerceivedSeverity | Perceived severity of an alarm as defined in ITU-T Rec. X. 733 [4] | O |
| backedUpStatus | boolean | Indicating if the object emitting the alarm has been backed up as defined in ITU-T Recommendation X. 733 [4] | O |
| backUpObject | Dn | Indicating the backup object of the alarmed object as defined in ITU-T Rec. X. 733 [4] | O |
| trendIndication | TrendIndication | Severity trend of the alarmed object as defined in ITU-T Rec. X. 733 [4] | O |
| thresholdInfo | ThresholdInfo | Provides additional information for threshold crossing alarms as defined in ITU-T Rec. X. 733 [4] | O |
| correlatedNotifications | array(CorrelatedNotification) | Set of all notifications to which this notification is considered to be correlated as defined in ITU-T Rec. X. 733 [4] | O |
| stateChangeDefinition | AttributeValueChangeSet | Indicates a state transition associated to an alarm as defined in ITU-T Rec. X. 733 [4] | O |
| monitoredAttributes | AttributeNameValuePairSet | Defines one or more attributes of the alarmed manged object and their corresponding values at the time of the alarm as defined in ITU-T Rec. X. 733 [4]. | O |
| proposedRepairActions | string | Used if the cause is known and the system being managed can suggest one or more solutions to fix the problem causing the alarm as defined in ITU-T Rec. X. 733 [4] | O |
| additionalText | string | Allows a free form text description to be reported as defined in ITU-T Rec. X. 733 [4] | O |
| additionalInformation | AttributeNameValuePairSet | Allows the inclusion of a set of additional information in the event report as defined in ITU-T Rec. X. 733 [4] | O |
| rootCauseIndicator | boolean | Indicates if this event is the root cause of the events captured by the notifications whose identifiers are in the related correlatedNotifications attribute, see clause 11.2.2.1.5.1 | O |
| changedAlarmAttributes | AttributeNameValuePairSet | Indicating the alarm attributes that have changed | O |

###### 12.2.1.4.1a.17	Type NotifyChangedSecAlarmGeneral

Table 12.2.1.4.1a.17-1: Definition of type NotifyChangedSecAlarmGeneral

| Attribute name | Data type | Description | S |
| --- | --- | --- | --- |
| href | Uri | URI of the resource where the event (alarm) occurred | M |
| notificationId | NotificationId | Notification identifier as defined in ITU-T Rec. X. 733 [4] | M |
| notificationType | NotificationType | Notification type (notifyChangedAlarmGeneral) | M |
| eventTime | DateTime | Event occurrence time | M |
| systemDN | SystemDN | System DN | M |
| alarmId | AlarmId | Alarm identifier, see clause 11.2.2.1.5.1 | M |
| alarmType | AlarmType | Alarm type as defined in ITU-T Rec. X. 733 [4] | M |
| probableCause | ProbableCause | Probable cause of an alarm as defined in ITU-T Rec. X.733 [4] | O |
| perceivedSeverity | PerceivedSeverity | Perceived severity of an alarm as defined in ITU-T Rec. X. 733 [4] | O |
| correlatedNotifications | array(CorrelatedNotification) | Set of all notifications to which this notification is considered to be correlated as defined in ITU-T Rec. X. 733 [4] | O |
| additionalText | string | Allows a free form text description to be reported as defined in ITU-T Rec. X. 733 [4] | O |
| additionalInformation | AttributeNameValuePairSet | Allows the inclusion of a set of additional information in the event report as defined in ITU-T Rec. X. 733 [4] | O |
| rootCauseIndicator | boolean | Indicates if this event is the root cause of the events captured by the notifications whose identifiers are in the related correlatedNotifications attribute, see clause 11.2.2.1.5.1 | O |
| serviceUser | string | Identifies the service-user whose request for service provided by the serviceProvider led to the generation of the security alarm, see clause 11.2.2.1.5.1 | M |
| serviceProvider | string | Identifies the service-provider whose service is requested by the serviceUser and the service request provokes the generation of the security alarm, see clause 11.2.2.1.5.1 | M |
| securityAlarmDetector | string | Identity of the detector of the security alarm, see clause 11.2.2.1.5.1 | M |
| changedAlarmAttributes | AttributeNameValuePairSet | Indicating the alarm attributes that have changed | O |

###### 12.2.1.4.1a.18	Type NotifyCorrelatedNotificationChanged

Table 12.2.1.4.1a.18-1: Definition of type NotifyCorrelatedNotificationChanged

| Attribute name | Data type | Description | S |
| --- | --- | --- | --- |
| href | Uri | URI of the resource where the event (alarm) occurred | M |
| notificationId | NotificationId | Notification identifier as defined in ITU-T Rec. X. 733 [4] | M |
| notificationType | NotificationType | Notification type (notifyCorrelatedNotificationChanged) | M |
| eventTime | DateTime | Event occurrence time | M |
| systemDN | SystemDN | System DN | M |
| alarmId | AlarmId | Alarm identifier, see clause 11.2.2.1.5.1 | M |
| correlatedNotifications | array(CorrelatedNotification) | Set of all notifications to which this notification is considered to be correlated as defined in ITU-T Rec. X. 733 [4] | M |
| rootCauseIndicator | boolean | Indicates if this event is the root cause of the events captured by the notifications whose identifiers are in the related correlatedNotifications attribute, see clause 11.2.2.1.5.1 | O |

###### 12.2.1.4.1a.19	Type NotifyAckStateChanged

Table 12.2.1.4.1a.19-1: Definition of type NotifyAckStateChanged

| Attribute name | Data type | Description | S |
| --- | --- | --- | --- |
| href | Uri | URI of the resource where the event (alarm) occurred | M |
| notificationId | NotificationId | Notification identifier as defined in ITU-T Rec. X. 733 [4] | M |
| notificationType | NotificationType | Notification type (notifyAckStateChanged) | M |
| eventTime | DateTime | Event occurrence time | M |
| systemDN | SystemDN | System DN | M |
| alarmId | AlarmId | Alarm identifier, see clause 11.2.2.1.5.1 | M |
| alarmType | AlarmType | Alarm type as defined in ITU-T Rec. X. 733 [4] | M |
| probableCause | ProbableCause | Probable cause of an alarm as defined in ITU-T Rec. X.733 [4] | M |
| perceivedSeverity | PerceivedSeverity | Perceived severity of an alarm as defined in ITU-T Rec. X. 733 [4] | M |
| ackState | string | Acknowledgement state, see clause 11.2.2.1.5.1 | M |
| ackUserId | string | Identifier of a system acknowledging an alarm, see clause 11.2.2.1.5.1 | M |
| ackSystemId | string | Identifier of a user acknowledging an alarm, see clause 11.2.2.1.5.1 | O |

###### 12.2.1.4.1a.20	Type NotifyComments

Table 12.2.1.4.1a.20-1: Definition of type NotifyComments

| Attribute name | Data type | Description | S |
| --- | --- | --- | --- |
| href | Uri | URI of the resource where the event (alarm) occurred | M |
| notificationId | NotificationId | Notification identifier as defined in ITU-T Rec. X. 733 [4] | M |
| notificationType | NotificationType | Notification type (notifyComments) | M |
| eventTime | DateTime | Event occurrence time | M |
| systemDN | SystemDN | System DN | M |
| alarmId | AlarmId | Alarm identifier, see clause 11.2.2.1.5.1 | M |
| alarmType | AlarmType | Alarm type as defined in ITU-T Rec. X. 733 [4] | M |
| probableCause | ProbableCause | Probable cause of an alarm as defined in ITU-T Rec. X.733 [4] | M |
| perceivedSeverity | PerceivedSeverity | Perceived severity of an alarm as defined in ITU-T Rec. X. 733 [4] | M |
| comments | map(Comment) | Set of all comments related to an alarm | M |

###### 12.2.1.4.1a.21	Type NotifyPotentialFaultyAlarmList

Table 12.2.1.4.1a.21-1: Definition of type NotifyPotentialFaultyAlarmList

| Attribute name | Data type | Description | S |
| --- | --- | --- | --- |
| href | Uri | URI of the resource where the event (alarm) occurred | M |
| notificationId | NotificationId | Notification identifier as defined in ITU-T Rec. X. 733 [4] | M |
| notificationType | NotificationType | Notification type (notifyPotentialFaultyAlarmList) | M |
| eventTime | DateTime | Event occurrence time | M |
| systemDN | SystemDN | System DN | M |
| reason | string | Indicating the reason why the alarm list has to be rebuilt. | M |

###### 12.2.1.4.1a.22	Type NotifyAlarmListRebuilt

Table 12.2.1.4.1a.22-1: Definition of type NotifyAlarmListRebuilt

| Attribute name | Data type | Description | S |
| --- | --- | --- | --- |
| href | Uri | URI of the resource where the event (alarm) occurred | M |
| notificationId | NotificationId | Notification identifier as defined in ITU-T Rec. X. 733 [4] | M |
| notificationType | NotificationType | Notification type (notifyAlarmListRebuilt) | M |
| eventTime | DateTime | Event occurrence time | M |
| systemDN | SystemDN | System DN | M |
| reason | string | Indicating the reason why the alarm list has been rebuilt | M |
| alarmListAlignmentRequirement | AlarmListAlignmentRequirement | Indicating if alarm list alignment is required or not | O |

##### 12.2.1.4.2	Void

##### 12.2.1.4.3	Void

##### 12.2.1.4.4	Simple data types and enumerations

###### 12.2.1.4.4.1	General

This clause defines simple data types and enumerations that are used by the data structures defined in the previous clauses.

###### 12.2.1.4.4.2	Simple data types

Table 12.2.1.4.4.2-1: Simple data types

| Type name | Type definition | Description |
| --- | --- | --- |
| AlarmId | string | Alarm identifier, see clause 11.2.2.1.5.1 |

###### 12.2.1.4.4.3	Enumeration AlarmAckState

Table 12.2.1.4.4.3-1: Enumeration AlarmAckState

| Enumeration value | Description |
| --- | --- |
| ALL_ALARMS | All alarms shall be returned or counted. |
| ALL_ACTIVE_ALARMS | All active alarms shall be returned or counted. |
| ALL_ACTIVE_AND_ACKNOWLEDGED _ALARMS | All active and acknowledged alarms shall be returned or counted. |
| ALL_ACTIVE_AND_UNACKNOWLEDGED_ALARMS | All active and unacknowledged alarms shall be returned or counted. |
| ALL_CLEARED_AND_ACKNOWLEDGED_ALARMS | All cleared and unacknowledged alarms shall be returned or counted. |
| ALL_UNACKNOWLEDGED_ALARMS | All unacknowledged alarms shall be returned or counted |

###### 12.2.1.4.4.4	Enumeration AckState

Table 12.2.1.4.4.4-1: Enumeration ackState

| Enumeration value | Description |
| --- | --- |
| ACKNOWLEDGED | State acknowledged. |
| UNACKNOWLEDGED | State unacknowledged. |

###### 12.2.1.4.4.5	Enumeration AlarmListAlignmentRequirement

Table 12.2.1.4.4.5-1: Enumeration AlarmListAlignmentRequirement

| Enumeration value | Description |
| --- | --- |
| ALIGNMENT_REQUIRED | Alarm list alignment is required |
| ALIGNMENT_NOT_REQUIRED | Alarm list alignment is not required |

###### 12.2.1.4.4.6	Enumeration AlarmType

Table 12.2.1.4.4.6-1: Enumeration AlarmType

| Enumeration value | Description |
| --- | --- |
| COMMUNICATIONS_ALARM | An alarm of this type is principally associated with the procedures and/or processes required to convey information from one point to another (Rec. ITU-T X. 733 [4]). |
| PROCESSING_ERROR_ALARM | An alarm of this type is principally associated with a software or processing fault (Rec. ITU-T X. 733 [4]). |
| ENVIRONMENTAL_ALARM | An alarm of this type is principally associated with a condition relating to an enclosure in which the equipment resides (Rec. ITU-T X. 733 [4]). |
| QUALITY_OF_SERVICE_ALARM | An alarm of this type is principally associated with a degradation in the quality of a service (Rec. ITU-T X. 733 [4]). |
| EQUIPMENT_ALARM | An alarm of this type is principally associated with an equipment fault (Rec. ITU-T X. 733 [4]). |
| INTEGRITY_VIOLATION | An indication that information may have been illegally modified, inserted or deleted. |
| OPERATIONAL_VIOLATION | An indication that the provision of the requested service was not possible due to the unavailability, malfunction or incorrect invocation of the service. |
| PHYSICAL_VIOLATION | An indication that a physical resource has been violated in a way that suggests a security attack. |
| SECURITY_SERVICE_OR_MECHANISM_VIOLATION | An indication that a security attack has been detected by a security service or mechanism. |
| TIME_DOMAIN_VIOLATION | An indication that an event has occurred at an unexpected or prohibited time. |

###### 12.2.1.4.4.7	Enumeration ProbableCause

Table 12.2.1.4.4.7-1: Enumeration ProbableCause

| Enumeration value | Description |
| --- | --- |
| PROBABLE_CAUSE_001 | Generic probable cause string 001, mapping to a concrete probable cause is vendor specific |
| PROBABLE_CAUSE_002 | Generic probable cause string 002, mapping to a concrete probable cause is vendor specific |
| PROBABLE_CAUSE_003 | Generic probable cause string 003, mapping to a concrete probable cause is vendor specific |
| PROBABLE_CAUSE_004 | Generic probable cause string 004, mapping to a concrete probable cause is vendor specific |
| PROBABLE_CAUSE_005 | Generic probable cause string 005, mapping to a concrete probable cause is vendor specific |

###### 12.2.1.4.4.8	Enumeration AlarmNotificationTypes

Table 12.2.1.4.4.8-1: Enumeration AlarmNotificationTypes

| Enumeration value | Description |
| --- | --- |
| notifyNewAlarm | Notification type is notifyNewAlarm |
| notifyNewSecurityAlarm | Notification type is notifyNewSecurityAlarm |
| notifyAckStateChanged | Notification type is notifyAckStateChanged |
| notifyClearedAlarm | Notification type is notifyClearedAlarm |
| notifyAlarmListRebuiltAlarm | Notification type is notifyAlarmListRebuiltAlarm |
| notifyChangedAlarm | Notification type is notifyChangedAlarm |
| notifyComments | Notification type is notifyComments |
| notifyPotentialFaultyAlarmList | Notification type is notifyPotentialFaultyAlarmList |
| notifyCorrelatedNotificationChanged | Notification type is notifyCorrelatedNotificationChanged |
| notifyChangedAlarmGeneral | Notification type is notifyChangedAlarmGeneral |

###### 12.2.1.4.4.9	Enumeration PerceivedSeverity

Table 12.2.1.4.4.9-1: Enumeration PerceivedSeverity

| Enumeration value | Description |
| --- | --- |
| CRITICAL | The Critical severity level indicates that a service affecting condition has occurred and an immediate corrective action is required (Rec. ITU-T X. 733 [4]). |
| MAJOR | The Major severity level indicates that a service affecting condition has developed and an urgent corrective action is required (Rec. ITU-T X. 733 [4]). |
| MINOR | The Minor severity level indicates the existence of a non-service affecting fault condition and that corrective action should be taken in order to prevent a more serious (for example, service affecting) fault (Rec. ITU-T X. 733 [4]). |
| WARNING | The Warning severity level indicates the detection of a potential or impending service affecting fault, before any significant effects have been felt (Rec. ITU-T X. 733 [4]). |
| INDETERMINATE | The Indeterminate severity level indicates that the severity level cannot be determined (Rec. ITU-T X. 733 [4]). |
| CLEARED | The Cleared severity level indicates the clearing of one or more previously reported alarms (Rec. ITU-T X. 733 [4]). |

###### 12.2.1.4.4.10	Enumeration TrendIndication

Table 12.2.1.4.4.10-1: Enumeration TrendIndication

| Enumeration value | Description |
| --- | --- |
| MORE_SEVERE | Severity trend of the alarmed object is more severe (Rec. ITU-T X.733 [4]) |
| NO_CHANGE | Severity trend of the alarmed object is no change (Rec. ITU-T X.733 [4]) |
| LESS_SEVERE | Severity trend of the alarmed object is less severe (Rec. ITU-T X.733 [4]) |

End of modifications