- 3GPP TSG SA WG5 Meeting #136-e	S5-212032

- March 01 – 09, 2021, e-Meeting	revision of S5-2abcde

- Source:	Intel

- Title:	pCR 28.814 add solutions for UPF selection to assist EAS deployment

- Document for:	Approval

Agenda Item:	6.5.3

# 1	Decision/action requested

The group is asked to discuss and approve the proposals.

# 2	References

[1] 	TR 28.814 v0.0.0 “Study on enhancements of edge computing management”

# 3	Rationale

This contribution proposes to add the solutions to support the use case and requirements for UPF selection to support EAS deployment in TR 28.814 [1].

# 4	Detailed proposal

1st modified section

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.558 “Architecture for enabling Edge Applications”.

[3]	3GPP TR 23.758 “Study on application architecture for enabling Edge”.

[4]		3GPP TS 23.501 “System architecture for the 5G System (5GS)”.

[5]					3GPP TS 28.531 “Management and orchestration; Provisioning”.

[6]	ETSI GS NFV-IFA 011 V3.3.1 “Network Functions Virtualisation (NFV) Release 3; Management and Orchestration; VNF Descriptor and Packaging Specification”.

[7]	ETSI GS NFV-IFA 013 V3.4.1 “Network Functions Virtualisation (NFV) Release 3; Management and Orchestration; Os-Ma-nfvo reference point -Interface and Information Model Specification”.

[x]		3GPP TS 28.532 “Management and orchestration; Generic management services”.

Next modified section

# 7	Potential solutions

Editor’s note: this clause is to describe the potential solution to support orchestration and management of edge computing.

## 7.x	UPF selection to assist EAS deployment

### 7.x.1	General

This subclause provides potential solutions for use case of UPF selection to support EAS deployment (see subclause 6.2.x). It is assumed that the EAS has been deployed based on the solution described in subclause 7.1.

This solution applies to the scenario where PLMN operator had deployed UPF(s) in 5GC, and ECSP management system needs to request PLMN management system to select a UPF in 5GC to serve the EAS.

### 7.x.2	Solution #1 – UPF selecton using LCM MnS

Figure 7.x.3-1 shows that ECSP management system requests PLMN management system to identify a UPF via the LCM MnS.

Figure 7.x.3-1: UPF selection via the LCM MnS

ECSP management system consumes the LCM MnS with operation selectUpfReq to request the PLMN management system to select a UPF, with the following attributes:

-	EAS Topological Service Area: A Topological Service Area is defined in relationship with a UE's point of connection to the network, such as: a collection of Cell IDs, Tracking Area Identities or the PLMN ID (see subclause 7.3.3.2 in TS 23.558 [2]).

- EAS Geographical Service Area: A Geographical Service Area is an area that is specified by geographical units such as: Geographical coordinates ID (see subclause 7.3.3.3 in TS 23.558 [2]).

- List of N6 traffic routing requirements: The N6 traffic routing information and/or routing profile ID corresponding to each EAS DNAI.

PLMN management system sends a selectUpfResp to notify the consumer that the request is in progress.

PLMN management system finds a UPF among the UPF(s) being deployed, based on the EAS Topological Service Area, EAS Geographical Service Area (see subclause 7.3.3 in TS 28.558 [2]), and QoS requirements, and may perform the following operations if no UPF can be found:

- Instantiate a new UPF.

- Configure the SMF – SMFFunction IOC to add the UPF to the available UPF list (see clasue 6.3.3.2 in TS 23.501 [4]).

PLMN management system sends the selectUpfResp to notify ECSP management system with the UPF information for the UPF being selected.

### 7.x.3	Solution #2 – UPF selecton using provisioning MnS

Figure 7.x.2-1 shows that ECSP management system requests 3GPP management system to identify a UPF via the provisioning MnS.

Figure 7.x.2-1: UPF selection via the provisioning MnS

ECSP management system needs to send the UPF selection requirements in an IOC. So, the upfSelection IOC should be defined to support the UPF selection solution. The upfSelection IOC should contain, but not limited to, the following attributes:

- EAS Topological Service Area: A Topological Service Area is defined in relationship with a UE's point of connection to the network, such as: a collection of Cell IDs, Tracking Area Identities or the PLMN ID (see subclause 7.3.3.2 in TS 23.558 [2]).

- EAS Geographical Service Area: A Geographical Service Area is an area that is specified by geographical units such as: Geographical coordinates ID (see subclause 7.3.3.3 in TS 23.558 [2]).

- List of N6 traffic routing requirements: The N6 traffic routing information and/or routing profile ID corresponding to each EAS DNAI (see table 8.2.4.1 in TS 23.558 [2]).

ECSP management system consumes the provisioning MnS with operation createMOI for upfSelection IOC with EAS Topological Service Area, EAS Geographical Service Area, list of N6 traffic routing requirements, EAS connection information, to request PLMN management system to select a UPF information.

PLMN management system finds a UPF among the UPF(s) being deployed, based on the service area requirements (e.g. Topological Service Area, Geographical Service Area, (see subclause 7.3.3 in TS 28.558 [2])) and QoS requirements, contained in the upfSelection IOC, and may perform the following operations if no UPF can be found:

- Instantiate a new UPF.

- Configure the SMF – SMFFunction IOC to add the UPF to the available UPF list (see clasue 6.3.3.2 in TS 23.501 [4]).

PLMN management system creates the upfSelection MOI, and sends notifyMOICreation with attributeList, including the UPF information.

PLMN management system returns the output parameters with the UPF information.

End of modified section